| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink base codebooks proposal for 2 transmit antennas |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Alexei Davydov Qinghua Li Guangjie Li Intel Corporation | alexei.davydov@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 (DL-MIMO)” |  |
| Abstract | Proposal on DL MIMO codebooks for 2 TX antennas |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink base codebooks for 2 transmit antennas

Alexei Davydov, Qinghua Li, Guangjie Li

Intel Corporation

- Introduction

The performance of DL CL MIMO precoding using 2TX antenna can be improved by using improved codebook.

- Performance analysis

- Propose text change

------------------------------------ Start text ------------------------------------------

                - SU-MIMO base codebook

The rank-1 downlink codebook for two transmit antennas are defined as

vi=αi⋅realvi+j⋅βi⋅imag(vi)						()

where vi is the vector codeword in the base codebook; i=1,2,...,2L is the codeword index; and αi, βi are scaling factors of vector vi. The parameters αi, βi, and vi parameters for codebook C(2, 1, 3) is defined in Table 1.

Table 1 Base codebook C(2,1,3)

| Index | Value |
| --- | --- |
| 1 | α =12, β =1 |
|  | v = [1, 1]T |
| 2 | α =12, β =12 |
|  | v = [1, j]T |
| 3 | α = 0.2412, β = 0.4938 |
|  | v = [-3, 2+j]T |
| 4 | α = 0.6619, β = 0.3516 |
|  | v = [-1, -1+j]T |
| 5 | α = 0.2342, β = 0.5355 |
|  | v = [-3, 2+j]T |
| 6 | α = 0.1768, β = 0.6843 |
|  | v = [-4, -1+j]T |
| 7 | α = 0.7069, β = 0.0245 |
|  | v = [-1, 1+j]T |
| 8 | α = 0.2378, β = 0.5147 |
|  | v = [-3, -2-j]T |

The rank-2 codebook C(2, 2, 3) is generated from C(2, 1, 3). One operation Hv is employed to construct the second column for Rank 2 case.

Hv=I,v=e1I−pwwH,otherwise,

where

v is an vector from C(2, 1, 3)

w=v−e1ande1=[10...0]T;

p=2wHw

Iis the 2x2 indentity matrix;

Hdenotes the conjugate transpose operation

So C2,2,3=HC2,1,3

------------------------------------ End Text ------------------------------------------