| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for 8Tx Pilot Pattern for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-23 |  |
| Source(s) | Ramesh Annavajjala, Philip V. Orlik, Amine Maaref, Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Labs Toshiyuki Kuze Mitsubishi Electric Corp | Voice: 617-621-{7558, 7518, 7570, 7595} Fax: 617-621-7550 {annavajjala, porlik,maaref,tao,jzhang}@merl.com Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | IEEE 802.16m AWD |  |
| Abstract | This contribution proposes a pilot pattern for 8 transmit antennas in the current 802.16m Amendment Document |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of 8-Antenna Pilot Patterns for the IEEE 802.16m Amendment

Ramesh Annavajjala, Phil Orlik, Amine Maaref, Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Labs

Toshiyuki Kuze

Mitsubishi Electric Corp

# Introduction

In this contribution we propose the text of 8-antenna pilot patterns to be included in the 802.16m amendment document ([1]). The proposed text is developed in such a way that it can be readily combined with the IEEE P802.16 Rev2/D8 ([2]). It is compliant to the 802.16m SRD ([3]) and the 802.16m SDD ([4]), and it follows the style and format guidelines in ([5]).

.

# References

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[3] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[4] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Text Proposal for Inclusion in the IEEE 802.16m Amendment Draft

Insert the following text in red on page. 47, at the end of Section 15.3.5.4.1, of [3] as follows:

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

15.3.5.4.1 Pilot Patterns

With 8-stream MIMO, the ABS shall use the following pilot patterns, shown in Figs. 421 through 428. In each one of Figs. 421 through 428, the subfigure (a) indicates the pilot patterns for a  sub-frame with 6 OFDM symbols, the subfigure (b) indicates the pilot patterns for a sub-frame with 5 OFDM symbol whereas the subfigure (c) indicates the pilot patterns for a sub-frame with 7 OFDM symbols.

Figure 421: Pilot patterns on stream 0 for 8 data streams

Figure 422: Pilot patterns on stream 1 for 8 data streams

Figure 423: Pilot patterns on stream 2 for 8 data streams

Figure 424: Pilot patterns on stream 3 for 8 data streams

Figure 425: Pilot patterns on stream 4 for 8 data streams

Figure 426: Pilot patterns on stream 5 for 8 data streams

Figure 427: Pilot patterns on stream 6 for 8 data streams

Figure 428: Pilot patterns on stream 7 for 8 data streams

[-------------------------------------------------End of Text Proposal----------------------------------------------------]