| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SDD 10.13 Multi-Radio Coexistence |  |
| Date Submitted | 2009-05-02 |  |
| Source(s) | Jing Zhu Intel Corporation Jan Suumaki Nokia Chi-Chen Lee, I-Kang Fu, Li-Chun Ko, Chih-Hao Yeh, Kyle Hsu, Paul Cheng MediaTek Inc. Yousuf Saifullah NSN Yung-Han Chen ITRI Jaeweon Cho Samsung Electronics | +1-503-2647073 jing.z.zhu@intel.com +358503367739 jan.suumaki@nokia.com chichen.lee@mediatek.com IK.Fu@mediatek.com LC.Ko@mediatek.com yousuf.saifullah@nsn.com chenyunghan@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: SDD comments / Area: Chapter 10.13 (Multi-Radio Coexistence) “Comments on SDD 10.13 Multi-Radio Coexistence” |  |
| Abstract | Proposed text for multi-radio coexistence in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SDD 10.13 Multi-Radio Coexistence

Jing Zhu

Intel Corporation

Jan Suumaki

Nokia

Chi-Chen Lee, I-Kang Fu, Li-Chun Ko, Chih-Hao Yeh, Kyle Hsu, Paul Cheng

MediaTek Inc.

Yousuf Saifullah

NSN

Yung-Han Chen

ITRI

Jaeweon Cho

Samsung Electronics

- Introduction

The contribution proposes text for Section 10.13 Multi-Radio Coexistence in 802.16m SDD. The proposed text is developed so that it can be readily combined with the 802.16m SDD [1].

- References

[1] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

### Change Section 10.13 according to the following text

---------start of the text -----------------------------

10.13 Multi-Radio Coexistence

Multi-Radio Coexistence supports concurrent operations of IEEE 802.16m and non IEEE 802.16m radios collocated on the same mobile station

AMS conducts pre-negotiated periodic absences from the serving ABS to support concurrent operation of co-located non 802.16 radios, e.g. IEEE 802.11, IEEE 802.15.1, etc., and the time pattern of such periodic absence is referred by ABS and AMS as CLC class.

The following parameters are defined to support CLC class operation:

- CLC start time: the start time of a CLC class

- CLC active interval: the time duration of a CLC class designated for co-located non 802.16 radio activities.

- CLC active cycle: the time interval of the active pattern of a CLC class repeating

- CLC active ratio: the time ratio of CLC active intervals to CLC active cycle of a CLC class

- number of active CLC classes: the number of active CLC classes of the same type of an AMS

802.16m supports three types of CLC classes, and they differ from each other in terms of the time unit of CLC start time, active cycle and active interval, as shown in Table X1.

Type I CLC class is recommended for non 802.16 radio activity that is low duty cycle, and may not align with 802.16 frame boundary. Otherwise, Type II CLC class is recommended for better scheduling flexibility. Type III CLC class is recommended for continuous non-802.16 radio activity that lasts seconds, and has only one cycle.

Table X1: Time Unit of CLC Class Parameters

|  | CLC active cycle | CLC active interval | CLC start time |
| --- | --- | --- | --- |
| Type I | microsecond | subframe | subframe |
| Type II | frame | subframe | frame |
| Type III | not applicable | superframe | superframe |

AMS determines CLC active interval and cycle based on the activities of its co-located non 802.16 radios. AMS determines CLC start time only for Type I CLC class, and ABS determines CLC start time for Type II and III CLC class for better scheduling flexibility.

The serving ABS shall not schedule A-MAP, data, and HARQ feedback of the AMS’s allocations in CLC active interval of an active CLC class. Whether only DL or only UL or both are prohibited depends on the configuration of the CLC class. The default is both DL and UL allocations are prohibited.

---------end of the text -----------------------------