| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femtocell BS Overview |  |
| Date Submitted | 2009-04-24 |  |
| Source(s) | Kamran Etemad Joey Chou Intel | E-mail: kamran.etemad@intel.com joey.chou@intel.com |
| Re: | TGm SDD: SON |  |
| Abstract | This contribution proposes text for Femtocell BS overview. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femtocell BS Overview

Kamran Etemad, Joey Chou

Intel

- Introduction

This contribution proposes the text for Femtocell BS overview.

- Proposed text

15 Support for Femtocell BS

The IEEE802.16 Femtocell BS is a simplified and low power version of IEEE802.16 macro BS with IP-backhaul through a local broadband connection (e.g. DSL, cable) or a fixed wireless backhaul. Femtocell BS’s are typically deployed indoors, at homes or SOHO, by end users to improve in-building wireless coverage to MS with low mobility. IEEE802.16 Femtcells typically operate in licensed spectrum and reuse the same IEEE802.16 air interface designed for wide area cellular networks. Such femtocell BS’s support both legacy MS’s with no support for femtocell specific functionalities and AMS’s.

Femtocell BS’s may be deployed in the same or different RF carrier than the macro-BS’s and may be configured to provide exclusive or preferential access to MSs selected by the femtocell’s primary subscriber.  Compared with macro and picocell BS’s, the femtocell BS’s deployments are driven by customers and can be installed with minimal or no prior on-site survey or on-site frequency planning.

15.1 Types of Base stations

A Femtocell BS is a BS with low transmit power, typically installed by a subscriber in home or SOHO to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A Femtocell BS is connected to the service provider’s network via broadband (such as DSL, or cable).