| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Header Restructure |  |
| Date Submitted | 2009-04-24 |  |
| Source(s) | Joey Chou Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm SDD: SON |  |
| Abstract | This contribution proposes text for MAC header restructure. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Header Restructure

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui,

Intel

- Introduction

IEEE 802.16 GMH has been loaded with excessive features in the previous amendments and revisions, In order to support many advanced features in 802.16m efficiently, the restructure of the MAC header is needed. Here are some guidelines for the MAC header restructure:

- Reuse – 802.16m is just an amendment to 802.16Rev2 base standard; therefore, the restructure should take into account the reuse of existing MAC management and signaling messages.

- Efficiency – The existing GMH is 6 bytes long that is extremely inefficient in sending small singling messages. The new header structure should take into account not only the size of the message, but also how often a message is sent.

- Flexibility – It seems that a new set of features is created; a new layer identified by the subtype is added to the existing MAC header. The new header should have the flexibility to add new features in the future.

Since 802.16Rev2 is the base standard for 802.16m, the first thing we need to consider is to insure the new MAC header structure is able to support the subheaders, extended subheaders, control and signaling messages of 802.16Rev2 as shown in Figure 1 to Figure 4 below.

Figure 1: GMH UL/DL MAC PDU with data payload

Figure 2: UL MAC Signaling Header Type I without data payload

Figure 3: UL MAC Signaling Header Type II without data payload

Figure 4: GMH UL/DL Extended Subheaders

- Proposed text

10.12.1MAC header formats

10.12.1.1 Generic MAC Header

Flow ID (4)

Length (3)

Length (8)

EH (1)

Figure 22 Generic MAC header format

- FlowID (Flow Identifier): This field indicates the service flow that is addressed. This field is 4bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Length: Length of the payload. This field is 11bits long

10.12.2.1 Fragmentation and packing extended header for transport connection

This fragmentation and packing extended header is shown in Figure 25. This header shall be used when MAC PDU contains single transport connection payload. The location of this header exists after the last extended header ( i.e. extended header with ‘Last’ = ‘1’) if ‘EH’ in GMH set to ‘1’ or after the GMH if ‘EH’ in GMH set to ‘0’.

Figure 25 Fragmentation and packing extended header format for transport connection

- SN (10 bits): Payload sequence number

- FC (2 bits): Fragmentation control bits definition is given in Table 2.

- End (1 bit): If this bit set to ‘0’, another ‘Length’ and ‘End’ field are followed. If this bit set to ‘1’, reserved bits may follow for byte alignment

- Length (11bits): This field represents the length of SDU/SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 length fields are present in the header

- Rsvd: Reserved bits for byte alignment.

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only First fragment of an SDU b) MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | a) MPDU with only Last fragment of an SDU b) MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only middle fragment of an SDU b) MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

Table 2 Fragmentation control information

10.12.2.2 Fragmentation extended header for management connection

This fragmentation extended header is shown in Figure 26. This header shall be used when MAC PDU contains single management message payload

Figure 26 Fragmentation extended header format for management connection

- Last (1 bit): always set to ‘1’

- TYPE(TBD): Extended header type field

- SN (8 bits): Payload sequence number

- FC (2 bits): Fragmentation control bits definition is given in Table 2.

- Rsvd: Reserved bits for byte alignment.