| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Header Restructure |  |
| Date Submitted | 2009-04-30 |  |
| Source(s) | Joey Chou Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm SDD: SON |  |
| Abstract | This contribution proposes text for MAC header restructure. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Header Restructure

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Jie Hui,

Intel

- Introduction

IEEE 802.16 GMH has been loaded with excessive features in the previous amendments and revisions. In order to support many advanced features in 802.16m efficiently, the restructure of the MAC header is needed. Here are some guidelines for the MAC header restructure:

- Reuse – The restructure should take into account the reuse of existing MAC management and signaling messages in order to minimize the impacts to 802.16Rev2 standard.

- Efficiency – The existing GMH is 6 bytes long that is extremely inefficient in sending small singling messages. The new header structure should take into account not only the size of the message, but also how often a message is sent.

- Flexibility – The existing MAC header has two many layers identified by the subtypes that are not flexible to support new features.

802.16Rev2 MAC Header Structure

Since reuse is one of the guidelines, we first look at the MAC header structure in its base standard 802,16Rev2 that can be divided into the following categories:

- Per PDU, per SDU subheader, extended subheader (Figure 1, Figure 3)

  - Packing

  - Fragmentation

  - MAC management message

- UL MAC Signaling Header Type I & Type II without data payload (Figure 2 & Figure 4)

Figure 1: GMH UL/DL MAC PDU with data payload

Figure 2: UL MAC Signaling Header Type I without data payload

Figure 3: GMH UL/DL Extended Subheaders

Figure 4: UL MAC Signaling Header Type II without data payload

802.16Rev2 MAC Header Structure

Figure 5: Generic MAC header format

Figure 5 is the generic MAC header, as defined in the current SDD. Flow ID is 4 bits long, and is defined as the following to address up to 16 service flows.

- 0000: MAC signaling channel

- 0001: MAC management channel

- 0010: L2_transfer channel

- 0011 – 1011: MAC data transport

- 1100 – 1111: reserved

MAC Signaling Header

Figure 6: MAC Signaling Header

Figure 6 defines the MAC signaling header to support UL MAC Signaling Header Type I & Type II without data payload, as shown in Figure 2 & 4. Type I and Type II signaling data ranges from 5 bits to 32 bits. MAC Signaling Header can scale from 2 to 6 bytes that can support up to 40 bits signaling data.

- Flow ID: = 0000

- EH: = 1 to indicate the existence of MAC signaling header

- SigType: is 5 bits long, and can represent up to 32 MAC signals

- SigLen: indicates the length of MAC signaling data in bytes, 3 bits long with the maximum value of 5 to dynamically support signaling data up to 40 bits

MAC Payload with Extended Header

Figure 7: MAC Payload Data without Extended Header

Figure 7 shows the format of MAC payload data that does not include any headers or extended headers. The MAC payload channels are identified by flow ID 0011 – 1011.

10.12.2 and 10.12.2.1 define the Extended Header and fragmentation / packing header that will be present when EH bit is set in GMH. It is not possible to have one EH bit to indicate two headers. 802.16Rev2 has EH, FSH, and PSH bits to support extended header, fragmentation, and packing headers.

MAC Payload Data with Extended Header & Fragmentation / Packing Header

Figure 8: MAC Payload Data with Extended Header & Fragmentation / Packing Header

Figure 8 shows a MAC payload data that includes extended header and fragmentation / packing header to support per PDU, per SDU subheader, and extended subheader, as shown in Figure 2 & 4. The extended header is changed slightly from the Extended Subheader of 802.16Rev2 (in Fig 35) since a single EH bit is used to control the co-location of the extended header and fragmentation / packing header. The format of the extended header is described below.

- EHLen: 8 bits long to indicate the length of extended headers

- EHType: indicate the type of the extended header

- EHType specific data: payload specific to each extended header type

- FPH: is only present in the last extended header. When FPH = 1, indicates the presence of fragmentation / packing header; FPH = 0, no fragmentation / packing header

The format of the fragmentation / packing header is described below.

- SN: fragment sequence number, 10 bits long

- FC: Fragmentation control bits, 2 bits long

- FLen: the length of each fragment, 11 bits long

MAC Payload Data with Fragmentation / Packing Header

Figure 9: MAC Payload Data with Fragmentation / Packing Header

Figure 9 show a MAC payload data with fragmentation / packing header. EHLen = 0 means the extended header is missing.

GMH

EH = 1,  LEN = a

…

FPEH

SN = P

FC = 2,  FLEN = b

Last fragment of MAC SDU #1

FPEH

SN = 0

FC = 0,  FLEN = c

unfragmented MAC SDU #2

FPEH

SN = 0

FC = 0,  FLEN = d

unfragmented MAC SDU #3

FPEH

SN = 1

FC = 1,  FLEN = e

1st fragment MAC SDU #N

GMH

EH = 1,  LEN = a

..

FPEH

SN = x

FC = 3,  FLEN = b

Continuing fragment of MAC SDU #1

FPEH

SN = 0

FC = 0,  FLEN = c

unfragmented MAC SDU #2

GMH

EH = 1,  LEN = d

GMH

EH = 1,  LEN = f

FPEH

SN = x+1

FC = 3,  FLEN = e

Continuing fragment of MAC SDU #1

FPEH

SN = x+2

FC = 3,  FLEN = g

Continuing fragment of MAC SDU #1

MAC PDU

MAC PDU

MAC PDU

MAC PDU

a = b + c+ d + e + size(FPEH) * 4

a = b + c + size(FPEH) * 2

Figure 10: Fragmentation / Packing examples

MAC management Message without Fragmentation

Figure 10 shows an example of MAC payload with fragmentation and packing.

Figure 11: MAC Management Message without fragmentation

Figure 11 shows the format of MAC management message without fragmentation.

- Flow ID = 0001

- EH = 0

MAC management Message with Fragmentation

Figure 12: MAC Management Message with fragmentation

Figure 12 shows the format of MAC management message with fragmentation.

- Flow ID = 0001

- EH = 1

- Proposed text

10.12.1 MAC header formats

10.12.1.1 Generic MAC Header

Figure 22 Generic MAC header format

- FlowID (Flow Identifier): This field indicates the service flow that is addressed. This field is 4bits long, and is defined as the following to address up to 16 flows.

- 0000: MAC signaling channel

- 0001: MAC management channel

- 0010: L2_transfer channel

- 0011 – 1011: MAC data transport

- 1100 – 1111: reserved

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Length: Length of the payload. This field is 11bits long

10.12.1.3 MAC Payload without Extended Header

Figure 23 shows the format of MAC payload data that does not include any headers or extended headers. The MAC payload channels are identified by flow ID 0011 – 1011.

Figure 23: MAC Payload Data without Extended Header

10.12.1.4 MAC Management Message without Extended Header

Figure 24: MAC Management Message without fragmentation

Figure 24 shows the format of MAC management message without fragmentation.

- Flow ID = 0001

- EH = 0

10.12.2 Extended header

The inclusion of extended header is indicated by EH indicator bit in MAC Header. The EH format is shown in Figure 24 and will be used unless specified otherwise.

Figure 24 Extended Header Format

- Last: When the “Last” bit is set, this extended header is the last one. If this bit is not set, another extended header will follow the current extended header.

- Type: indicates the type of extended header. The length is TBD.

- Body Contents: Type-dependent contents.

10.12.2.1 MAC Payload with Extended Header and Fragmentation and packing extended header

Figure 25 shows a MAC payload data that includes extended header and fragmentation / packing header. This fragmentation and packing extended header is shown in Figure 25. This header shall be used when MAC PDU contains single transport connection payload. The location of this header exists after the last extended header ( i.e. extended header with ‘Last’ = ‘1’) if ‘EH’ in GMH set to ‘1’ or after the GMH if ‘EH’ in GMH set to ‘0’.

Figure 25: MAC Payload Data with Extended Header & Fragmentation / Packing Header

The format of the extended header is described below.

- EHLen: 8 bits long to indicate the length of extended headers

- EHType: indicate the type of the extended header

- EHType specific data: payload specific to each extended header type

- FPH: is only present in the last extended header. When FPH = 1, indicates the presence of fragmentation / packing header; FPH = 0, no fragmentation / packing header

Figure 25 Fragmentation and packing extended header format for transport connection

The format of the fragmentation / packing header is described below.

- SN (10 bits): Payload sequence number

- FC (2 bits): Fragmentation control bits definition is given in Table 2.

- End (1 bit): If this bit set to ‘0’, another ‘Length’ and ‘End’ field are followed. If this bit set to ‘1’, reserved bits may follow for byte alignment

- FLenLength (11bits): This field represents the length of SDU/SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 length fields are present in the header

- Rsvd: Reserved bits for byte alignment.

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only First fragment of an SDU b) MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | a) MPDU with only Last fragment of an SDU b) MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only middle fragment of an SDU b) MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

Table 2 Fragmentation control information

GMH

EH = 1,  LEN = a

…

FPEH

SN = P

FC = 2,  FLEN = b

Last fragment of MAC SDU #1

FPEH

SN = 0

FC = 0,  FLEN = c

unfragmented MAC SDU #2

FPEH

SN = 0

FC = 0,  FLEN = d

unfragmented MAC SDU #3

FPEH

SN = 1

FC = 1,  FLEN = e

1st fragment MAC SDU #N

GMH

EH = 1,  LEN = a

..

FPEH

SN = x

FC = 3,  FLEN = b

Continuing fragment of MAC SDU #1

FPEH

SN = 0

FC = 0,  FLEN = c

unfragmented MAC SDU #2

GMH

EH = 1,  LEN = d

GMH

EH = 1,  LEN = f

FPEH

SN = x+1

FC = 3,  FLEN = e

Continuing fragment of MAC SDU #1

FPEH

SN = x+2

FC = 3,  FLEN = g

Continuing fragment of MAC SDU #1

MAC PDU

MAC PDU

MAC PDU

MAC PDU

a = b + c+ d + e + size(FPEH) * 4

a = b + c + size(FPEH) * 2

Figure 26: Fragmentation / Packing examples

10.12.2.2 MAC Payload with fragmentation and packing extended header

Figure 27 show a MAC payload data with fragmentation / packing header.

- EH = 1 indicates the presence of extended header and fragmentation / packing header

- EHLen = 0 means the extended header is missing.

Figure 27: MAC Payload Data with Fragmentation / Packing Header

10.12.2.3 MAC Management Message with Fragmentation extended header

Figure 28 shows the MAC management message with fragmentation / packing header.

- Flow ID = 0001

- EH = 1

- SN (10 bits): Payload sequence number

- FC (2 bits): Fragmentation control bits definition is given in Table 2.

- Rsvd: Reserved bits for byte alignment.

- FLen(11 bits): length of the MAC management message fragment

Figure 28: MAC Management Message with fragmentation / packing header

10.12.3 MAC Signaling header

Figure 29 defines the MAC signaling header that is used to send short or real time signaling messages.

- Flow ID: = 0000

- EH: = 1 to indicate the existence of MAC signaling header

- SigType: is 5 bits long, and can represent up to 32 MAC signals

- SigLen: indicates the length of MAC signaling data in bytes, 3 bits long with the maximum value of 5 to dynamically support signaling data up to 40 bits.

Figure 29: MAC Signaling Header