| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Multi-Cell PMI Coordination |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Hajime Kanzaki Seishi Hanaoka Hitachi, Ltd. | Voice: +81-42-323-1111 E-mail: hajime.kanzaki.ad@hitachi.com |
| Re: | IEEE802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” |  |
| Abstract | This contribution proposes the text for Multi-Cell PMI Coordination |  |
| Purpose | To discuss and adopt the proposed text in the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Multi-Cell PMI Coordination

Hajime Kanzaki and Seishi Hanaoka

Hitachi, Ltd.

# Introduction

This contribution proposes the text for inter-cell PMI (Precoding Matrix Index) Coordination to be included in the 802.16m amendment [1]. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9a [2], it is compliant with the 802.16m SDD [3], and it follows the style and format guidelines in [4].

# Outline of Proposal

As described in the current SDD [3] section 18, inter-cell PMI coordination is a core technique to mitigate inter-cell interference (ICI). There are Open-Loop and Closed-Loop schemes for PMI coordination. As for Open-Loop scheme, the AMSs do not report preferred/not-preferred PMIs, while the AMSs report PMIs such as described in the current SDD section 18.2.1 for Closed-Loop scheme. In this contribution, we propose the Open-Loop inter-cell PMI coordination technique.

# Open-Loop inter-cell PMI coordination

In case of downlink transmission, AMSs need to have at least one low ICI subband. ICIcan be practically is mitigated by reporting low ICI subbands from AMS and scheduling resources based on the reported results. When non-adaptive precoding technique is applied to neighboring ABSs, each AMS experiences both low ICI subbands and high ICI subbands since the precoding matrix changes every predefined contiguous physical subcarriers. Then, each AMS will have low ICI in at least one subband. Moreover, the precoding matrix can change v subframes to achieve diversity gain. If each ABS switches precoding matrices on time direction independently, the channel quality of downlink data transmission is different against the channel quality estimated by AMS. When ICI at downlink data transmission is lower than at estimated by AMS, the capacity loss is occurred because of low MCS transmission. On the other hand, when ICI at downlink data transmission is higher than at estimated by AMS, the data transmission error will occur by increased ICI.

Since these are caused by channel quality measurement error, each ABS needs to switch the precoding matrix with inter-cell coordination. The concept of proposal is shown in Figure 1 and 2. Figure 1 shows cell topology, and an example of precoding matrix switching is shown in Figure 2. The proposed scheme sets the common switching interval v and the common switching subframe between ABSs. Figure 2 shows the case where switching interval v equals to 5, and Ai, Bi, ... ,Gi (i=0,1,...) represents the PMI at subframe number i on a certain subband. Each ABS uses the same precoding matrix during 5 DL subframes, and switches it simultaneously. In this case, each ABS uses A0, B0,...,G0 during subframe number 0 to 4, and A5, B5,...,G5 during subframe number 5 to 10. AMSs can estimate channel quality during using same precoding matrix. When precoding matrix switching occurs, AMSs should estimate channel quality independently from the previous estimation because ICI experienced by AMS fluctuates. However, if switching interval v is short, channel quality estimation by AMSs can not track ICI fluctuation. Especially cell edge AMSs experiences high ICI subframes and low ICI subframes in the same subband due to large variance of ICI power. If AMS calculates the average of these estimation results in several subframes, capacity loss by involving high ICI and low ICI subframes will occur. Thus in proposed scheme, ABSs switch precoding matrix periodically as shown in Figure 2 in order to enable AMSs to estimate channel quality by periodic measurement. In this case, the same precoding matrix is used during 5 DL subframes every 4 x 5 = 20 DL subframes. The capacity loss can be reduced for AMSs to calculate the average of channel quality measurement results among subframes where the same precoding matrix is used between ABSs.

Figure 1. Cell topology

Figure 2. An example of periodic precoding matrix switching

# Proposed Text

-----------------------------------------  Start of the Text---------------------------------------------------

15.x Interference Mitigation Techniques

15.x.x Inter-cell PMI coordination

15.x.x.1 Open-Loop coordination

ABSs can support non-adaptive precoding which use the same precoding matrix among v subframes. Neighboring ABSs have common v. The precoding matrix changes in the same subframe periodically between ABSs.

15.x.x.2 Closed-Loop coordination

-----------------------------------------  End of the Text---------------------------------------------------

- References

[1] IEEE 802.16m-08/010r1a, “IEEE 802.16m Amendment Working Document,” Mar. 2009

[2] IEEE P802.16 Rev2 / D9a, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” Mar. 2009

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document,” Mar. 2009

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment,” Sep. 2008