| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Amendment Text Proposal for IEEE 802.16m Interference Mitigation Capability Negotiation |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Luciano Sarperi, Yanling Lu, Kevin Power, Mythri Hunukumbure Fujitsu | E-mail: luciano.sarperi@uk.fujitsu.com yanling.lu@cn.fujitsu.com kevin.power@uk.fujitsu.com mythri.hunukumbure@uk.fujitsu.com |
| Re: | 802.16m AWD Target topic: Interference Mitigation |  |
| Abstract | This contribution proposes amendment text for an interference mitigation capability negotiation, which forms part of the network entry capability negotiation. |  |
| Purpose | To be discussed and adopted by TGm for incorporation in the 802.16m Amendment Working Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for IEEE 802.16m Interference Mitigation

Capability Negotiation

Luciano Sarperi, Yanling Lu, Kevin Power, Mythri Hunukumbure

Fujitsu

- Introduction

This contribution proposes amendment text for an interference mitigation capability negotiation, as part of the Capability Exchange and Registration phase of the network entry procedure. The SDD [1] covers in Section 20 several interference mitigation methods, which can play an important role in improving the cell edge throughput [1], [2].

However, supporting some of these interference mitigation methods requires additional capabilities in the AMS such as channel measurements with respect to multiple ABS and therefore can increase the AMS complexity. As a consequence, a basic AMS may support only a subset of these interference mitigation methods. Furthermore, Femtocell or Microcell BSs will most likely also support only a reduced number of interference mitigation methods in order to keep their cost low. Therefore, there is a need for a capability negotiation between the AMS and the ABS with the goal that both the AMS and the ABS agree on a subset of mutually supported interference mitigation methods for which the AMS provides reports and can subsequently be employed.

- Proposed Interference Mitigation Capability Negotiation

The proposed interference mitigation capability negotiation follows the same concept as the capability negotiations in IEEE 802.16e: The AMS first informs the ABS of its supported interference mitigation methods, subsequently the ABS reports the subset of interference mitigation methods which both the ABS and AMS support back to the AMS.

This allows the ABS and AMS to employ one or multiple interference mitigation methods as long as they are part of the subset of mutually supported methods, as shown in Figure 1.

Figure 1 Interference Mitigation Capability Negotiation between AMS and ABS

For informative purposes, Table 1 provides a summary of the interference mitigation methods defined in the SDD [1] in Section 20, which are included in the proposed capability negotiation. As can be seen from the column “AMS Functionality Required”, interference mitigation specific measurements and associated reports must be supported by the AMS for a specific method. Similarly, the relevant functionality must be supported by the ABS. In case of e.g. femtocells it is expected that they will not support all interference mitigation methods due to cost restrictions or technical limitations.

Table 1 Interference Mitigation Methods in SDD [1] and Required Functionality

| # | Interference Mitigation Method | AMS Functionality Required |
| --- | --- | --- |
| 1 | FFR | Report preferred frequency partition |
| 2 | PMI Restriction | Report neighbouring cell’s restricted PMIs, CQIs and BS IDs |
| 3 | PMI Recommendation | Report neighbouring cell’s recommended PMIs, CQIs and BS IDs |
| 4 | Collaborative MIMO | Report neighbouring collaborating cell’s PMIs, CQIs and BS IDs |
| 5 | Closed-Loop Macro Diversity | Report neighbouring collaborating cell’s PMIs, CQIs and BS IDs |

The proposed text focuses on the high level requirements for the interference mitigation capability negotiation messages, while keeping the details of the exact number and type of interference mitigation methods open because the detailed interference mitigation methods have not yet been addressed in the amendment text.

- Proposed Changes in the 802.16 Amendment Working Document (802.16m-09/0010)

--------------------------------------------------------Start of Text-----------------------------------------------------------

### 15.2.x XXX-REQ (AMS capability request) message

The XXX-REQ [TBD] shall be transmitted by the AMS during the Capability Exchange and Registration phase of the network entry procedure. An AMS shall generate XXX-REQ messages in the form shown in Table xx.

Table xx-XXX-REQ message format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| XXX-REQ_Message_Format(){ | — | — |
| Management Message Type = yy [TBD] | 8 | — |
| TLV Encoded Information | variable | TLV-specific |
| } | — | — |

The following parameter shall be included in the Capability Request if the AMS intends to request ABS to perform interference mitigation when it experiences high interference levels:

Interference Mitigation Capability (See 11.x.y)

If any bit is set to 1, it means the AMS supports the corresponding interference mitigation method.

### 15.2.y XXX-RSP (AMS capability response) message

The XXX-RSP [TBD] shall be transmitted by the ABS during the Capability Exchange and Registration phase. An ABS shall generate XXX-RSP messages in the form shown in Table xx.

Table xx-XXX-RSP message format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| XXX-RSP_Message_Format(){ | — | — |
| Management Message Type = yy [TBD] | 8 | — |
| TLV Encoded Information | variable | TLV-specific |
| } | — | — |

The following parameter shall be included in the Capability Response if the ABS intends to perform interference mitigation if requested:

Interference Mitigation Capability (See 11.x.y)

If any bit is set to 1, it means the ABS supports the corresponding interference mitigation method.

### 11.x XXX-REQ/RSP management message encodings

Table xx ― XXX-REQ/RSP management message encodings

| Type | Parameter |
| --- | --- |
| … | … |
| [TBD] | Interference mitigation capability |
| … | … |

### 11.x.y Interference mitigation capability

This field indicates interference mitigation capabilities of the AMS when included with the XXX-REQ and the interference mitigation capabilities of the ABS when included with the XXX-RSP.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | 1 | [TBD] [Example: Bit #0: FFR Bit #1: PMI Restriction Bit #2: PMI Recommendation Bit #3: Collaborative MIMO Bit #4: Closed-Loop Macro Diversity Bit #5~7: Reserved] | XXX-REQ XXX-RSP |

--------------------------------------------------------End of  Text-----------------------------------------------------------

- References

[1] 	IEEE 802.16m System Description Document, IEEE 802.16m-08/003r8, 2009-04-10

[2] 	Efficient MIMO downlink intercell interference cancellation using pre-coding, IEEE C802.16m-08/449, 	2008-05-02