| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 8TX Dedicated Pilot Pattern Text Proposal for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jiann-An Tsa, Jerry Pi, Taeyoung Kim, Hai Wang Samsung Electronics Co., Ltd. | jtsai@sta.samsung.com |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes the text of 8Tx Pilot Patterns to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of 8Tx Pilot Patterns
for the IEEE 802.16m AWD

Jiann-An Tsai, Jerry Pi

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of 8Tx pilot patterns to be included in the 802.16m AWD ([5]). The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([1]), it is compliant to the 802.16m SRD ([2])). The link performance of the 8Tx pilot pattern proposals is shown in [4].

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-09/00805,” 8TX Dedicated Pilot Pattern Simulation Results”

[5] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD

-------------------------------  Text Start  ---------------------------------------------------

Insert the following text at section 15.3.5.4.1, page 47, line 54:

Figure xxx. Pilot Patterns for 8 pilot streams

The pilot patterns for eight pilot streams are shown in Figure xxx with the subcarrier index increasing from top to bottom and the OFDM symbol index increasing from left to right. Subfigure (a) in Figure xxx shows the pilot pattern for eight pilot streams in subframe with six OFDM symbols; Subfigure (b) in Figure xxx shows the pilot pattern for eight pilot streams in subframe with five OFDM symbols; Subfigure (c) in Figure xxx shows the pilot pattern for eight pilot streams in subframe with seven OFDM symbols.

-------------------------------  Text End  ---------------------------------------------------