| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for 2TX Pilot Pattern Interlacing for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jiann-An Tsa, Jerry Pi, Taeyoung Kim, Hai Wang Samsung Electronics Co., Ltd. | jtsai@sta.samsung.com |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes the text of 2Tx pilot pattern interlacing to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of 2TX Pilot Pattern Interlacing
for the IEEE 802.16m AWD

Jiann-An Tsai, Jerry Pi

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of 2Tx dedicated pilot pattern interlacing to be included in the 802.16m AWD ([4]). The interlaced pilot patterns are used to mitigate collision of dedicated pilot among neighboring BSs for transmissions with one or two streams. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([1]), it is compliant to the 802.16m SRD ([2])).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD

-------------------------------  Text Start  ---------------------------------------------------

Replace page 44, line 5 – 15, section 15.3.5.4.1, with the following text:

The interlaced pilot patterns are generated by cyclic shifting the base pilot patterns. There are 3 pilot interlaces for each pilot pattern. The pilot interlaces for pilot pattern 0 are shown in Figure 414 and the pilot interlaces for pilot pattern 1 are shown in Figure 415, respectively.

The index of the pilot interlace used by a particular BS with Cell_ID = k in a PRU with PRU_ID = j is denoted by pk, j. The index of the pilot interlace for a given mobile station with STID =m is determined by the Cell_ID and PRU_ID according to the following equation:

pk, j = mod(k + m +j, 3)                                         (189)

The index of the pilot pattern used by pilot stream i, where i = 0, 1 in a particular BS with Cell_ID = k in a PRU with PRU_ID = j is denoted by sk, j, i. The index of the pilot pattern for a given mobile station with STID =m is determined by the stream index, the Cell_ID, and the PRU_ID according to the following equation:

sk, j, i = mod(k + m + j+ i, 2)                                         (190)

.

-------------------------------  Text End  ---------------------------------------------------