| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-optimizing FFR |  |
| Date Submitted | 2009-04-28 |  |
| Source(s) | Joey Chou Intel Kun Liu Zhaohua Lu Ying Liu ZTE | E-mail: joey.chou@intel.com lu.zhaohua@zte.com.cn |
| Re: | TGm SDD: SON |  |
| Abstract | This contribution proposes text for Self-optimizing FFR. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Self-optimizing FFR

Joey Chou, Intel

Kun Liu, ZTE

- Introduction

This contribution proposes the text for Self-optimizing FFR.

- Proposed text

### 16.2.4	Self-optimizing FFR

Self-optimizing FFR is designed to automatically adjust FFR parameters, frequency partitions and power levels, among macro BS sectors in order to optimize system throughput and user experience.

The following lists the example of BS parameters that each macro BS may use to support FFR optimization and load balancing among macro BS by taking into account factors such as MS distribution, SINR distribution, resource utilization (metrics), and traffic load for each partition, BS may optimize FFR parameters and support load balancing among macro BS.

- BSID

- Total number of MS connected to a macro BS

- MS location distribution – is indicated by the mean and standard deviation of MS timing advances that are measured in the periodic ranging process.

- MS UL/DL SINR distributions per FFR partition – are indicated by the mean and standard deviation of MS UL/DL SINR that are measured on per FFR partition basis

- UL / DL traffic distribution per FFR partition – are indicated by the mean and standard deviation of UL/DL traffic load samples, on per FFR partition basis. The traffic load samples count the number of octets of MAC PDUs transmitted or received at the macro BS in a sampling interval.

- Converged resource metrics per FFR partition – see section 20.1.1.1 for resource metrics description

The following lists the example of parameters that each macro BS in the serving area may use to tune the FFR parameters for optimal performance:

- FFR partitions – frequency partitions for frequency reuse factors 1 and 1/3

- Power levels – the power level should be used in each partition

- Relative Load indicator – the average traffic of a the given macro  BS in comparison with other BS

- Time stamp for action – indicates when the change will take effective in all macro BS in the serving area

- Reference resource metric per frequency partition - utilized by the BS to optimize resource allocation