| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network entry of ARS |  |
| Date Submitted | 2009-04-24 |  |
| Source(s) | Kaibin Zhang Gang Shen Dongyao Wang Alcatel-Lucent Shanghai Bell Huiying Fang ZTE Corporation | E-mail: kaibin.zhang@alcatel-sbell.com.cn fang.huiying@zte.com.cn |
| Re: | IEEE 80216m-09_0021-Call for Comments on Project 802.16m System Description Document ” IEEE 802.16m-08/003r8” (SDD), on section 14 “Support for Multi-hop relay” |  |
| Abstract | This contribution porposes modification on the Access state description of ARS. |  |
| Purpose | To be discussed and adopted into the 802.16m SDD by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Network entry of ARS

Kaibin Zhang, Gang Shen, Dongyao Wang

Alcatel-Lucent Shanghai Bell Co., Ltd.

Huiying Fang

ZTE Corporation

# 1. Introduction

- This contribution proposes changes for the section “10.4.1.2 Access State” in IEEE 802.16m System Description Document.

- As described in the current SDD text, the network entry of ARS only includes some steps that are quite similar with that of the AMS network entry, such as ranging, pre-authentication capability negotiation, authentication and authorization, capability exchange, registration, neighbor station measurement & access station selection. However, there is one key step missed, in which the operation profile is conveyed from the superbordinate ABS to the ARS. The operation profile may include the operation parameters of the ARS, for instance, the relay model, the scheduling model and security model, parameters of DL access link, etc. And the ABS may send ARS the information of neighbor stations as well to facilitate the neighbor station measurement in the following step. Therefore, the access state transition process and the related diagram need to be updated.

- It is proposed to accept the text changes as follows.

# 2. Text Proposal

----------------------------------------------------------Start of text ---------------------------------------------------------

#### 14.4.1.2Access State

The ARS performs network entry with the target ABS while in the Access state. Network entry is a multi step process consisting of ranging, pre-authentication capability negotiation, authentication and authorization, capability exchange, registration, acquisition of operation profile, neighbor station measurement & access station selection (optional), and ARS operation. The ARS receives its Station ID and transitions to the Operational state. Upon failure to complete any one of the steps of network entry the ARS transitions to the Initialization state.

Figure 1 Access State Transition Diagram of IEEE802.16m Relay

----------------------------------------------------------End of text ---------------------------------------------------------