| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SDD chapter 16 - Support for Self-organization |  |
| Date Submitted | 2009-05-03 |  |
| Source(s) | Roberto Albanese, Andrea Bacioccola Nokia | E-mail: roberto.albanese@nokia.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: SDD comments - Chapter 16 (Support for Self-organization) |  |
| Abstract | Proposed text for SON chapter in the SDD document |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SDD chapter 16 - Support for Self-organization

Roberto Albanese, Andrea Bacioccola

Nokia

- Introduction

The contribution proposes text for Section 16 “Support for Self-organization” in 802.16m SDD. The proposed text is developed so that it can be readily combined with the 802.16m SDD [1].

- References

[1] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

### Change Section 16 according to the following text

---------start of the text -----------------------------

16.1.2 Neighbor Discovery

The initial of neighbor list is obtained from core network automatically. Any change of the neighbor environment such as BSs are added or removed should automatically trigger the BS to generate an updated neighbor list. The information for updating the neighbor list (e.g. macro BS, Femtocell BS) is collected by BS/RS/MS measurement, core network, inter-BS network signaling, BS’s own management.

The BS should direct an MS to report perform the frequency measurements of serving BS and/or non-serving BS (e.g. inter-RAT neighbor cell measurement may be based on MS traffic conditions). and BS may use cached and or feedback information on signal strength, BSID and some additional information, e. g. MS position, battery status and report history for a certain MS, in order to reduce the undesirable transmission from the MS (e.g. BS may select a subset of MSs to perform measurements and produce reports).

---------end of the text -----------------------------