| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for Uplink Open-Loop Power Control |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Rongzhen Yang, Apostolos Papathanassiou, Wei Guan, Ali T. Koc, Hujun Yin, Yang-seok Choi Intel Corporation Jeongho Park, Suryong Jeong, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Dong-cheol Kim, Wookbong Lee, HanGyu Cho LG Electronics Jihyung Kim, Dong Seung Kwon ETRI Xiaoyi Wang Nokia Siemens Networks Zexian Li Nokia | apostolos.papathanassiou@intel.com rongzhen.yang@intel.com jeongho.jh.park@Samsung.com hgcho@lge.com savant21@etri.re.kr xiaoyi.wang@nsn.com zexian.li@nokia.com |
| Re: | Category: AWD DG comment / Area: Power control/Link adaptation DG “Comments on C80216m-09/866: 15.3.x.2.1 (Uplink OLPC)” |  |
| Abstract | The contribution proposes the text of uplink OLPC section to be included in the 802.16m AWD |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for Uplink Open-Loop Power Control

Rongzhen Yang, Apostolos Papathanassiou,

Wei Guan, Ali T. Koc, Hujun Yin, Yang-seok Choi

Intel Corporation

Jeongho Park, Suryong Jeong, Jaehee Cho, Heewon Kang, Hokyu Choi

Samsung Electronics

Dong-cheol Kim, Wookbong Lee, HanGyu Cho

LG Electronics

Jihyung Kim, Dong Seung Kwon

ETRI

Xiaoyi Wang

Nokia Siemens Networks

Zexian Li

Nokia

- Introduction

The contribution proposes text for the uplink OLPC section to be included in the 802.16m amendment ([1]).

- References

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[3] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m AWD

------------------------------- Text Start ---------------------------------------------------

## 15.3 Physical layer

## 15.3.x Power Control

- Downlink Power Control

- Uplink Power Control

  - UL Open-Loop Power Control

When the open-loop power control is used, the power per subcarrier and per transmission antenna shall be maintained for the UL transmission as follows

P(dBm)=L+SINRTarget+NI+OffsetAMSperAMS+OffsetABSperAMS	Equation 1

Where

SINRTarget is the target uplink SINR received by the ABS. The mode used to calculate this value is signaled through a power control message.

P is the TX power level (dBm) per subcarrier for the current transmission.

L is the estimated average current UL propagation loss. It shall include AMS’s Tx antenna gain and path loss.

NI is the estimated average power level (dBm) of the noise and interference per subcarrier at the ABS, not including ABS’s Rx antenna gain.

OffsetAMSperAMS is a correction term for AMS-specific power offset. It is controlled by the AMS. Its initial value is zero.

OffsetABSperAMS is a correction term for AMS-specific power offset. It is controlled by the ABS through power control messages.

The estimated average current UL propagation loss, L, shall be calculated based on the total power received on the active subcarriers of the frame preamble.

When the user connects to network, it can negotiate the parameters for the calculation of SINRTarget which is defined as:

SINRTarget=SINROPT, OLPC Mode 1CN−10log10(R), OLPC Mode 2		Equation 2

Where

C⁄N is the normalized C/N of the modulation/FEC rate for the current transmission, as appearing in Table 1.

R is the number of repetitions for the modulation/FEC rate

SINROPT is the target SINR value for IoT control and tradeoff between overall system throughput and cell edge performance, decided by the control parameter [γ or Δ] and SINRMIN:

SINROPT= [

10log10 max10(SINRMIN10), γ×SIRDL−1Nr

or

max SINRMIN, min(CN−10log10(R),ΔIoT+10log10(SIRDL))

]

Where

SINRMIN is the SINR requirement for the minimum rate expected by ABS which is set by a unicast power control message. SINRMIN has 4 bits to represent the value in dB among {-3, -2.5, -2, -1.5, -1, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5} (TBD)

[γ is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}

ΔIoT is the parameter for IoT control and broadcast by the ABS. It has 3 or 4 bits and the exact values in dB are TBD. It can also be different for each frequency partition.]

[Nr is the number of receive antennas at the ABS]

SIRDL is the ratio of the downlink signal vs. interference power, measured by the AMS.

Table 1 – Normalized C/N per modulation

| Modulation/FEC rate | Required C/N |
| --- | --- |
| ACK/NAK | -3.0 |
| CQI | 0 |
| MAP ACK/NAK | 0 |
| Ranging code | 3 |
| QPSK 1/3 | 0.5 |
| QPSK 1/2 | 6 |
| QPSK 2/3 | 7.5 |
| QPSK 3/4 | 9 |
| 16-QAM-1/2 | 12 |
| 16-QAM-2/3 | 14.5 |
| 16-QAM-3/4 | 15 |
| 16-QAM-5/6 | 17.5 |
| 64-QAM-1/2 | 18 |
| 64-QAM-2/3 | 20 |
| 64-QAM-3/4 | 21 |
| 64-QAM-5/6 | 23 |

------------------------------- Text End ---------------------------------------------------