| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Connection Management and QoS DG AWD text with DG comment resolution |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Xiangying Yang Connection Management/QoS DG Chair | E-mail: Xiangying.yang@intel.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Category: AWD – DG Comment / Area: Connection Management/QoS DG |  |
| Abstract | This contribution proposes IEEE 802.16m AWD text on Connection Management and QoS |  |
| Purpose | To be discussed in 802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

QoS DG AWD

List of latest contributions

| Contribution | REV | TITLE | AUTHORS | AFFILIATION | note |
| --- | --- | --- | --- | --- | --- |
| C80216m-09_0689 |  | Bandwidth Request Procedure | Hua Xu, Stavros Tzavidas, Igor Filipovich¦ | Motorola, Inc | BWREQ-request |
| C80216m-09_0664 |  | Enhanced Bandwidth Grant | Kuk-Jin Song, Yerang Hur, Hyung-Joon Jeon, Seunghyun Lee, Jaehyeong Kim, Kyounghwan Lee, Bong Ho Kim | Posdata | BWREQ-grant |
| C80216m-09_0633 |  | Proposed Text for Connection Management | Sang-Heon Shin, Bong Ho Kim, Yerang Hur | POSDATA | connection |
| C80216m-09_0609 |  | Proposed contents of the Quick Access Message in the bandwidth request procedure | Jaehyuk Jang, Hwasun Yoo, Rakesh Taori, Jung Je Son | Samsung Electronics Co., Ltd. | BWREQ-request |
| C80216m-09_0596 C80216m-09_QoS_r1_Samsung | r1 | Bandwidth request scheme to support multiple connection in an AMS | Okseon Lee;Jaehyuk Jang;Sungjin Lee | Samsung Electronics Co., Ltd. | BWREQ-request |
| C80216m-09_0592 |  | Proposed Amendment Text for Addressing and Connections | Dongyao Wang, Gang Shen, Kaibin Zhang, Shan Jin | Alcatel-Lucent Shanghai Bell | connection |
| C80216m-09_0523 |  | Blocking before Network Access -- Based on QoS | John Harris, Hua Xu | Motorola, Inc | BWREQ-request |
| C80216m-09_0522 | r1 | Update QoS Parameters | Hua Xu | Motorola, Inc | QoS |
| C80216m-09_0518 C80216m_09_QoS_Content_NNSNComments |  | Connection Management in 16m | Haihong Zheng, Shashikant Maheshwari, Yousuf Saifullah, Zexian Li, Jan Suumaki | NSN, Nokia | connection |
| C80216m-09_0508 | R4 | Proposed Text for the QoS and Connection Management Enhancements for the IEEE 802.16m Amendment | Jie Hui, et. al. | Intel, LGE, MediaTek, Nokia, NSN, Motorola | QoS |
| C80216m-09_QuickAccessMessage_III_r1 |  | Quick Access Message for IEEE 802.16m | Kanchei (Ken) Loa, Yi-Ting Lin, Yi-Hsueh Tsai, Chun-Yen Hsu, Youn-Tai Lee, Tsung-Yu Tsai, Chiu-Wen Chen, Jiun-Je Jian | Institute for Information Industry(III) | BWREQ-request |
| C80216m-09_BWREQ_MediaTek_r2 C80216m-09_QoS_r1_mtk |  | Proposed Amendment Text on 802.16m Contention-Based Bandwidth Request Procedure | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng | MediaTek | BWREQ-request |
| C80216m-09_QuickAccessMessage_Intel |  | IEEE 802.16m Quick Access Message | Yuan Zhu, Hui Jie | Intel | BWREQ-request |
| C80216m-09_QoS_DG_Contents_LGE |  | Proposed Text Related to Bandwidth Request Procedure for the IEEE 802.16m Amendment | Heejeong Cho, Kiseon Ryu, Aeran Youn, Doohyun Sung, Yongho Kim, Jin Sam Kwak | LG Electronics | BWREQ-request |
| C80216m-09_QoS_r1_LGE_MTK |  | Comment on QoS DG AWD 1st Draft on BR Procedures | Kelvin Chou, et. al. | MediaTek/LGE | BWREQ-request |
| Amendment Text on contention based BW (Fujitsu)) |  | Amendment Text on Contention Based Random Access Bandwidth Request | Yanling Lu, Masato Okuda, Kevin Power | Fujitsu | BWREQ-request |
| C80216m-09_QoS_LGE_NSN_Nokia |  | QoS DG AWD | Aeran Youn et. al. | Nokia/NSN/LGE | QoS |
| C80216m-09_QoSr1_aGPS_comments_r2 |  | QoS and Connection management in 802.16m | Jie Hui et. al. | Intel/MediaTek/ITRI/Fujitsu | QoS |
| C80216m-09_QoS_r2_BWREQheader |  | N/A | Jie Hui | Intel | BWREQ-reuest |

# Introduction

This contribution proposes amendment text on Connection Management and Quality of Service (QoS) support.

Note:

The [bracket text] has not been fully agreed among the contributor. Further discussion is needed for these text.

The strikethrough text is to be removed based on DG discussion or chair decision.

# ToC

15.2.x Connection Management

15.2.x.1 Management connections

15.2.x.2 Transport connections

15.2.y Bandwidth Request and Allocation Mechanism

15.2.y.1 Bandwidth Request

15.2.y.1.1 Contention-based random access bandwidth request

15.2.y.1.2 Standalone Bandwidth Request Signaling

15.2.y.1.3 Piggybacked Bandwidth Request

15.2.y.1.4 Bandwidth Request using FFB

15.2.y.1.5 Bandwidth request message format

15.2.y.1.5.1 quick access message format

15.2.y.1.5.2 bandwidth request header format

15.2.y.1.5.3 Piggyback bandwidth request extended header format

15.2.y.2 Grant

15.2.z Quality of Service (QoS)

15.2.z.1 Service classes

15.2.z.2 Service Flow Management

15.2.z.3 Scheduling services

15.2.z.3.1 Adaptive granting and polling service

15.2.z.4 Emergency Service Flow

15.2.z.4.1.1 Emergency Service Notification during initial ranging

15.2.z.4.1.2 Emergency Service Notification during connected state

# Text Proposal

================= Start of Proposed Text ========================

15.2.x Connection Management

Connection is a mapping between MAC peers of an ABS and one or more AMSs. When the mapping applies to ABS and one AMS, the connection is a unicast connection. Otherwise it is a multicast or broadcast connection. Unicast connections are identified by the combination of a 12-bit STID and a 4-bit FID. Multicast and broadcast connections are identified by the reserved STIDs.

Two types of connections are used – management connections and transport connections. Management connections are used to carry MAC management messages. Transport connections are used to carry user data including upper layer signaling messages such as DHCP, etc and data plane signaling such as ARQ feedback. MAC management message shall never be transferred over transport connection, and user data shall never be transferred over management connections.

15.2.x..1 Management connections

Two pairs of bi-directional unicast management connections – [basic connection and primary management connection] [encrypted and non-encrypted management connections], are automatically established when an AMS performs initial network entry. [The basic connection is used by the ABS MAC and AMS MAC to exchange short, time-urgent MAC management messages. The primary management connection is used by the ABS MAC and AMS MAC to exchange longer, more delay-tolerant MAC management messages.] [FID with value 0 and 1 are reserved for these two management connections respectively.] {CC#3: let us add security issue later if later decision in security section is inconsistent with our text.}

Once the STID is allocated to the AMS, the [basic and primary] management connections are established automatically. FID for the management connection shall never be changed during WirelessMAN-OFDMA Advanced System handover or network reentry.

15.2.x.2 Transport connections

All the user data communications are in the context of transport connections. A transport connection is uni-directional and established with unique FID assigned using DSA procedure per section 15.2.z.3. Each transport connection is associated with an active service flow to provide various levels of QoS required by the service flow. The transport connection is established when the associated active service flow is created, and released when the associated service flow becomes non-active. Once established, the FID of the transport connection is not changed during WirelessMAN-OFDMA Advanced System handover.

To reduce bandwidth usage, the ABS and AMS may establish/change/release multiple connections using a single message transaction on a management connection.

Transport connections can be pre-provisioned or dynamically created. Pre-provisioned connections are those established by system for an AMS during the AMS network entry. On the other hand, ABS or AMS can create new connections dynamically if required. A connection can be created, changed, or torn down on demand.

15.2.y Bandwidth Request and Allocation Mechanism

15.2.y.1 Bandwidth Request

Bandwidth Requests (BR) refer to the mechanism that AMSs use to indicate to the ABS that they need UL bandwidth allocation. The AMS shall either use a contention-based [or non-contention based] random access based BR indicator and an optional quick access message on BR channel, a standalone bandwidth request, a piggybacked bandwidth request carried in an Extended Header in the MPDU or a bandwidth request using fast feedback channel. Bandwidth requests in the standalone bandwidth request and a piggybacked bandwidth request shall be made in terms of the number of bytes [LRUs] needed to carry the MAC PDU excluding PHY overhead. {CC#3: Xiangying/Jack/Lei: bytes makes more sense, LRU has resolution issue. Shashi: MS may save additional BWREQ if it expects more resource than it needs. Note if we go with LRU, we may have to always use LRU for all BWREQ so that we don’t introduce yet another option.}

An AMS requests UL bandwidth on a per-connection basis. In addition, the AMS [may][can] request bandwidth for multiple connections in one [piggyback] [standalone] BR.

15.2.y.1.1 Contention-based random access bandwidth request

Contention-based random access bandwidth request follows the procedure defined in section 15.2.x.

[

When an AMS has information to send and wants to enter the contention resolution process, the AMS checks if the information the AMS has to send is for an access class with priority higher than or equal to the minimum access class advertised by BR channel configuration within the [A-MAP][UCD message]. If it is not (the minimum access class is not sufficiently low such that the AMS access class is allowed), then the AMS must wait until the BR channel configuration within [A-MAP] [UCD]advertises a minimum access class, which is less than or equal to the access class of the data and the AMS. When the AMS access class is allowed, the AMS sets its internal backoff window equal to the Request (or Ranging for initial ranging).

]{CC#3:AMAP may not have space for this due to fixed-size Non-user specific region. If this in SFH, this may cause extra latency if it impacts change counts. Keep this text in bracket. We may achieve contention resolution based on different (QoS) classes with different timers.}

Contention-based random access bandwidth request follows the procedure defined in section 15.2.x.

The 3-step random access based BR procedure is illustrated in Figure X1. At step 1, the AMS transmits a BR indicator and a quick access message on a randomly selected slot within the bandwidth request channel. At step 2, the ABS may transmit explicit acknowledgement to AMS to indicate its success of decoding BR indicator or Quick access message in the next DL frame as a response to previous bandwidth request channel opportunity. If ABS fails to decode BR indicator or Quick access message, ABS shall transmit BW ACK A-MAP IE in the next DL frame indicating the negative acknowledgement for the corresponding bandwidth request opportunity. BW ACK A-MAP IE specifies to the decoding status of BR opportunity in the previous UL frame.

AMS shall start BR timer upon one of the following conditions:

- AMS does not receive any Negative-ACK or BW grant (i.e., implicit ACK);

- Received BW ACK A-MAP IE indicating Quick access message decoding failure (with BR indicator successfully decoded) at the BR opportunity used by the AMS

On the first condition, the BR Timer value is the differentiated value defined for the service flow based on e.g. service type and priority. The differentiated values are transmitted in the DSx transaction. On the second condition, the BR Timer value is fixed.

AMS shall stop BR timer upon one of the following conditions:

- Received either UL Assignment A-MAP IE or CDMA_Allocation A-MAP IE. In this case AMS processes the UL grant.

- Received BW ACK A-MAP IE indicating BR Indicator decoding failure. In this case AMS may restart the BR procedure.

If BR timer expires and AMS receives neither UL grant nor BW ACK A-MAP IE from the ABS, AMS considers the BR was failed and may restart the BR procedure.

Figure X1 — 3-step random access BR procedure

In case the quick access message is or not decodable, a 5-step BR procedure as a fallback mode for the 3-step BR procedure is illustrated in Figure X2. The AMS obtains resource grant from ABS using CDMA Allocation A-MAP IE at step 2 and transmits a new BW-REQ message at step 3. ABS may allocate UL grant for uplink transmission at step 4. The AMS then performs its uplink transmission at step 5. After step 3, the AMS shall start the BR timer with the differentiated value defined during DSx transaction. The ABS shall transmit a negative acknowledgement for the corresponding BW-REQ message in case it is not decoded in step 3.

The AMS may restart the BR procedure if timer is expired.

{ CC#4. Step-3 may be not a BW-REQ message, as specified in Rev.2 “CDMA IE allocation”. Please comment whether this feature is needed in 16m. }

Figure X2 — Example of 5-step random access BR procedure as a fallback mode for the 3-step BR procedure

In 5-step regular random access BR procedure, AMS sends a bandwidth request indicator only. After step 2, only fixed value timer is activated. The rest of the BR procedure shall be the same as the 5-step procedure in Figure X2.

{Editor’s note: two proposed text from MTK/Fujitsu on acknowledgment A-MAP IE. Suggest we leave these after we decide on the 3 BWREQ acknowledgement/retry text above.}

[{Editor’s note: text #1 Proposed text from MTK}

The BR Acknowledgement (BR-ACK) A-MAP IE indicates the BR status of all BR opportunities in the previous UL frame, as illustrated in Figure X3.

Figure X3 – Example of a BR-ACK A-MAP IE indicating the decoding status of previous BR opportunities

Table xx – BR-Acknowledgement (BR-ACK) A-MAP IE Format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| BR_Acknowledgement_A-MAP IE() { | - | - |
| BR subframe bitmap | 8 | Bitmap to indicate whether the subframe in the previous frame contains ranging opportunity or not |
| for (i=0; i<N; i++) { | - | N: number of 1s in the BR subframe bitmap |
| Number of BR opportunities (M) | 2 | Number of BR opportunities in the previous corresponding UL subframe |
| for (j=0; j<M, j++) { | - | - |
| BR Status indicator | 1 | The MSB of the status indicator represents the BR code decoding status. 1: at least one BR preamble code is successfully decoded. 0: no BR preamble code is decoded. |
| If (BR Status indicator == 1) { |  |  |
| Number of received codes (L) |  | Number of received codes |
| For (k=0; k<L; k++) { |  |  |
| Received code | Optional ? | Received codes |
| } |  |  |
| } |  |  |
| } | - | - |
| } | - | - |

]

{Editor’s note: Proposed text #4 from Fujitsu}

[The AMS transmits a BR indicator and a quick access message on a randomly selected slot within the bandwidth request channel.

The ABS should respond with BW_ACK IE and a BW-RSP message in a subsequent and predefined DL subframe. Each bit of the BW ACK bitmap within BW_ACK IE corresponds to one opportunity in a previous UL subframe. All the successfully decoded sequences shall be listed in the BW-RSP message. If necessary, the ABS may include the bandwidth grant in the BW-RSP message.

The AMS should detect the BW_ACK IE after a predefined interval. If it finds the bit corresponding to itself is “0”, it should perform backoff algorithms and retry process, otherwise it should decode BW-RSP message.

In the BW-RSP message, if the AMS finds the sequence transmitted by itself is not listed, it should perform backoff algorithms and retry process, otherwise, the AMS should check whether or not a BW grant is included in the BW-RSP message. If the requested UL bandwidth is currently available, the bandwidth grant will be issued via BW-RSP message and may be used for a scheduled data transmission (in 3-step case) or bandwidth request message (in 5-step case). If the requested UL bandwidth is currently unavailable, the AMS may receive the resource allocation by a UL Assignment A-MAP IE (in 3-step case)or a CDMA_Allocation_A-MAP IE (in 5-step case).

Upon successfully decoding BW-RSP message and in the case where no bandwidth grant is included, the AMS should start a Timer (Tn) which has a predefined value. If the AMS receives BW grant via CDMA_Allocation_A-MAP IE or UL Assignment A-MAP IE before Tn expires, then the timer shall be deactivated.  If the AMS does not receive BW grant before Tn expires, the AMS should perform a backoff algorithm and attempt the retry procedure.

Table xx defines the format of BW_ACK IE. This IE is applied for quick indication of decoding status for the bandwidth sequence to reduce latency of contention based BW request. This BW_ACK IE is linked to a previous UL subframe.

Table xx –BW_ACK IE

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| BW_ACK IE() { | - | - |
| BW ACK Bitmap | TBD | Size depends on number of the BW slots in one sub-frame If anyone bit is set 1, it indicates in the previous uplink sub-frame, a minimum of one BR sequence was successfully decoded |
| } | - | - |

BW ACK Bitmap (BAB)

If anyone bit is set 1, it indicates in the previous uplink sub-frame, a minimum of one BR sequence was successfully decoded on a particular BR slot

Table xxx defines the BW-RSP message. This message is used to inform the AMS of the decoding status and may also provide a BW grant for the AMS

Table xxx BW-RSP Message Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| BW ACK Message () { | - | - |
| for (n=0; n < BAB Size; n++) { |  | BAB = BW ACK Bitmap from BW_ACK IE |
| if (BAB [n] == 1 { |  | - |
| Received Sequence count | 2 | The number of sequences successfully received in the same slot |
| For ( i=0; i< Received Sequence count; i++){ | - | - |
| Sequence Index | 8(FFS) | Identifies the sequence |
| BW Grant Indicator | 1 | Indicate whether or not the BW grant is included. 1 =yes,0= no. |
| If (Indicator ==1){ |  |  |
| Resource Assignment Information | TBD | Size of Resource Allocation sub fields depends on system bandwidth or frequency partition size and number of concatenated subframes allocated by this IE. |
| MCS | 5 | Modulation and coding scheme used for the burst |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding bits |  |  |
| } | - | - |

]

15.2.y.1.2 Standalone Bandwidth Request Header

Standalone bandwidth request header is used by AMS to send bandwidth request in step 3 in the 5-step contention-baesd random access BR procedure, or as a response to the polling from the ABS. The AMS can use any UL resource allocated to itself to send the standalone BR header.

15.2.y.1.3 Piggybacked Bandwidth Request

Piggybacked bandwidth request is used by the AMS to request for bandwidth for the same or different connection, to which the user data in the MPDU is mapped to. It is carried in the extended header.

15.2.y.1.4 Bandwidth Request using FFB

15.2.y.1.5 Bandwidth request message format

15.2.y.1.5.1 quick access message format

When 3-step BW-REQ procedure is used, in step-1 the quick access message carries 12 bit information including MS addressing information with [12][16][24] TBD quick access sequences carrying additional BW-REQ information. The information bits are defined in the following format.

[{NSN/Nokia/Samsung}

The 12-bit quick access message shall contain 12-bit STID of the AMS that requests for the bandwidth, and 4-bit Request_Size_Indicator in quick access sequence. The Request_Size_Indicator indicates the size of the requested bandwidth and the priority of the request. Each value and the correspondent meaning of Request_Size_Indicator is provided using [AAI_RNG-RSP or AAI_DSA-REQ/RSP][AAI_DSx-REQ/RSP].

| Name | Length (bit) | Description |
| --- | --- | --- |
| STID | 12 | Station Identifier. |
| Request_Size_Indicator | 4 | Index of bandwidth request size. The index of specific services for UL bandwidth request. Detailed parameters (e.g., size, QoS) are negotiated using the AAI_DSx-REQ/RSP message. |

] {NSN/Nokia/Samsung}

[{Intel/MTK}

Both the 12-bit quick access message and the selected BWREQ preamble index out of total 24 (FFS) sequences carry BWREQ related information.

The 10bit MS BWREQ-ID is carried in quick access message. The BWREQ-ID is allocated during network entry. The quick access sequence selection is generated by a pseudo-random function (taking values from 0~23) of frame number, BWREQ-ID and input Selection-ID (also taking values from 0~23). Given a frame number and BWREQ-ID, the quick access sequence is a one-to-one mapping function of Selection-ID.

The remaining 2 bits in quick access message are used differently based on Selection-ID as explained below.

- When AMS uses quick access message to request bandwidth, it sets Selection-ID to be the corresponding F-ID. The 2 remaining bits in quick access message carries request size information, where the size granularity is defined during connection setup for this flow.

- When AMS uses quick access message to request aGPS adaptation for a particular F-ID, it sets the Selection-ID to be a reserved value (13~23) negotiated during connection setup, and indicates the adaptation request in the 2 remaining bits in quick access message.

]{Intel/MTK}

[{LGE}

24 BWREQ sequences are divided into 3 subsets.

These subsets are Priority_Class1_Sequence, Priority_Class2_Sequence and Priority_Class3_Sequence. Priority_Class1_Sequence, Priority_Class2_Sequence are used only for the 3-step BR procedure and Priority_Class3_Sequence is used only for the 5-step BR procedure. The number of BWREQ sequences of each subset is transmitted in S-SFH. 12-bit STID is carried in the quick access message and bandwidth request size is carried in BWREQ sequence.

]{LGE}

[{III}

Both the 12-bit quick access message and the BW REQ preamble index out of total 24 (FFS) sequences carry the FID information. The transmitted BW REQ preamble has a mapping to the requesting FID based on a synchronized random number generator(RNG) maintained in the ABS and the AMS. The algorithm of RNG is FFS. In the quick access message, the 2-bit BW REQ info. and 10-bit CRC are included. The 10-bit CRC is generated by the reduced ID(or 10-bit LSB STID) and 2-bit BW REQ info. for error detection and assisting the ABS to decode the requesting station. The 2-bits BW REQ information in the quick access message is the requesting index of a given flow and can be assigned differently by different flows. The mapping between requesting index and associated requested BW is negotiated during flow establishing or service change. If the FID is associated with adaptive grant and polling service (aGPS) scheduling class, the requesting index is used for QoS parameter adaptation. If the FID is associated with non-aGPS scheduling class, the requesting index is used for indicating the BW REQ size.

|  | Length (bit) | Description |  | Note |
| --- | --- | --- | --- | --- |
| BW REQ Info. | 2 | BW REQ size(2-bits) | Identifies the number of (LRUs * lowest MCS) requested by a flow. The mapping between requesting index and associated requested BW is negotiated during flow establishing or service change | For the common transport connections |
|  |  | SF QoS Parameter Set Flag (for aGPS) (1-bit, 1-bits reserved) | Switches the QoS parameter sets of aGPS: 0: Primary SF QoS parameter set 1: Secondary SF QoS parameter set | Only for the aGPS connections |
| CRC | 10 | Generated by an AMS’s 10-bit LSB STID and Info. |  |  |

]{III}

{CC#3: CRC protection may still be limited. Need to investigate its benefit more carefully. Is there a way to show it is anyway better than putting a non-scrambled ID?}

15.2.y.1.5.2 bandwidth request header format

{Editor’s note: limit the scope to functionality only. Defer specific header format definition.}

[ {NSN/Nokia/Intel}

When the standalone Bandwidth Request Header is transmitted in step 3 in the 5-step contention-based random access BR procedure, it shall contain the following parameters:

- STID of the AMS

- FID of the requesting connection

- Aggregate bandwidth to request

When the standalone Bandwidth Request Header is transmitted using the UL grant specifically for the AMS (e.g., polling from ABS), it may request bandwidth for one flow or multiple flows if allowed by space; it may request grant interval change for aGPS or BE, and it may contain the following parameters. ABS shall use the size of single flow standalone bandwidth request header for polling allocation.

- FID of the requesting connection

- Aggregate or incremental bandwidth to request for one or multiple flows

- Grant interval for aGPS or BE

] {NSN/Nokia/Intel}

[{NSN/Nokia}

When the standalone Bandwidth Request Header is transmitted in step 3 in the 5-step contention-based random access BR procedure, it is termed as BR Header Type 1 and its format is shown in table x.

Table x Format of Bandwidth Request Header Type 1

| Field | Length (bit) | Description |
| --- | --- | --- |
| Station ID | 12 | Station ID of the AMS |
| Flow ID | 4 | Flow ID of the connection requesting for bandwidth |
| Bandwidth Request Size | 11 | Aggregated bandwidth request |
| if (scheduling service == aGPS or BE) { |  |  |
| Indication | 1 | Indicating of the presence of the Grant Interval field and its menaing 0 – If the scheduling service is aGPS, indicating request to switch between primary and secondary service flow parameters, and Grant Interval field is not valid; If the scheduling service is BE, indicating Grant Interval field is not valid. 1 – if the scheduling service is aGPS, indicating the Grant Interval field is valid and the new GPI and Grant to be used for the service flow are carried in Grant Interval and Bandwidth Request Size fields,; if the scheduling service is BE, indicating the Grant Interval field is valid and the interval for the next grant and the grant size are carried in Grant Interval and Bandwidth Request Size fields, |
| Grant Interval | 4 | New GPI for aGPS service and new next grant interval for BE service |
| } else { |  |  |
| Reserved | 5 |  |
| } |  |  |

When the standalone Bandwidth Request Header is transmitted using the UL grant specifically for the AMS (e.g., polling from ABS), it is termed as Bandwidth Request Header Type 2. It may contain bandwidth request for one flow or multiple connections. It may contain the following parameters.

- FID of the requesting connection

- Aggregate bandwidth to request

- Grant interval for aGPS or BE

] {NSN/Nokia}

[{Intel}

The Standalone Bandwidth Request Header message Type 0 to be used in bandwidth request procedure step 3 is defined in the following table.

| Field | Length (bit) | Description |
| --- | --- | --- |
| Bandwidth request type = 0 | 1 | BW-REQ with STID to be used in bandwidth request procedure step 3 |
| STID | 12 | Station identifier |
| Flow ID | 4 |  |
| Bandwidth request size | 11 | Aggregated bandwidth request |
| Reserved | 4 |  |

The Standalone Bandwidth Request Header message Type 1 to be transmitted using the UL grant specifically for the AMS is defined in the following table. The size is variable from 3 bytes for single flow and up to 11 bytes for 5 flows.

| Field | Length (bit) | Description |
| --- | --- | --- |
| Bandwidth request type = 1 | 1 | BW-REQ without STID |
| Multiple-Flow Indicator (MFI) | 1 | 0: single flow; 1: multiple flow |
| If MFI ==0 { |  |  |
| Flow ID | 4 |  |
| BR type | 1 | 0 : aggregate BR; 1 : incremental BR |
| Bandwidth request size | 11 |  |
| Grant Interval | 6 | 0: the field is not used 1~2^6-1: If (scheduling type == BE) minimum grant interval (frames) for the next allocation If (scheduling type == aGPS) new GPI (frames) to use for future allocation |
| } |  |  |
| If MFI==1 { |  |  |
| Number of Flows | 2 | 0b00: 2 flow; 0b01: 3 flows; 0b10: 4 flows; 0b11: 5 flows. |
| Below fields will be repeated for number of flows | variable |  |
| { Flow ID | 4 |  |
| BR type | 1 | 0 : aggregate BR; 1 : incremental BR |
| Bandwidth request size } | 11 |  |
| } |  |  |

] {Intel}

[{Samsung}

Standalone bandwidth request header has several types and each type has different properties.

Table xx – Stand-alone Bandwidth Request Header type

| BR type | Name | BR message size without STID (byte) | BR message size with STID (byte) |
| --- | --- | --- | --- |
| 1 | BR for the AMS | 2 | 4 |
| 2 | BR for the connection | 3 | 4 |
| 3 | BR for multiple connections | 1 + 2*(Number of BR) | 3 + 2*(Number of BR) |
| 4 | BR for multiple connections with BR FID MAP | 4 | 6 |

BR type 1 is the BR for the AMS used when the AMS requests bandwidth for all the connections serviced in the AMS.

Table xx – BR type 1: BR for the AMS

| Field | Length (bit) | Description |
| --- | --- | --- |
| BR type | 2 | 00: BR for the AMS |
| Request type | 1 | 0: aggregate, 1: incremental |
| STID indicator | 1 | 0: Not included, 1: included |
| BR size | 11 | Amount of bandwidth requested |
| If STID indicator = = 0{ |  |  |
| Reserved | 1 | Shall be set to zero |
| } |  |  |
| If STID indicator = = 1{ |  |  |
| STID | 12 | STID |
| Reserved | 5 | Shall be set to zero |
| } |  |  |

(a) BR for the AMS without STID

(b) BR for the AMS with STID

Figure xx. BR type 1: BR for the AMS

BR type 2 is the BR used when the AMS requests bandwidth for a specific connection.

Table xx – BR type 2: BR for a single connection

| Field | Length (bit) | Description |
| --- | --- | --- |
| BR type | 2 | 01: BR for a single connection |
| Request type | 1 | 0: aggregate, 1: incremental |
| STID indicator | 1 | 0: Not included, 1: included |
| FID | 4 | Flow identifier |
| BR size | 11 | Amount of bandwidth requested |
| If STID indicator = = 0{ |  |  |
| Reserved | 5 | Shall be set to zero |
| } |  |  |
| If STID indicator = = 1{ |  |  |
| STID | 12 | STID |
| Reserved | 1 | Shall be set to zero |
| } |  |  |

(a) BR for a single connection without STID

(b) BR for a single connection with STID

Figure xx. BR type 2: BR for a single connection

BR type 3 is the BR used when the AMS requests bandwidth for multiple connections.

Table xx – BR type 3: BR for multiple connections

| Field | Length (bit) | Description |
| --- | --- | --- |
| BR type | 2 | 10: BR for multiple connections |
| Reserved | 1 | Shall be set to zero |
| STID indicator | 1 | 0: Not included, 1: included |
| Number of BR | 4 | The number of connections to request bandwidth |
| For (i=0; i<Number of BR; i++){ |  |  |
| Request type | 1 | 0: aggregate, 1: incremental |
| FID | 4 | Flow identifier |
| BR size | 11 | Amount of bandwidth requested |
| } |  |  |
| If STID indicator = = 1{ |  |  |
| STID | 12 | STID |
| Reserved | 4 | Shall be set to zero |
| } |  |  |

(a) BR for multiple connections without STID

(b) BR for multiple connections with STID

Figure xx. BR type 3: BR for multiple connections – An example when the number of BR is 2.

BR type 4 is another BR type for multiple connections used when the AMS requests bandwidth for multiple connections with shorter message in comparison of the previous one. The purpose of this type is not to indicate exact bandwidth requested by each FID but to meet minimum QoS for each FID when the UL bandwidth is lack. If UL bandwidth is enough, then the ABS can allocate bandwidth as requested. But if not, the ABS just tries to allocate bandwidth to meet minimum QoS of each FID based on the QoS parameter set negotiated during DSx procedure even though the ABS knows exact BW requested by each FID.

Table xx – BR type 4: BR for multiple connections with BR FID MAP

| Field | Length (bit) | Description |
| --- | --- | --- |
| BR type | 2 | 11: BR for multiple connections with BR FID MAP |
| Request type | 1 | 0: aggregate, 1: incremental |
| STID indicator | 1 | 0: Not included, 1: included |
| BR FID MAP | 16 | Each bit is set to 1 if the corresponding FID wants BR |
| BR size | 11 | Amount of bandwidth requested |
| If STID indicator = = 0{ |  |  |
| Reserved | 1 | Shall be set to zero |
| } |  |  |
| If STID indicator = = 1{ |  |  |
| STID | 12 | STID |
| Reserved | 5 | Shall be set to zero |
| } |  |  |

(a) BR for multiple connections without STID using BR FID MAP

(b) BR for multiple connections with STID using BR FID MAP

Figure xx. BR type 4: BR for multiple connections using BR FID MAP

]{Samsung}

15.2.y.1.5.3 Piggyback bandwidth request extended header format

{Editor’s note: limit the scope to functionality only. Defer specific header format definition.}

Piggybacked bandwidth request shall contain the FID of the requesting connection and the aggregate bandwidth to request. Multiple requests can be included in one piggybacked bandwidth request.

[{NSN/Nokia}

Piggyback bandwidth request contains one or more following compound fields:

- Flow ID

- Bandwidth request size

- Last: indicating if there is more Flow ID + Bandwidth request size + Last fields following

] {NSN/Nokia}

[{Samsung}

Piggyback bandwidth request (PBR) has several types and each type has different properties.

Table xx – Piggyback Bandwidth Request type

| PBR type | Name | PBR message size (byte) |
| --- | --- | --- |
| 1 | PBR for the AMS | 2 |
| 2 | PBR for the same connection with GMH | 2 |
| 3 | PBR for the different connection with GMH | 3 |
| 4 | PBR for multiple connections | 1+ (Number of PBR)*2 |
| 5 | PBR for multiple connections with BR FID MAP | 4 |

PBR type 1 is the PBR for the AMS used when the AMS requests bandwidth for all the connections serviced in the AMS.

Table xx – PBR type 1: PBR for the AMS

| Field | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0000: PBR for the AMS |
| BR size | 11 | Amount of bandwidth requested |

Figure xx. PBR type 1: PBR for the AMS

PBR type 2 is the PBR for the AMS used when the AMS requests bandwidth for the same connection with Generic MAC Header.

Table xx – PBR type 2: PBR for the same connection with GMH

| Field | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0001: PBR for the same connection with GMH |
| BR size | 11 | Amount of bandwidth requested |

PBR type 3 is the PBR for the AMS used when the AMS requests bandwidth for the different connection with Generic MAC Header.

Table xx – PBR type 3: PBR for the different connection with GMH

| Field | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0010: PBR for the different connection with GMH |
| FID | 4 | Flow identifier |
| BR size | 11 | Amount of bandwidth requested |
| Reserved | 4 | Shall be set to zero |

Figure xx. PBR type 3: PBR for the different connection with GMH

PBR type 4 is the PBR used when the AMS requests bandwidth for multiple connections.

The length of reserved field depends on the remaining bit to make a byte in the last byte in the format from 0 to 7 bits.

Table xx – PBR type 4: PBR for multiple connections

| Field | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0011: PBR for multiple connections |
| Number of PBR | 3 | The number of connections to request bandwidth |
| For(i=0; i<Number of PBR; i++){ |  |  |
| Request type | 1 | 0: aggregate, 1: incremental |
| FID | 4 | Flow identifier |
| BR size | 11 | Amount of bandwidth requested |
| } |  |  |

Figure xx. PBR type 4: PBR for multiple connections – An example when the number of BR is 2.

PBR type 5 is another PBR type for multiple connections used when the AMS requests bandwidth for multiple connections with shorter message in comparison of the previous one. The purpose of this type is not to indicate exact BW requested by each FID but to meet minimum QoS for each FID when the UL BW is lack. If UL BW is enough, then the ABS can allocate BW as requested. But if not, the ABS just tries to allocate BW to meet minimum QoS of each FID based on the QoS parameter set negotiated during DSx procedure even though the ABS knows exact BW requested by each FID.

Table xx – PBR type 5: BR for multiple connections with BR FID MAP

| Field | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Set to 0 if this is the last extended header |
| Type | 4 | 0100: PBR for multiple connections with BR FID MAP |
| BR FID MAP | 16 | Each bit is set to 1 if the corresponding FID wants BR |
| BR size | 11 | Amount of bandwidth requested |

Figure xx. PBR type 5: BR for multiple connections with BR FID MAP

]{Samsung}

15.2.y.2 Grant

In the bandwidth request procedure, the grant for BW-REQ message is allocated by CDMA Allocation A-MAP IE, which is shown in Table yy. [AMS decodes the IE and checks the MCRC field by its specific RA-ID. The RA-ID is calculated by a hash function with AMS’ bandwidth request ranging attributes (i.e., frame number index, subframe number index, BR opportunity, and BR preamble code index).]

The grant for UL transmission is allocated by UL Basic Assignment A-MAP IE (15.3.6.4.2.3).

Table yy –CDMA Allocation A-MAP IE Format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { | - | - |
| Resource Assignment Information | TBD | Size of Resource Allocation sub fields depends on system bandwidth or frequency partition size and number of concatenated subframes allocated by this IE. |
| MCS | 5 | Modulation and coding scheme used for the burst |
| [ MCRC | 16 | CRC masked by RA-ID. RA-ID: The identification for the anonymous CDMA Allocation A-MAP IE, and is derived by RA-ID = func ( frame number index, subframe number index, BR opportunity, BR preamble code). (eq. xxx) ] |
| [ Frame number index | 4 | Indentifies the frame in which the CDMA code was transmitted |
| Subframe number Index | 3 | Identifies the subframe in which the CDMA code was transmitted |
| BAB Index | 4(FFS) | Identifies the index in the BAB to which the slot in the subframe is mapped |
| Sequence Index | 2 | Identifies the sequence index from within the BW_ACK message ] |
| } | - | - |

]

For the bandwidth requested by the AMS, the uplink grant can shall be identified by Station ID of the AMS.

[In addition, to facilitate scheduling of the ABS, the ABS may indicate the purpose of the grant the ABS provides. In such a case, the ABS may indicate which grant is for the management connection and which grant is for the transport connection using Grant Purpose Indication Flag. If the ABS indicates the purpose of a grant, the AMS shall use granted bandwidth accordingly. For each granted allocation, the AMS may perform piggyback request or bandwidth stealing based on the uplink grant scheduling type usage rules.]

15.2.z Quality of Service (QoS)

15.2.z.1 Service classes

[

Table 124a—QoS parameters

| Name | Size (bits) | Value |
| --- | --- | --- |
| Uplink/Downlink indicator | 1 | 0 or 1 0=uplink; 1=downlink |
| Maximum sustained traffic rate per flow | 6 | Extensible look-up Table 124b (value 0b111111 indicates TLV to follow) |
| [Maximum sustained traffic rate per AMS | 6 | Extensible look-up Table 124b (value 0b111111 indicates TLV to follow) ] |
| Traffic indication preference | 1 | 0 or 1: 0=No traffic indication; 1=Traffic indication |
| Maximum traffic burst | 6 | Extensible look-up Table 124b (value 0b111111 indicates TLV to follow) |
| Minimum reserved traffic rate | 6 | Extensible look-up Table 124b (value 0b111111 indicates TLV to follow) |
| Maximum latency | 6 | Extensible look-up Table 124c (value 0b111111 indicates TLV to follow) |
| Fixed-length versus variable length SDU indicator | 1 | 0 or 1: 0=variable length; 1=fixed length |
| Paging preference | 1 | 0 or 1: 0 = No paging generation 1 = Paging generation |
| Reserved | 4 | Shall be set to 0b0000 |

Maximum sustained traffic rate per flow

A parameter that defines the peak information rate of the service. The rate is expressed in bits per second and pertains to the service data units (SDUs) at the input to the system. Explicitly, his parameter does not include transport, protocol, or network overhead such as MAC headers or CRCs, or non-payload session maintenance overhead like SIP, MGCP, H.323 administration, etc. This parameter does not limit the instantaneous rate of the service since this is governed by the physical attributes of the ingress port. However, at the destination network interface in the uplink direction, the service shall be policed to conform to this parameter, on the average, over time. The time that the traffic rate is averaged over shall be defined during service negotiation. On the network in the downlink direction, it may be assumed that the service was already policed at the ingress to the network. If this parameter is set to zero, then there is no explicitly mandated maximum rate. The maximum sustained traffic rate field specifies only a bound, not a guarantee that the rate is available. The algorithm for policing this parameter is left to vendor differentiation and is outside the scope of the standard.

Maximum sustained traffic rate per AMS

A parameter that defines the peak information rate of the aggregated services for the MS. The rate is expressed in bits per second and pertains to the service data units (SDUs) at the input to the system. Explicitly, this parameter does not include transport, protocol, or network overhead such as MAC headers or CRCs, or nonpayload session maintenance overhead like SIP, MGCP, H.323 administration, etc. This parameter does not limit the instantaneous rate of the service since this is governed by the physical attributes of the ingress port. However, at the destination network interface in the UL direction, the service shall be policed to conform to this parameter, on the average, over time. The time that the traffic rate is averaged over shall be defined during service negotiation. On the network in the DL direction, it may be assumed that the service was already policed at the ingress to the network. If this parameter is set to zero, then there is no explicitly mandated maximum rate. The maximum sustained traffic rate field specifies only a bound, not a guarantee that the rate is available. The algorithm for policing this parameter is left to vendor differentiation and is outside the scope of the standard.

]

15.2.z.2 Service Flow Management

[In addition to the legacy scheduling services described in 6.3.5,] {CC#4: More discussions among members is needed.}IEEE 802.16m supports adaptation of service flow (SF) QoS parameters.[ One or more QoS parameter set(s) may be defined during the initial service negotiation, e.g., a mandatory primary SF QoS parameter set, and an optional secondary SF QoS parameter set, etc.  Each SF QoS parameter set defines a set of QoS parameters. If multiple SF QoS parameter sets are defined, each of them corresponds to a specific traffic characteristic for the user data mapped to the same service flow. When QoS requirement/traffic characteristics for UL traffic changes, the ABS may autonomously perform adaptation by either changing the SF QoS parameters or switching among multiple SF QoS parameter sets. The AMS may also request the ABS to perform adaptation using explicit signaling. The ABS then allocates resource according to the adapted SF QoS parameters.]{CC#4: NO fundamental difference from current ertPS behavior and Network does not track airlink behavior-only provision for it.}

[

The value of FID field specifies the FID assigned by the ABS to a service flow of an AMS with a non-null AdmittedQosParamSet or ActiveQosParamSet. The 4-bit value of this field is used in BRs and in MAC PDU headersGMH. This field shall be present in a ABS-initiated DSA-REQ or DSC-REQ message related to establishing an admitted or active service flow. This field shall also be present in DSA-RSP and DSC-RSP messages related to the successful establishment of an admitted or active service flow.

If a service flow has been successfully admitted or activated (i.e., has an assigned FID) the SFID shall NOT be used for subsequent DSx message signaling as FID is the primary handle for a service flow. If a service flow is no longer admitted or active (via DSC-REQ), its FID may be reassigned by the BS.

]

15.2.z.3 Scheduling services

Scheduling services represent the data handling mechanisms supported by the MAC scheduler for data transport on a connection. Each service flow is associated with a single scheduling service as in WirelessMAN-OFDMA reference system. A scheduling service is determined by a set of SF QoS parameters that quantify aspects of its behavior. These parameters are established or modified using service flow management procedures.

15.2.z.3.1 Adaptive granting and polling service

The ABS may grant or poll AMS periodically and may negotiate [one or multiple][for only primary] SF QoS parameter sets, [or both primary and secondary QoS parameter sets] with the AMS. Initially, ABS uses QoS parameters defined in the primary SF QoS parameter set including Primary Grant and Polling Interval (GPI) and Primary Grant Size. During the service, the traffic characteristics and QoS requirement may change, [for example silence-suppression enabled VoIP alternates between talk spurt and silence period,] which triggers adaptation of SF QoS parameters as described below. [Adaptation includes switching between primary and secondary SF QoS parameter sets or changing of GPI/Grant size to values other than what are defined in the primary or secondary SF QoS parameter set. ]

Depending on the adaptation method specified during the service flow negotiation, the grant size or GPI can be changed by ABS automatically upon detecting certain traffic condition, or can be triggered by explicit signaling from AMS, such as Piggybacked Bandwidth Request, Bandwidth request signaling, QACH or Fastfeedback Channel.

There are three adaptation methods:

- Implicit: upon detecting of certain traffic condition with respect to a pre-negotiation SF QoS parameter set, ABS automatically changes GPI and/or Grant size; or switches between GPI_primary/ Grant_Size_primary and GPI_secondary/Grant_Size_secondary if secondary SF QoS parameter set is defined.

- Explicit, sustained: GPI and grant size change is triggered by explicit signaling from AMS such as Piggybacked Bandwidth Request, bandwidth request signaling, QACH, or fast feedback channel. Such change is sustained until next change request. If GPI_secondary/Grant_Size_secondary is defined, GPI and grant size switches between GPI_primary/ Grant_Size_primary and GPI_secondary/Grant_Size_secondary as requested by the explicit signaling; otherwise, GPI and grant size changes as indicated by QoS requirement carried in the explicit signaling as mechanisms mentioned above.

- Explicit, one time only: GPI and grant size change is triggered by explicit signaling from AMS such as in Piggybacked Bandwidth Request, bandwidth request signaling, QACH, or fast feedback channel. Such change is one-time only. If GPI_secondary/Grant_Size_secondary is defined, GPI and grant size one-time switches from GPI_primary/ Grant_Size_primary to GPI_secondary/Grant_Size_secondary; otherwise, GPI and grant size changes as indicated by QoS requirement carried in the explicit signaling as mechanisms mentioned above.

Table xxx describes the SF QoS parameters for Adaptive grant and polling scheduling service (aGPS).

Table xxx—SF QoS parameters for aGPS scheduling service

| Information Element | Notes |
| --- | --- |
| Maximum Latency (unsigned int) |  |
| Tolerated Jitter (unsigned int) |  |
| Minimum Reserved Traffic Rate (unsigned int) |  |
| Maximum Sustained Traffic Rate (unsigned int) |  |
| Traffic Priority (unsigned int) |  |
| Request/Transmission Policy (unsigned int) |  |
| If (uplink service flow) { (Boolean ) |  |
| Scheduling Type (unsigned int) | aGPS |
| GPI_primary (unsigned int) | Primary GPI, used initially |
| GrantSize_primary (unsigned int) | Primary grant size. If the primary grant size equals to x bytes (the newly defined bandwidth request header size), this indicates a primarily polling based service; otherwise, it is primarily granting based service. |
| GPI_secondary (unsigned int) | Secondary GPI (optional) |
| Grant_Size_secondary (unsigned int) | Secondary grant size (optional). If the secondary grant size is defined and equals to x bytes (the newly defined bandwidth request header size), this indicates a secondarily polling based; otherwise, it is a secondarily granting based service. |
| Adaptation Method (unsigned int) | - Implicit adaptation [ABS triggered adaptation] - Explicit adaptation, sustained [AMS triggered adaptation - sustained] - Explicit adaptation, one time only[AMS triggered adaptation - one time only] |
| } |  |

[

During AMS handover from Mzone/ABS to LZone/YBS, a ABS can map an aGPS service flow to a service flow of legacy scheduling type.

If the “Adaptation Method” QoS parameter of aGPS indicates adaptation, the aGPS service flow can be mapped to an ertPS service flow if it is primarily granting based; or it can be mapped to a rtPS service flow if it is primarily polling based.

Table yyy-2 — mapping from aGPS (without adaptation) to ertPS/rtPS

| 16m aGPS service flow | mapped 16e service flow |  |
| --- | --- | --- |
| QoS Parameters | Scheduling Type | QoS Parameters |
| Adaptation Method = BS triggered adaptation or MS triggered adaptation - sustained or MS triggered adaptation - one time only GrantSize_primary != x bytes (the newly defined bandwidth request header size) | ertPS | - UGI = Primary GPI - and map equally all the other common QoS parameters |
| Adaptation Method = BS triggered adaptation or MS triggered adaptation - sustained or MS triggered adaptation - one time only GrantSize_primary = x bytes (the newly defined bandwidth request header size) | rtPS | - UPI = Primary GPI - and map equally all the other common QoS parameters |

{CC#4 To revise the naming to be consistent with previous agreed text#1. Members please provide further comments on this table.}

]

15.2.z.4 Emergency Service Flow

Service flow parameters for emergency service may be pre-defined in the system.

15.2.z.4.1.1 Emergency Service Notification during initial ranging

The AMS may request for Emergency Service flow setup during initial ranging process [by using a predefined ranging preamble code] [or by including an Emergency Service Indicator in the AAI_RNG-REQ message.]

{CC#4: option-1: Samsung/LGE option-2: Intel. MTK: It may be too early to decide since ranging code is not determined. Lei: ranging code may not be sufficient for all purposes; more text is needed to clarify what is use for which case. Agreed: unbraket both and ask for member comments to down-select.}

Default service flow parameters are defined for emergency service flow. [When FID is predefined for Emergency Service request, an AMS sends the GMH including the predefined FID as an emergency service request indicator. Upon receiving the emergency service flow indicator, when FID is not predefined for Emergency Service, the ABS may directly allocates FIDs to the AMS using AAI_RNG-RSP without going through the complete service flow setup procedure. The ABS then grants resources according to the default service flow parameters defined for emergency service.]

{CC#4: Need more discussion for bracket’ed text: how to do it? A new call flow or simplifying existing DSA.}

[If a service provider wants to support National Security/emergency Preparedness (NS/EP) priority services, the

ABS uses its own algorithm as defined by its local country regulation body. For example, in the US the algorithm to support NS/EP is defined by the FCC in Hard Public Use Reservation by Departure Allocation (PURDA).]

{CC#4 to adopt this text from SDD}

15.2.z.4.1.2 Emergency Service Notification during connected state

[When an AMS requests for Emergency Service in the connected state and default service flow parameters are defined for the emergency service, the AMS shall send a bandwidth request with the indication of emergency service. When the FID is predefined for Emergency Service request, an AMS transmits the FID in the bandwidth request for emergency service. Upon receiving such request, when the FID is not predefined for Emergency Service, the ABS shall allocate new FID and SFID to the AMS using AAI_DSA-RSP, and establish the emergency service flow without going through the complete service flow setup procedure. When an AMS requests for Emergency Service in the connected state and no default service flow parameters are defined for the emergency service, the AMS shall establish the emergency service flow using service flow setup procedure and include the emergency service notification in the AAI_DSA-REQ] [or AAI_DSC-REQ].

============================== End of Proposed Text ===============