| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Sounding Sequence Section in IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | KeunChul Hwang, Jing Li, Seung-Won Kang, In-Seok Hwang, Soon-Young Yoon Samsung Electronics | E-mail: keunchul.hwang@samsung.com |
| Re: | Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content |  |
| Abstract | The contribution proposes the text of UL sounding sequence to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Sounding Sequence Section in IEEE 802.16m AWD

KeunChul Hwang, Jing Li, Seungwon Kang, In-seok Hwang, Soon-Young Yoon

Samsung Electronics

1. Introduction

This contribution proposes a new sounding sequence, and includes the proposed text on sounding sequence of the P802.16m AWD [1]. The performance evaluation results of proposed sounding sequence are shown in the contribution of S802.16m-09/00849 [2].

2. References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD),” March 2009.

[2] IEEE S802.16m-09/0849, “Evaluation of Proposed Sounding Sequence for IEEE 802.16m Amendment working Document,” May 2009.

3. Proposed Text

[Insert the following proposed text in blue at line 60, page 118 of 802.16m-09_0010r1a (section 15.3.9.2.3.1 Sounding sequence of 802.16m-09_0010r1)a. ]

----------------------------------------------------------- Text Start ---------------------------------------------------------------

15.3.9.2.3.1 Sounding sequence

For the cyclic shift separability, the sounding sequence is defined as

(xx)

where

k is the subcarrier index ( 0  k  Nused1),

Nused is the number of usable subcarriers in the sounding symbol,

P is the max cyclic shift index (from the sounding instructions),

n is the assigned cyclic shift index (also from the sounding instructions), which ranges from 0 to P–1,

B is the group of allocated subcarriers/bands according to the sounding instructions,

m is the cell-specific parameter representing index of short sounding sequence (from the sounding

instructions), and

m(x) is the phase of short sounding sequence, which is defined in Tables xx-xx for various P.

The long sounding sequence f(k) is defined as

(xx)

where

 is the integer value, which is dependent on P as defined in Table xx,

NG is given by the smallest prime number larger than Nused, which is given in Table xx, and

d is the cell common or specific parameter, with which the number of sounding sequence increases ( 0  d 

NG1),

Table xx – Phase of sounding short sequence for P = 4 and D = 4

| m x | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 0.0000 | 0.2500 | 1.0000 | 0.2500 |
| 1 | 0.0000 | 1.2500 | 0.0000 | 0.2500 |
| 2 | 0.0000 | 1.2500 | 1.0000 | 1.2500 |
| 3 | 0.0000 | 0.2500 | 0.0000 | 1.2500 |

Table xx – Phase of sounding short sequence for P = 6 and D = 6

| m x | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0000 | 0.5000 | 1.3333 | 0.5000 | 0.0000 | 1.8333 |
| 1 | 0.0000 | 0.8333 | 0.0000 | 1.5000 | 1.3333 | 1.5000 |
| 2 | 0.0000 | 1.1667 | 0.6667 | 0.5000 | 0.6667 | 1.1667 |
| 3 | 0.0000 | 1.5000 | 1.3333 | 1.5000 | 0.0000 | 0.8333 |
| 4 | 0.0000 | 1.8333 | 0.0000 | 0.5000 | 1.3333 | 0.5000 |
| 5 | 0.0000 | 0.1667 | 0.6667 | 1.5000 | 0.6667 | 0.1667 |

Table xx – Phase of sounding short sequence for P = 8 and D = 8

| m x | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0000 | 0.1250 | 0.5000 | 1.1250 | 0.0000 | 1.1250 | 0.5000 | 0.1250 |
| 1 | 0.0000 | 0.6250 | 1.0000 | 0.1250 | 1.0000 | 0.6250 | 0.0000 | 0.1250 |
| 2 | 0.0000 | 1.1250 | 1.5000 | 1.1250 | 0.0000 | 0.1250 | 1.5000 | 0.1250 |
| 3 | 0.0000 | 1.6250 | 0.0000 | 0.1250 | 1.0000 | 1.6250 | 1.0000 | 0.1250 |
| 4 | 0.0000 | 1.1250 | 0.5000 | 0.1250 | 0.0000 | 0.1250 | 0.5000 | 1.1250 |
| 5 | 0.0000 | 1.6250 | 1.0000 | 1.1250 | 1.0000 | 1.6250 | 0.0000 | 1.1250 |
| 6 | 0.0000 | 0.1250 | 1.5000 | 0.1250 | 0.0000 | 1.1250 | 1.5000 | 1.1250 |
| 7 | 0.0000 | 0.6250 | 0.0000 | 1.1250 | 1.0000 | 0.6250 | 1.0000 | 1.1250 |

Table xx – Phase of sounding short sequence for P = 9 and D = 9

| m x | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0000 | 0.2222 | 0.6667 | 1.3333 | 0.2222 | 1.3333 | 0.6667 | 0.2222 | 0.0000 |
| 1 | 0.0000 | 0.8889 | 1.3333 | 0.0000 | 1.5556 | 0.6667 | 0.0000 | 0.2222 | 0.0000 |
| 2 | 0.0000 | 1.5556 | 0.0000 | 0.6667 | 0.8889 | 0.0000 | 1.3333 | 0.2222 | 0.0000 |
| 3 | 0.0000 | 0.8889 | 0.0000 | 1.3333 | 0.8889 | 0.6667 | 0.6667 | 0.8889 | 1.3333 |
| 4 | 0.0000 | 1.5556 | 0.6667 | 0.0000 | 0.2222 | 0.0000 | 0.0000 | 0.8889 | 1.3333 |
| 5 | 0.0000 | 0.2222 | 1.3333 | 0.6667 | 1.5556 | 1.3333 | 1.3333 | 0.8889 | 1.3333 |
| 6 | 0.0000 | 1.5556 | 1.3333 | 1.3333 | 1.5556 | 0.0000 | 0.6667 | 1.5556 | 0.6667 |
| 7 | 0.0000 | 0.2222 | 2.0000 | 0.0000 | 0.8889 | 1.3333 | 0.0000 | 1.5556 | 0.6667 |
| 8 | 0.0000 | 0.8889 | 0.6667 | 0.6667 | 0.2222 | 0.6667 | 1.3333 | 1.5556 | 0.6667 |

Table xx – Phase of sounding short sequence for P = 16 and D = 16

| m x | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0000 | 1.3125 | 0.5000 | 0.0625 | 1.5000 | 1.3125 | 1.0000 | 1.0625 | 1.0000 | 1.3125 | 1.5000 | 0.0625 | 0.5000 | 1.3125 | 0.0000 | 1.0625 |
| 1 | 0.0000 | 1.5625 | 0.7500 | 0.5625 | 0.0000 | 0.0625 | 1.7500 | 0.0625 | 0.0000 | 0.5625 | 0.7500 | 1.5625 | 0.0000 | 1.0625 | 1.7500 | 1.0625 |
| 2 | 0.0000 | 1.8125 | 1.0000 | 1.0625 | 0.5000 | 0.8125 | 0.5000 | 1.0625 | 1.0000 | 1.8125 | 0.0000 | 1.0625 | 1.5000 | 0.8125 | 1.5000 | 1.0625 |
| 3 | 0.0000 | 0.0625 | 1.2500 | 1.5625 | 1.0000 | 1.5625 | 1.2500 | 0.0625 | 0.0000 | 1.0625 | 1.2500 | 0.5625 | 1.0000 | 0.5625 | 1.2500 | 1.0625 |
| 4 | 0.0000 | 0.3125 | 1.5000 | 0.0625 | 1.5000 | 0.3125 | 0.0000 | 1.0625 | 1.0000 | 0.3125 | 0.5000 | 0.0625 | 0.5000 | 0.3125 | 1.0000 | 1.0625 |
| 5 | 0.0000 | 0.5625 | 1.7500 | 0.5625 | 0.0000 | 1.0625 | 0.7500 | 0.0625 | 0.0000 | 1.5625 | 1.7500 | 1.5625 | 0.0000 | 0.0625 | 0.7500 | 1.0625 |
| 6 | 0.0000 | 0.8125 | 0.0000 | 1.0625 | 0.5000 | 1.8125 | 1.5000 | 1.0625 | 1.0000 | 0.8125 | 1.0000 | 1.0625 | 1.5000 | 1.8125 | 0.5000 | 1.0625 |
| 7 | 0.0000 | 1.0625 | 0.2500 | 1.5625 | 1.0000 | 0.5625 | 0.2500 | 0.0625 | 0.0000 | 0.0625 | 0.2500 | 0.5625 | 1.0000 | 1.5625 | 0.2500 | 1.0625 |
| 8 | 0.0000 | 0.3125 | 0.5000 | 1.0625 | 1.5000 | 0.3125 | 1.0000 | 0.0625 | 1.0000 | 0.3125 | 1.5000 | 1.0625 | 0.5000 | 0.3125 | 0.0000 | 0.0625 |
| 9 | 0.0000 | 0.5625 | 0.7500 | 1.5625 | 0.0000 | 1.0625 | 1.7500 | 1.0625 | 0.0000 | 1.5625 | 0.7500 | 0.5625 | 0.0000 | 0.0625 | 1.7500 | 0.0625 |
| 10 | 0.0000 | 0.8125 | 1.0000 | 0.0625 | 0.5000 | 1.8125 | 0.5000 | 0.0625 | 1.0000 | 0.8125 | 0.0000 | 0.0625 | 1.5000 | 1.8125 | 1.5000 | 0.0625 |
| 11 | 0.0000 | 1.0625 | 1.2500 | 0.5625 | 1.0000 | 0.5625 | 1.2500 | 1.0625 | 0.0000 | 0.0625 | 1.2500 | 1.5625 | 1.0000 | 1.5625 | 1.2500 | 0.0625 |
| 12 | 0.0000 | 1.3125 | 1.5000 | 1.0625 | 1.5000 | 1.3125 | 0.0000 | 0.0625 | 1.0000 | 1.3125 | 0.5000 | 1.0625 | 0.5000 | 1.3125 | 1.0000 | 0.0625 |
| 13 | 0.0000 | 1.5625 | 1.7500 | 1.5625 | 0.0000 | 0.0625 | 0.7500 | 1.0625 | 0.0000 | 0.5625 | 1.7500 | 0.5625 | 0.0000 | 1.0625 | 0.7500 | 0.0625 |
| 14 | 0.0000 | 1.8125 | 0.0000 | 0.0625 | 0.5000 | 0.8125 | 1.5000 | 0.0625 | 1.0000 | 1.8125 | 1.0000 | 0.0625 | 1.5000 | 0.8125 | 0.5000 | 0.0625 |
| 15 | 0.0000 | 0.0625 | 0.2500 | 0.5625 | 1.0000 | 1.5625 | 0.2500 | 1.0625 | 0.0000 | 1.0625 | 0.2500 | 1.5625 | 1.0000 | 0.5625 | 0.2500 | 0.0625 |

Table xx – Phase of sounding short sequence for P = 18 and D = 18

| m x | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0000 | 0.0556 | 0.2222 | 0.5000 | 0.8889 | 1.3889 | 0.0000 | 0.7222 | 1.5556 | 0.5000 | 1.5556 | 0.7222 | 0.0000 | 1.3889 | 0.8889 | 0.5000 | 0.2222 | 0.0556 |
| 1 | 0.0000 | 0.3889 | 0.5556 | 0.8333 | 1.5556 | 0.0556 | 0.6667 | 1.7222 | 0.5556 | 1.5000 | 0.8889 | 0.0556 | 1.3333 | 1.0556 | 0.5556 | 0.1667 | 0.2222 | 0.0556 |
| 2 | 0.0000 | 0.7222 | 0.8889 | 1.1667 | 0.2222 | 0.7222 | 1.3333 | 0.7222 | 1.5556 | 0.5000 | 0.2222 | 1.3889 | 0.6667 | 0.7222 | 0.2222 | 1.8333 | 0.2222 | 0.0556 |
| 3 | 0.0000 | 1.0556 | 1.2222 | 1.5000 | 0.8889 | 1.3889 | 0.0000 | 1.7222 | 0.5556 | 1.5000 | 1.5556 | 0.7222 | 0.0000 | 0.3889 | 1.8889 | 1.5000 | 0.2222 | 0.0556 |
| 4 | 0.0000 | 1.3889 | 1.5556 | 1.8333 | 1.5556 | 0.0556 | 0.6667 | 0.7222 | 1.5556 | 0.5000 | 0.8889 | 0.0556 | 1.3333 | 0.0556 | 1.5556 | 1.1667 | 0.2222 | 0.0556 |
| 5 | 0.0000 | 1.7222 | 1.8889 | 0.1667 | 0.2222 | 0.7222 | 1.3333 | 1.7222 | 0.5556 | 1.5000 | 0.2222 | 1.3889 | 0.6667 | 1.7222 | 1.2222 | 0.8333 | 0.2222 | 0.0556 |
| 6 | 0.0000 | 0.7222 | 1.5556 | 0.5000 | 1.5556 | 0.7222 | 0.0000 | 1.3889 | 0.8889 | 0.5000 | 0.2222 | 0.0556 | 0.0000 | 0.0556 | 0.2222 | 0.5000 | 0.8889 | 1.3889 |
| 7 | 0.0000 | 1.0556 | 1.8889 | 0.8333 | 0.2222 | 1.3889 | 0.6667 | 0.3889 | 1.8889 | 1.5000 | 1.5556 | 1.3889 | 1.3333 | 1.7222 | 1.8889 | 0.1667 | 0.8889 | 1.3889 |
| 8 | 0.0000 | 1.3889 | 0.2222 | 1.1667 | 0.8889 | 0.0556 | 1.3333 | 1.3889 | 0.8889 | 0.5000 | 0.8889 | 0.7222 | 0.6667 | 1.3889 | 1.5556 | 1.8333 | 0.8889 | 1.3889 |
| 9 | 0.0000 | 1.7222 | 0.5556 | 1.5000 | 1.5556 | 0.7222 | 0.0000 | 0.3889 | 1.8889 | 1.5000 | 0.2222 | 0.0556 | 0.0000 | 1.0556 | 1.2222 | 1.5000 | 0.8889 | 1.3889 |
| 10 | 0.0000 | 0.0556 | 0.8889 | 1.8333 | 0.2222 | 1.3889 | 0.6667 | 1.3889 | 0.8889 | 0.5000 | 1.5556 | 1.3889 | 1.3333 | 0.7222 | 0.8889 | 1.1667 | 0.8889 | 1.3889 |
| 11 | 0.0000 | 0.3889 | 1.2222 | 0.1667 | 0.8889 | 0.0556 | 1.3333 | 0.3889 | 1.8889 | 1.5000 | 0.8889 | 0.7222 | 0.6667 | 0.3889 | 0.5556 | 0.8333 | 0.8889 | 1.3889 |
| 12 | 0.0000 | 1.3889 | 0.8889 | 0.5000 | 0.2222 | 0.0556 | 0.0000 | 0.0556 | 0.2222 | 0.5000 | 0.8889 | 1.3889 | 0.0000 | 0.7222 | 1.5556 | 0.5000 | 1.5556 | 0.7222 |
| 13 | 0.0000 | 1.7222 | 1.2222 | 0.8333 | 0.8889 | 0.7222 | 0.6667 | 1.0556 | 1.2222 | 1.5000 | 0.2222 | 0.7222 | 1.3333 | 0.3889 | 1.2222 | 0.1667 | 1.5556 | 0.7222 |
| 14 | 0.0000 | 0.0556 | 1.5556 | 1.1667 | 1.5556 | 1.3889 | 1.3333 | 0.0556 | 0.2222 | 0.5000 | 1.5556 | 0.0556 | 0.6667 | 0.0556 | 0.8889 | 1.8333 | 1.5556 | 0.7222 |
| 15 | 0.0000 | 0.3889 | 1.8889 | 1.5000 | 0.2222 | 0.0556 | 0.0000 | 1.0556 | 1.2222 | 1.5000 | 0.8889 | 1.3889 | 0.0000 | 1.7222 | 0.5556 | 1.5000 | 1.5556 | 0.7222 |
| 16 | 0.0000 | 0.7222 | 0.2222 | 1.8333 | 0.8889 | 0.7222 | 0.6667 | 0.0556 | 0.2222 | 0.5000 | 0.2222 | 0.7222 | 1.3333 | 1.3889 | 0.2222 | 1.1667 | 1.5556 | 0.7222 |
| 17 | 0.0000 | 1.0556 | 0.5556 | 0.1667 | 1.5556 | 1.3889 | 1.3333 | 1.0556 | 1.2222 | 1.5000 | 1.5556 | 0.0556 | 0.6667 | 1.0556 | 1.8889 | 0.8333 | 1.5556 | 0.7222 |

Table xx – Phase of sounding short sequence for P = 32 and D = 32

| m x | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0000 | 0.0313 | 0.1250 | 0.2813 | 0.5000 | 0.7813 | 1.1250 | 1.5313 | 0.0000 | 0.5313 | 1.1250 | 1.7813 | 0.5000 | 1.2813 | 0.1250 | 1.0313 | 0.0000 | 1.0313 |
| 1 | 0.0000 | 0.1563 | 0.2500 | 0.5313 | 0.7500 | 1.1563 | 1.5000 | 0.0313 | 0.5000 | 1.1563 | 1.7500 | 0.5313 | 1.2500 | 0.1563 | 1.0000 | 0.0313 | 1.0000 | 0.1563 |
| 2 | 0.0000 | 0.2813 | 0.3750 | 0.7813 | 1.0000 | 1.5313 | 1.8750 | 0.5313 | 1.0000 | 1.7813 | 0.3750 | 1.2813 | 0.0000 | 1.0313 | 1.8750 | 1.0313 | 0.0000 | 1.2813 |
| 3 | 0.0000 | 0.4063 | 0.5000 | 1.0313 | 1.2500 | 1.9063 | 0.2500 | 1.0313 | 1.5000 | 0.4063 | 1.0000 | 0.0313 | 0.7500 | 1.9063 | 0.7500 | 0.0313 | 1.0000 | 0.4063 |
| 4 | 0.0000 | 0.5313 | 0.6250 | 1.2813 | 1.5000 | 0.2813 | 0.6250 | 1.5313 | 0.0000 | 1.0313 | 1.6250 | 0.7813 | 1.5000 | 0.7813 | 1.6250 | 1.0313 | 0.0000 | 1.5313 |
| 5 | 0.0000 | 0.6563 | 0.7500 | 1.5313 | 1.7500 | 0.6563 | 1.0000 | 0.0313 | 0.5000 | 1.6563 | 0.2500 | 1.5313 | 0.2500 | 1.6563 | 0.5000 | 0.0313 | 1.0000 | 0.6563 |
| 6 | 0.0000 | 0.7813 | 0.8750 | 1.7813 | 0.0000 | 1.0313 | 1.3750 | 0.5313 | 1.0000 | 0.2813 | 0.8750 | 0.2813 | 1.0000 | 0.5313 | 1.3750 | 1.0313 | 0.0000 | 1.7813 |
| 7 | 0.0000 | 0.9063 | 1.0000 | 0.0313 | 0.2500 | 1.4063 | 1.7500 | 1.0313 | 1.5000 | 0.9063 | 1.5000 | 1.0313 | 1.7500 | 1.4063 | 0.2500 | 0.0313 | 1.0000 | 0.9063 |
| 8 | 0.0000 | 1.0313 | 1.1250 | 0.2813 | 0.5000 | 1.7813 | 0.1250 | 1.5313 | 0.0000 | 1.5313 | 0.1250 | 1.7813 | 0.5000 | 0.2813 | 1.1250 | 1.0313 | 0.0000 | 0.0313 |
| 9 | 0.0000 | 1.1563 | 1.2500 | 0.5313 | 0.7500 | 0.1563 | 0.5000 | 0.0313 | 0.5000 | 0.1563 | 0.7500 | 0.5313 | 1.2500 | 1.1563 | 0.0000 | 0.0313 | 1.0000 | 1.1563 |
| 10 | 0.0000 | 1.2813 | 1.3750 | 0.7813 | 1.0000 | 0.5313 | 0.8750 | 0.5313 | 1.0000 | 0.7813 | 1.3750 | 1.2813 | 0.0000 | 0.0313 | 0.8750 | 1.0313 | 0.0000 | 0.2813 |
| 11 | 0.0000 | 1.4063 | 1.5000 | 1.0313 | 1.2500 | 0.9063 | 1.2500 | 1.0313 | 1.5000 | 1.4063 | 0.0000 | 0.0313 | 0.7500 | 0.9063 | 1.7500 | 0.0313 | 1.0000 | 1.4063 |
| 12 | 0.0000 | 1.5313 | 1.6250 | 1.2813 | 1.5000 | 1.2813 | 1.6250 | 1.5313 | 0.0000 | 0.0313 | 0.6250 | 0.7813 | 1.5000 | 1.7813 | 0.6250 | 1.0313 | 0.0000 | 0.5313 |
| 13 | 0.0000 | 1.6563 | 1.7500 | 1.5313 | 1.7500 | 1.6563 | 0.0000 | 0.0313 | 0.5000 | 0.6563 | 1.2500 | 1.5313 | 0.2500 | 0.6563 | 1.5000 | 0.0313 | 1.0000 | 1.6563 |
| 14 | 0.0000 | 1.7813 | 1.8750 | 1.7813 | 0.0000 | 0.0313 | 0.3750 | 0.5313 | 1.0000 | 1.2813 | 1.8750 | 0.2813 | 1.0000 | 1.5313 | 0.3750 | 1.0313 | 0.0000 | 0.7813 |
| 15 | 0.0000 | 1.9063 | 0.0000 | 0.0313 | 0.2500 | 0.4063 | 0.7500 | 1.0313 | 1.5000 | 1.9063 | 0.5000 | 1.0313 | 1.7500 | 0.4063 | 1.2500 | 0.0313 | 1.0000 | 1.9063 |
| 16 | 0.0000 | 1.0313 | 0.1250 | 1.2813 | 0.5000 | 1.7813 | 1.1250 | 0.5313 | 0.0000 | 1.5313 | 1.1250 | 0.7813 | 0.5000 | 0.2813 | 0.1250 | 0.0313 | 0.0000 | 0.0313 |
| 17 | 0.0000 | 1.1563 | 0.2500 | 1.5313 | 0.7500 | 0.1563 | 1.5000 | 1.0313 | 0.5000 | 0.1563 | 1.7500 | 1.5313 | 1.2500 | 1.1563 | 1.0000 | 1.0313 | 1.0000 | 1.1563 |
| 18 | 0.0000 | 1.2813 | 0.3750 | 1.7813 | 1.0000 | 0.5313 | 1.8750 | 1.5313 | 1.0000 | 0.7813 | 0.3750 | 0.2813 | 0.0000 | 0.0313 | 1.8750 | 0.0313 | 0.0000 | 0.2813 |
| 19 | 0.0000 | 1.4063 | 0.5000 | 0.0313 | 1.2500 | 0.9063 | 0.2500 | 0.0313 | 1.5000 | 1.4063 | 1.0000 | 1.0313 | 0.7500 | 0.9063 | 0.7500 | 1.0313 | 1.0000 | 1.4063 |
| 20 | 0.0000 | 1.5313 | 0.6250 | 0.2813 | 1.5000 | 1.2813 | 0.6250 | 0.5313 | 0.0000 | 0.0313 | 1.6250 | 1.7813 | 1.5000 | 1.7813 | 1.6250 | 0.0313 | 0.0000 | 0.5313 |
| 21 | 0.0000 | 1.6563 | 0.7500 | 0.5313 | 1.7500 | 1.6563 | 1.0000 | 1.0313 | 0.5000 | 0.6563 | 0.2500 | 0.5313 | 0.2500 | 0.6563 | 0.5000 | 1.0313 | 1.0000 | 1.6563 |
| 22 | 0.0000 | 1.7813 | 0.8750 | 0.7813 | 0.0000 | 0.0313 | 1.3750 | 1.5313 | 1.0000 | 1.2813 | 0.8750 | 1.2813 | 1.0000 | 1.5313 | 1.3750 | 0.0313 | 0.0000 | 0.7813 |
| 23 | 0.0000 | 1.9063 | 1.0000 | 1.0313 | 0.2500 | 0.4063 | 1.7500 | 0.0313 | 1.5000 | 1.9063 | 1.5000 | 0.0313 | 1.7500 | 0.4063 | 0.2500 | 1.0313 | 1.0000 | 1.9063 |
| 24 | 0.0000 | 0.0313 | 1.1250 | 1.2813 | 0.5000 | 0.7813 | 0.1250 | 0.5313 | 0.0000 | 0.5313 | 0.1250 | 0.7813 | 0.5000 | 1.2813 | 1.1250 | 0.0313 | 0.0000 | 1.0313 |
| 25 | 0.0000 | 0.1563 | 1.2500 | 1.5313 | 0.7500 | 1.1563 | 0.5000 | 1.0313 | 0.5000 | 1.1563 | 0.7500 | 1.5313 | 1.2500 | 0.1563 | 0.0000 | 1.0313 | 1.0000 | 0.1563 |
| 26 | 0.0000 | 0.2813 | 1.3750 | 1.7813 | 1.0000 | 1.5313 | 0.8750 | 1.5313 | 1.0000 | 1.7813 | 1.3750 | 0.2813 | 0.0000 | 1.0313 | 0.8750 | 0.0313 | 0.0000 | 1.2813 |
| 27 | 0.0000 | 0.4063 | 1.5000 | 0.0313 | 1.2500 | 1.9063 | 1.2500 | 0.0313 | 1.5000 | 0.4063 | 0.0000 | 1.0313 | 0.7500 | 1.9063 | 1.7500 | 1.0313 | 1.0000 | 0.4063 |
| 28 | 0.0000 | 0.5313 | 1.6250 | 0.2813 | 1.5000 | 0.2813 | 1.6250 | 0.5313 | 0.0000 | 1.0313 | 0.6250 | 1.7813 | 1.5000 | 0.7813 | 0.6250 | 0.0313 | 0.0000 | 1.5313 |
| 29 | 0.0000 | 0.6563 | 1.7500 | 0.5313 | 1.7500 | 0.6563 | 0.0000 | 1.0313 | 0.5000 | 1.6563 | 1.2500 | 0.5313 | 0.2500 | 1.6563 | 1.5000 | 1.0313 | 1.0000 | 0.6563 |
| 30 | 0.0000 | 0.7813 | 1.8750 | 0.7813 | 0.0000 | 1.0313 | 0.3750 | 1.5313 | 1.0000 | 0.2813 | 1.8750 | 1.2813 | 1.0000 | 0.5313 | 0.3750 | 0.0313 | 0.0000 | 1.7813 |
| 31 | 0.0000 | 0.9063 | 0.0000 | 1.0313 | 0.2500 | 1.4063 | 0.7500 | 0.0313 | 1.5000 | 0.9063 | 0.5000 | 0.0313 | 1.7500 | 1.4063 | 1.2500 | 1.0313 | 1.0000 | 0.9063 |
| m x | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |  |  |  |  |
| 0 | 0.1250 | 1.2813 | 0.5000 | 1.7813 | 1.1250 | 0.5313 | 0.0000 | 1.5313 | 1.1250 | 0.7813 | 0.5000 | 0.2813 | 0.1250 | 0.0313 |  |  |  |  |
| 1 | 1.2500 | 0.5313 | 1.7500 | 1.1563 | 0.5000 | 0.0313 | 1.5000 | 1.1563 | 0.7500 | 0.5313 | 0.2500 | 0.1563 | 0.0000 | 0.0313 |  |  |  |  |
| 2 | 0.3750 | 1.7813 | 1.0000 | 0.5313 | 1.8750 | 1.5313 | 1.0000 | 0.7813 | 0.3750 | 0.2813 | 0.0000 | 0.0313 | 1.8750 | 0.0313 |  |  |  |  |
| 3 | 1.5000 | 1.0313 | 0.2500 | 1.9063 | 1.2500 | 1.0313 | 0.5000 | 0.4063 | 0.0000 | 0.0313 | 1.7500 | 1.9063 | 1.7500 | 0.0313 |  |  |  |  |
| 4 | 0.6250 | 0.2813 | 1.5000 | 1.2813 | 0.6250 | 0.5313 | 0.0000 | 0.0313 | 1.6250 | 1.7813 | 1.5000 | 1.7813 | 1.6250 | 0.0313 |  |  |  |  |
| 5 | 1.7500 | 1.5313 | 0.7500 | 0.6563 | 0.0000 | 0.0313 | 1.5000 | 1.6563 | 1.2500 | 1.5313 | 1.2500 | 1.6563 | 1.5000 | 0.0313 |  |  |  |  |
| 6 | 0.8750 | 0.7813 | 0.0000 | 0.0313 | 1.3750 | 1.5313 | 1.0000 | 1.2813 | 0.8750 | 1.2813 | 1.0000 | 1.5313 | 1.3750 | 0.0313 |  |  |  |  |
| 7 | 0.0000 | 0.0313 | 1.2500 | 1.4063 | 0.7500 | 1.0313 | 0.5000 | 0.9063 | 0.5000 | 1.0313 | 0.7500 | 1.4063 | 1.2500 | 0.0313 |  |  |  |  |
| 8 | 1.1250 | 1.2813 | 0.5000 | 0.7813 | 0.1250 | 0.5313 | 0.0000 | 0.5313 | 0.1250 | 0.7813 | 0.5000 | 1.2813 | 1.1250 | 0.0313 |  |  |  |  |
| 9 | 0.2500 | 0.5313 | 1.7500 | 0.1563 | 1.5000 | 0.0313 | 1.5000 | 0.1563 | 1.7500 | 0.5313 | 0.2500 | 1.1563 | 1.0000 | 0.0313 |  |  |  |  |
| 10 | 1.3750 | 1.7813 | 1.0000 | 1.5313 | 0.8750 | 1.5313 | 1.0000 | 1.7813 | 1.3750 | 0.2813 | 0.0000 | 1.0313 | 0.8750 | 0.0313 |  |  |  |  |
| 11 | 0.5000 | 1.0313 | 0.2500 | 0.9063 | 0.2500 | 1.0313 | 0.5000 | 1.4063 | 1.0000 | 0.0313 | 1.7500 | 0.9063 | 0.7500 | 0.0313 |  |  |  |  |
| 12 | 1.6250 | 0.2813 | 1.5000 | 0.2813 | 1.6250 | 0.5313 | 0.0000 | 1.0313 | 0.6250 | 1.7813 | 1.5000 | 0.7813 | 0.6250 | 0.0313 |  |  |  |  |
| 13 | 0.7500 | 1.5313 | 0.7500 | 1.6563 | 1.0000 | 0.0313 | 1.5000 | 0.6563 | 0.2500 | 1.5313 | 1.2500 | 0.6563 | 0.5000 | 0.0313 |  |  |  |  |
| 14 | 1.8750 | 0.7813 | 0.0000 | 1.0313 | 0.3750 | 1.5313 | 1.0000 | 0.2813 | 1.8750 | 1.2813 | 1.0000 | 0.5313 | 0.3750 | 0.0313 |  |  |  |  |
| 15 | 1.0000 | 0.0313 | 1.2500 | 0.4063 | 1.7500 | 1.0313 | 0.5000 | 1.9063 | 1.5000 | 1.0313 | 0.7500 | 0.4063 | 0.2500 | 0.0313 |  |  |  |  |
| 16 | 0.1250 | 0.2813 | 0.5000 | 0.7813 | 1.1250 | 1.5313 | 0.0000 | 0.5313 | 1.1250 | 1.7813 | 0.5000 | 1.2813 | 0.1250 | 1.0313 |  |  |  |  |
| 17 | 1.2500 | 1.5313 | 1.7500 | 0.1563 | 0.5000 | 1.0313 | 1.5000 | 0.1563 | 0.7500 | 1.5313 | 0.2500 | 1.1563 | 0.0000 | 1.0313 |  |  |  |  |
| 18 | 0.3750 | 0.7813 | 1.0000 | 1.5313 | 1.8750 | 0.5313 | 1.0000 | 1.7813 | 0.3750 | 1.2813 | 0.0000 | 1.0313 | 1.8750 | 1.0313 |  |  |  |  |
| 19 | 1.5000 | 0.0313 | 0.2500 | 0.9063 | 1.2500 | 0.0313 | 0.5000 | 1.4063 | 0.0000 | 1.0313 | 1.7500 | 0.9063 | 1.7500 | 1.0313 |  |  |  |  |
| 20 | 0.6250 | 1.2813 | 1.5000 | 0.2813 | 0.6250 | 1.5313 | 0.0000 | 1.0313 | 1.6250 | 0.7813 | 1.5000 | 0.7813 | 1.6250 | 1.0313 |  |  |  |  |
| 21 | 1.7500 | 0.5313 | 0.7500 | 1.6563 | 0.0000 | 1.0313 | 1.5000 | 0.6563 | 1.2500 | 0.5313 | 1.2500 | 0.6563 | 1.5000 | 1.0313 |  |  |  |  |
| 22 | 0.8750 | 1.7813 | 0.0000 | 1.0313 | 1.3750 | 0.5313 | 1.0000 | 0.2813 | 0.8750 | 0.2813 | 1.0000 | 0.5313 | 1.3750 | 1.0313 |  |  |  |  |
| 23 | 0.0000 | 1.0313 | 1.2500 | 0.4063 | 0.7500 | 0.0313 | 0.5000 | 1.9063 | 0.5000 | 0.0313 | 0.7500 | 0.4063 | 1.2500 | 1.0313 |  |  |  |  |
| 24 | 1.1250 | 0.2813 | 0.5000 | 1.7813 | 0.1250 | 1.5313 | 0.0000 | 1.5313 | 0.1250 | 1.7813 | 0.5000 | 0.2813 | 1.1250 | 1.0313 |  |  |  |  |
| 25 | 0.2500 | 1.5313 | 1.7500 | 1.1563 | 1.5000 | 1.0313 | 1.5000 | 1.1563 | 1.7500 | 1.5313 | 0.2500 | 0.1563 | 1.0000 | 1.0313 |  |  |  |  |
| 26 | 1.3750 | 0.7813 | 1.0000 | 0.5313 | 0.8750 | 0.5313 | 1.0000 | 0.7813 | 1.3750 | 1.2813 | 0.0000 | 0.0313 | 0.8750 | 1.0313 |  |  |  |  |
| 27 | 0.5000 | 0.0313 | 0.2500 | 1.9063 | 0.2500 | 0.0313 | 0.5000 | 0.4063 | 1.0000 | 1.0313 | 1.7500 | 1.9063 | 0.7500 | 1.0313 |  |  |  |  |
| 28 | 1.6250 | 1.2813 | 1.5000 | 1.2813 | 1.6250 | 1.5313 | 0.0000 | 0.0313 | 0.6250 | 0.7813 | 1.5000 | 1.7813 | 0.6250 | 1.0313 |  |  |  |  |
| 29 | 0.7500 | 0.5313 | 0.7500 | 0.6563 | 1.0000 | 1.0313 | 1.5000 | 1.6563 | 0.2500 | 0.5313 | 1.2500 | 1.6563 | 0.5000 | 1.0313 |  |  |  |  |
| 30 | 1.8750 | 1.7813 | 0.0000 | 0.0313 | 0.3750 | 0.5313 | 1.0000 | 1.2813 | 1.8750 | 0.2813 | 1.0000 | 1.5313 | 0.3750 | 1.0313 |  |  |  |  |
| 31 | 1.0000 | 1.0313 | 1.2500 | 1.4063 | 1.7500 | 0.0313 | 0.5000 | 0.9063 | 1.5000 | 0.0313 | 0.7500 | 1.4063 | 0.2500 | 1.0313 |  |  |  |  |

Table xx – value for cyclic shift separability

| P |  |  |  |
| --- | --- | --- | --- |
|  | Nused = 433 | Nused = 865 | Nused = 1729 |
| 4 | 48 | 370 | 176 |
| 6 | 372 | 743 | 1456 |
| 8 | 381 | 507 | 1557 |
| 9 | 378 | 609 | 813 |
| 16 | 195 | 507 | 770 |
| 18 | 61 | 816 | 1043 |
| 32 | 218 | 370 | 33 |

Table xx – NG value for various BW

| BW (MHz) | Nused | NG |
| --- | --- | --- |
| 5 | 433 | 439 |
| 7 | 865 | 877 |
| 8.75 | 865 | 877 |
| 10 | 865 | 877 |
| 20 | 1729 | 1733 |

For the decimation separability, the sounding sequence is defined as

(xx)

where

k is the subcarrier index ( 0  k  Nused1),

Nused is the number of usable subcarriers in the sounding symbol,

D is the decimation value (from the sounding instructions),

B is the group of allocated subcarriers/bands according to the sounding instructions,

g is the actual decimation offset as defined in 16e,

m is the cell-specific parameter representing index of short sounding sequence (from the sounding

instructions),

m(x) is the phase of short sounding sequence, which is defined in Tables xx-xx for various D, and

f(k) is the long sounding sequence, which is given in (xx), and  in f(k) is defined in Table xx.

Table xx – value for decimation separability

| D |  |  |  |
| --- | --- | --- | --- |
|  | Nused = 433 | Nused = 865 | Nused = 1729 |
| 4 | 42 | 332 | 436 |
| 6 | 214 | 166 | 820 |
| 8 | 186 | 293 | 191 |
| 9 | 147 | 161 | 87 |
| 16 | 208 | 138 | 480 |
| 18 | 144 | 63 | 192 |
| 32 | 53 | 50 | 866 |

----------------------------------------------------------- Text End ---------------------------------------------------------------