| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Differential CQI structure in IEEE802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Isamu Yoshii Takaaki Kishigami Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: yoshii.isamu@jp.panasonic.com kishigami.takaaki@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.9 (UL-CTRL) Comments on 15.3.9.3.1 Fast Feedback back channel contents |  |
| Abstract | We discuss the differential structure of CQI |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the transmission format of FBCH in IEEE802.16m Amendment

Isamu YOSHII, Takaaki KISHIGAMI

Panasonic Corporation

# Introduction

This contribution proposes transmission format for PFBCH and SFBCH in order to reduce the overhead of UL Fast feedback Channel

AWD describes wideband CQI and subband CQI as following.

PFBCH carries 6 information bits using the 12 bit sequence. The payload of SFBCH is variable size and max size is 24 bits

# Potential contents of PFBCH and SFBCH [1]

The potential contents of PFBCH are described at Table 1 . In case of long period feedback timing, full of contents can not be transmitted by one resource of 　PFBCH.　Additional feedback contents are listed at Table 2.

Table 1  Potential feedback contents to PFBCH [1]

Table 2　 Additional feedback contents to PFBCH or SFBCH[1]

# Potential resource assignment schemes of PFBCH and SFBCH

Potential resource assignment schemes are following 3 schemes.

FBCH RA 1:  Two PFBCH resources are used in long period.

FBCH RA 2-1/2-2:  Simple differential CQI for WB/NB CQI

- In case of non-MIMO transmission, it is not necessary to use differential scheme for WB CQI in short period.

FBCH RA 2-3:  Layered differential CQI for WB/NB CQI

- In long period, CQI-1 is transmitted, in short period CQI-2. The range of CQI-2 depends on pervious CQI-1.

- In case of non-MIMO transmission, it is not necessary to use layered scheme for WB CQI in short period.

Figure 1  FBCH RA1

Figure 2  FBCH RA2-1

Figure 3　　FBCH RA2-2

Figure 4 FBCH RA2-3

Figure 5 MCS table for FBCH RA 2-3

# Conclusions

3 types of differential indication are described as the overhead reduction method.

# Proposed Change Text in 15.3.9.3.1 Fast feedback control channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable.

The payload size of PFBCH is from 4 to 6 bits and SFBCH is from 7 to 24 bits. The contents of PFBCH and SFBCH are following.

|  | Feedback information | Channel | Number of bits |
| --- | --- | --- | --- |
| Long period feedback | Rank information /sream index | PFBCH | 3 |
|  | Band selection | PFBCH | TBD |
| Short period feedback | Wideband CQI | PFBCH | TBD (Differential) |
|  | Wideband PMI | PFBCH | TBD |
|  | Subband CQI | SFBCH | TBD (Differential) |
|  | Subband PMI | SFBCH | TBD |

CQI uses differential scheme. Detail of differential scheme is TBD.

# References

[1] C802.16m-09/0673, “Mapping MIMO feedback contents to fast feedback channel”.

[2] C802.16m-09/0691, “High Fidelity Sub-band CQI Feedback”