| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for subband CQI feedback in IEEE802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7.2.6.1 Downlink post-processing CINR measurement feedback” |  |
| Abstract | We propose the subband CQI information. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for subband CQI feedback in IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

Current AWD text[1] for the subband CQI is defined as ‘one average CQI over the subband’ .

In this contribution, we propose to add an optional mode for subband CQI to reduce the feedback information since the size of SFBCH is limited such as the range from 7 bits to 24 bits.

# Proposed subband CQI feedback

We propose an optional subband CQI feedback mode in addition to the mandatory mode which is defined in current AWD text.

i) Mandatory mode:

The mobile station reports the CQI indices for each of the selected N subbands. [The number of N is TBD].

The CQI index is based on the average CQI over the subband.

In this mode, the total CQI information bits amount to [5] N bits for the selected N subbands when [5] bits carry one subband CQI information.

ii) Optional mode:[newly added]

The mobile station reports the CQI index for the selected N subbands. [The number of N is TBD. ]

The CQI index is based on the average CQI over the selected N subbands.

In this mode, the total CQI information bits amount to only [5] bits for the selected N subbands when [5] bits carry one subband CQI information.

The preferred subband CQI feedback format :

The event-driven feedback is used to inform the base station on preferred the subband CQI feedback format that the mobile station selects among above 2 modes.

The mode selection to be done by the mobile station is based on the propagation environment between the mobile station and the base station. When the deviation between CQIs for the selected N subbands is relatively small, the optional mode could be used. Because the gain loss caused by the CQI information compression could be relatively small in that case.

# References

[1]IEEE 802.16m-09/0010r1a,“802.16m Amendment Working Document (AWD)”

[2]IEEE 802.16 Rev2/D8

# Proposed Text

We propose the following text for the IEEE802.16m Amendment Working Document in Section 15.3.7.2.6.1

-------------------------------  Start text proposal  ---------------------------------------------------

- 15.3.7.2.6 Feedback mechanisms and operation

- 15.3.7.2.6.1 Downlink post-processing CINR measurement feedback

The reported channel quality indicator has two types: wideband CQI, subband CQI.

The wideband CQI is one average CQI over the whole band.

The subband CQI is one average CQI over the subband or one average CQI over the selected subbands as shown in Table x.

Table x – Feedback type and feedback contents

| Feedback information type | Feedback contents (Number of bits) | Description |
| --- | --- | --- |
| subband CQI information | i) ([5] × N )bits [The number of N is TBD.] | i) [mandatory mode] N CQIs for the selected N subbands are reported. Each CQI is averaged over each of the selected subband. |
|  | ii) [5] bits | ii) [optional mode] One average CQI over the selected subbands is reported. |
| Event-driven feedback | Subband CQI feedback format | 0b0: i) [mandatory mode] 0b1: ii) [optional mode] |

For MU-MIMO feedback modes, the CQI is calculated at the MS assuming that the interfering users are scheduled by the serving BS using rank-1 precoders orthogonal to each other and orthogonal to the rank-1 precoder represented by the reported PMI.

-------------------------------  End text proposal  ---------------------------------------------------