| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification on MCS Table for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii, Kenichi Kuri Panasonic Corporation | Voice: +65-65505304 E-mail: lei.huang@sg.panasonic.com |
| Re: | Category: AWD comments /Area: Chapter 15.3.12 (Channel Coding & HARQ) “Comments on AWD 15.3.12 Channel Coding & HARQ” |  |
| Abstract | 802.16m amendment text proposal on MCS table for uplink data channel |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification on MCS Table For IEEE 802.16m Amendment

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii, Kenichi Kuri

Panasonic Corporation

- Introduction

According to the IEEE 802.16m Amendment Working Document (AWD) [2], a single 4-bit MCS table as shown in Table 1 is defined for both downlink (DL) and uplink (UL) data channel. In this contribution, we propose a modification of MCS table shown in Table 1 for UL data channel.

Table 1: MCS table for downlink and uplink data channel

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| ‘0000’ | QPSK | 31/256 |
| ‘0001’ | QPSK | 48/256 |
| ‘0010’ | QPSK | 71/256 |
| ‘0011’ | QPSK | 101/256 |
| ‘0100’ | QPSK | 135/256 |
| ‘0101’ | QPSK | 171/256 |
| ‘0110’ | 16QAM | 102/256 |
| ‘0111’ | 16QAM | 128/256 |
| ‘1000’ | 16QAM | 155/256 |
| ‘1001’ | 16QAM | 184/256 |
| ‘1010’ | 64QAM | 135/256 |
| ‘1011’ | 64QAM | 157/256 |
| ‘1100’ | 64QAM | 181/256 |
| ‘1101’ | 64QAM | 205/256 |
| ‘1110’ | 64QAM | 225/256 |
| ‘1111’ | 64QAM | 237/256 |

- Proposed Modification on MCS Table for Uplink Data Channel

IEEE 802.16m UL supports adaptive and non-adaptive HARQ, i.e. retransmissions may come with or without an UL basic assignment A-MAP IE (i.e., grant). The main reason for having the possibility of having a grant for retransmissions is:

- change in resource allocation and/or MCS;

- control of SPID in case previous grant has been missed.

As a result, in the case of retransmissions with a grant, the resource allocation, MCS, and SPID should be indicated in the UL basic assignment A-MAP IE.

There are two alternatives of signaling SPID and MCS in an UL basic assignment A-MAP IE:

- Alternative 1: there exist a SPID field and a MCS field in UL basic assignment A-MAP IE;

- Alternative 2: there exist only a MCS field, which jointly indicates the SPID and the MCS.

In terms of Alternative 1, since the MCS field in an UL basic assignment A-MAP IE only indicates the MCS, the MCS table shown in Table 1 can be used for UL data channel. However, in terms of Alternative 2, since the MCS field in an UL basic assignment A-MAP IE indicates both SPID and MCS, the MCS table shown in Table 1 cannot directly used for UL data channel.

For Alternative 2, we propose the 3 MCS table entries are reserved for indicating SPID = 1, 2, or 3 for UL HARQ retransmissions, as shown in Table 2. The remaining MCS table entries are used to signal the MCS level implicitly indicating SPID = 0, for both initial grants and retransmission grants. In case of indicating SPID = 1, 2, or 3 in retransmission grants, the same modulation scheme as determined in the initial grant is used.

Table 2: Proposed MCS table for uplink data channel

| MCS index | Modulation | Code rate | SPID |
| --- | --- | --- | --- |
| ‘0000’ | QPSK | 31/256 | 0 |
| ‘0001’ | QPSK | 48/256 | 0 |
| ‘0010’ | QPSK | 71/256 | 0 |
| ‘0011’ | QPSK | 101/256 | 0 |
| ‘0100’ | QPSK | 135/256 | 0 |
| ‘0101’ | QPSK | 171/256 | 0 |
| ‘0110’ | 16QAM | 102/256 | 0 |
| ‘0111’ | 16QAM | 128/256 | 0 |
| ‘1000’ | 16QAM | 155/256 | 0 |
| ‘1001’ | 16QAM | 184/256 | 0 |
| ‘1010’ | 64QAM | 135/256 | 0 |
| ‘1011’ | 64QAM | 157/256 | 0 |
| ‘1100’ | 64QAM | 181/256 | 0 |
| ‘1101’ | N.A |  | 1 |
| ‘1110’ |  |  | 2 |
| ‘1111’ |  |  | 3 |

Compared with Alternative 1, Alternative 2 has a benefit of reduced signaling overhead. On the other hand, 3 MCS levels are lost for signaling SPID for Alternative 2. This may result in degradation in UL throughput. However, if 5-bit MCS table is adopted for IEEE 802.16m, the degradation in UL throughput may be trivial for Alternative 2. Please note that there exist a lot of interests in promoting 5-bit MCS table instead of 4-bit MCS table for IEEE 802.16m [3].

In addition, Alternative 2 is able to overcome the initial grant loss problem. If MS missed the initial grant, and but the BS is not sure about this, at the second grant, the following would happen for joint signaling scheme

- Case A: BS indicates SPID = 0 and MCS (i.e., MCS index between 0000 and 1100”), MS transmits PHY burst and protocol continues as usual (PHY burst not lost)

- Case B: BS indicates SPID = 1, 2 or 3 (i.e., MCS index between 1101 and 1111), then the MS does not know the PHY burst size, and MS would do DTX. If BS can do DTX detection of UL data transmission, after DTX detection of UL data transmission BS can react accordingly as in Case A. Note that DTX detection of UL data transmission is a bit more reliable than DTX detection of UL ACK/NACK. If BS cannot do DTX detection of UL data transmission, after some retransmissions BS will schedule SPID = 0 and MCS again (i.e., MCS index between 0000 and 1100”).

Based on the above analysis, we recommend that the proposed MCS table shown in Table 2 is used for uplink data channel.

- Summary

In this contribution, we propose a modification of MCS table for uplink data channel, as shown in Table 2. Note that Table 2 is a 4-bit MCS table for uplink data channel. However, it is straightforward to apply the suggested modification to 5-bit MCS table for uplink data channel.

- Reference

[1] IEEE 802.16m-08/003r8, IEEE 802.16m System Description Document [draft], April 2009

[2] IEEE 802.16m-09/0010r1a, IEEE 802.16m Amendment Working Document, March 2009

[3] IEEE C802.16m-09/0748, TGm channel coding and HARQ drafting group meeting summary

# Suggested Amendment Text

----------------------------------------------------------------------------------

# 15.3.12.1 Channel coding

# 15.3.12.1.3 Burst partition

Suggested Modification 1:

Modifying

‘The nominal MCS used in a data transmission shall be selected from Table 700’

To

‘The nominal MCS used in a downlink data transmission shall be selected from Table 700a’

Modifying the title of Table 700

‘Table 700: Proposed MCS table for downlink and uplink data channel’

To

‘Table 700a : Proposed MCS table for downlink data channel’

Suggested Modification 2: Inserting the following text and table after Table 700a.

The nominal MCS used in an uplink data transmission shall be selected from Table 700b

Table 700b: Proposed MCS table for uplink data channel

| MCS index | Modulation | Code rate | SPID |
| --- | --- | --- | --- |
| ‘0000’ | QPSK | 31/256 | 0 |
| ‘0001’ | QPSK | 48/256 | 0 |
| ‘0010’ | QPSK | 71/256 | 0 |
| ‘0011’ | QPSK | 101/256 | 0 |
| ‘0100’ | QPSK | 135/256 | 0 |
| ‘0101’ | QPSK | 171/256 | 0 |
| ‘0110’ | 16QAM | 102/256 | 0 |
| ‘0111’ | 16QAM | 128/256 | 0 |
| ‘1000’ | 16QAM | 155/256 | 0 |
| ‘1001’ | 16QAM | 184/256 | 0 |
| ‘1010’ | 64QAM | 135/256 | 0 |
| ‘1011’ | 64QAM | 157/256 | 0 |
| ‘1100’ | 64QAM | 181/256 | 0 |
| ‘1101’ | N.A |  | 1 |
| ‘1110’ |  |  | 2 |
| ‘1111’ |  |  | 3 |

---------------------------------------------------------------------------------