| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Channel Coding and HARQ for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Tom Harel, Huaning Niu, Changlong Xu, Yuval Lomnitz, Jong-Kae(JK) Fwu, Yangsoek Choi, Hujun Yin; Intel Corp Chien-Yu Kao, Yu-Tao Hsieh, Yu-Chuan Fang, Zheng Yan-Xiu, Pang-An Ting; ITRI Seunghyun Kang, Hyungho Park, Jinsam Kwak; LG Electronics Ciou-Ping Wu, Pei-Kai Liao, Yu-Hao Chang, Yih-Shen Chen, Paul Cheng; MediaTek Sun Bo, Xu Jin; ZTE Sung-Eun Park, Zheng Zhao, Lei Zhou, Seunghoon Choi, Chiwoo Lim, Songnam Hong; Samsung Electronics Lei Huang; Panasonic Singapore Laboratories Isamu Yoshii; Panasonic Corporation | tom.harel@intel.com Eric_Kao@itri.org.tw sh_kang@lge.com ciouping.wu@mediatek.com sun.bo1@zte.com.cn se.park@samsung.com lei.huang@sg.panasonic.com |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes the text of channel coding and HARQ section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Channel Coding and HARQ 
for the IEEE 802.16m Amendment

Tom Harel, Huaning Niu, Changlong Xu, Yuval Lomnitz, Jong-Kae(JK) Fwu, Yangsoek Choi,Hujun Yin

Intel Corp

Chien-Yu Kao, Yu-Tao Hsieh, Yu-Chuan Fang, Zheng Yan-Xiu, Pang-An Ting

ITRI

Seunghyun Kang, Hyungho Park, Jinsam Kwak

LG Electronics

Ciou-Ping Wu, Pei-Kai Liao, Yu-Hao Chang, Yih-Shen Chen, Paul Cheng

MediaTek

Sun Bo, Xu Jin

ZTE

Sung-Eun Park, Zheng Zhao, Lei Zhou, Seunghoon Choi, Chiwoo Lim, Songnam Hong

Samsung Electronics

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii

Panasonic Corporation

- Introduction

The contribution proposes the text of channel coding and HARQ section to be included in the 802.16m amendment ([1]).

- References

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[4] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- Abbreviations and acronyms

Insert the following at section 4 in alphabetic order:

CoRe		constellation re-arrangement

HARQ		hybrid-ARQ

IR		incremental redundancy

MCS		Modulation and Coding Scheme

SPID		subpacket ID

Insert the following subsection at section 15:

<<//////////          Proposed text #1 ////////////////////////>>

- 15.x.1.3. 	Burst partition

Only the burst size NDB limited in Table 15.x,1-1 are supported in PHY layer. These sizes include the addition of CRC (per burst and per FEC block) when application. Other sizes require padding to the next burst size. When the burst size including 16 burst CRC bits exceeds the maximum FEC block size, NFB_MAX, the burst is partitioned into KFB FEC blocks. The modulation order, the nominal code rate, the number of resource elements allocated for data transmission, the spatial multiplexing order, and the maximum FEC block size are denoted by Nmod, RFEC, NRE, NSM, NFB_MAX, respectively.

Nmod 	is the modulation order of the burst transmission, which is specified by MCS index

RFEC 	is the nominal code rate of the burst transmission, which is specified by MCS index

NRE 		is the number of resource elements allocated for burst transmission. The calculation of NRE shall exclude the resource elements used by pilot channels

NSM		is the spatial multiplexing order of the resource elements allocated for burst transmission. The value of NSM depends on the MIMO transmission mode and is defined in section x.x.x.x.

NFB_MAX	is the maximum FEC block size, which equals to 4800 bits.

Table 15.x.1-1. Burst size table

| idx | NDB | KFB | idx | NDB | KFB | idx | NDB (bytes) | KFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1 | 23 | 90 | 1 | 45 | 1200 | 2 |
| 2 | 8 | 1 | 24 | 100 | 1 | 46 | 1416 | 3 |
| 3 | 9 | 1 | 25 | 114 | 1 | 47 | 1584 | 3 |
| 4 | 10 | 1 | 26 | 128 | 1 | 48 | 1800 | 3 |
| 5 | 11 | 1 | 27 | 145 | 1 | 49 | 1888 | 4 |
| 6 | 12 | 1 | 28 | 164 | 1 | 50 | 2112 | 4 |
| 7 | 13 | 1 | 29 | 181 | 1 | 51 | 2400 | 4 |
| 8 | 15 | 1 | 30 | 205 | 1 | 52 | 2640 | 5 |
| 9 | 17 | 1 | 31 | 233 | 1 | 53 | 3000 | 5 |
| 10 | 19 | 1 | 32 | 262 | 1 | 54 | 3600 | 6 |
| 11 | 22 | 1 | 33 | 291 | 1 | 55 | 4200 | 7 |
| 12 | 25 | 1 | 34 | 328 | 1 | 56 | 4800 | 8 |
| 13 | 27 | 1 | 35 | 368 | 1 | 57 | 5400 | 9 |
| 14 | 31 | 1 | 36 | 416 | 1 | 58 | 6000 | 10 |
| 15 | 36 | 1 | 37 | 472 | 1 | 59 | 6600 | 11 |
| 16 | 40 | 1 | 38 | 528 | 1 | 60 | 7200 | 12 |
| 17 | 44 | 1 | 39 | 600 | 1 | 61 | 7800 | 13 |
| 18 | 50 | 1 | 40 | 656 | 2 | 62 | 8400 | 14 |
| 19 | 57 | 1 | 41 | 736 | 2 | 63 | 9600 | 16 |
| 20 | 64 | 1 | 42 | 832 | 2 | 64 | 10800 | 18 |
| 21 | 71 | 1 | 43 | 944 | 2 | 65 | 12000 | 20 |
| 22 | 80 | 1 | 44 | 1056 | 2 | 66 | 14400 | 24 |

The burst size index is calculated as , where  is a 5 bits index in A-MAP IE, and  is calculated based on Table 15.x.1-2 and the allocation size, The allocation size is defined as the number of LRUs multiplied by the MIMO rank that are allocated for the burst .

Table 15.x.1-2 - Minimal size index as function of number of allocated LRUs

| Alloc. size |  | Alloc. size |  | Alloc. size |  | Alloc. size |  | Alloc. size |  | Alloc. size |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 33 | 21 | 65 | 27 | 97 | 30 | 129 | 32 | 161 | 34 |
| 2 | 1 | 34 | 21 | 66 | 27 | 98 | 30 | 130 | 32 | 162 | 34 |
| 3 | 1 | 35 | 21 | 67 | 27 | 99 | 30 | 131 | 32 | 163 | 34 |
| 4 | 2 | 36 | 22 | 68 | 27 | 100 | 30 | 132 | 33 | 164 | 34 |
| 5 | 4 | 37 | 22 | 69 | 27 | 101 | 30 | 133 | 33 | 165 | 35 |
| 6 | 6 | 38 | 22 | 70 | 27 | 102 | 30 | 134 | 33 | 166 | 35 |
| 7 | 8 | 39 | 22 | 71 | 27 | 103 | 31 | 135 | 33 | 167 | 35 |
| 8 | 9 | 40 | 22 | 72 | 27 | 104 | 31 | 136 | 33 | 168 | 35 |
| 9 | 10 | 41 | 23 | 73 | 28 | 105 | 31 | 137 | 33 | 169 | 35 |
| 10 | 11 | 42 | 23 | 74 | 28 | 106 | 31 | 138 | 33 | 170 | 35 |
| 11 | 11 | 43 | 23 | 75 | 28 | 107 | 31 | 139 | 33 | 171 | 35 |
| 12 | 12 | 44 | 23 | 76 | 28 | 108 | 31 | 140 | 33 | 172 | 35 |
| 13 | 13 | 45 | 23 | 77 | 28 | 109 | 31 | 141 | 33 | 173 | 35 |
| 14 | 14 | 46 | 24 | 78 | 28 | 110 | 31 | 142 | 33 | 174 | 35 |
| 15 | 14 | 47 | 24 | 79 | 28 | 111 | 31 | 143 | 33 | 175 | 35 |
| 16 | 15 | 48 | 24 | 80 | 28 | 112 | 31 | 144 | 33 | 176 | 35 |
| 17 | 15 | 49 | 24 | 81 | 28 | 113 | 31 | 145 | 33 | 177 | 35 |
| 18 | 15 | 50 | 24 | 82 | 28 | 114 | 31 | 146 | 34 | 178 | 35 |
| 19 | 16 | 51 | 25 | 83 | 29 | 115 | 31 | 147 | 34 | 179 | 35 |
| 20 | 16 | 52 | 25 | 84 | 29 | 116 | 31 | 148 | 34 | 180 | 35 |
| 21 | 17 | 53 | 25 | 85 | 29 | 117 | 32 | 149 | 34 | 181 | 35 |
| 22 | 17 | 54 | 25 | 86 | 29 | 118 | 32 | 150 | 34 | 182 | 35 |
| 23 | 18 | 55 | 25 | 87 | 29 | 119 | 32 | 151 | 34 | 183 | 35 |
| 24 | 18 | 56 | 25 | 88 | 29 | 120 | 32 | 152 | 34 | 184 | 35 |
| 25 | 18 | 57 | 25 | 89 | 29 | 121 | 32 | 153 | 34 | 185 | 36 |
| 26 | 19 | 58 | 26 | 90 | 29 | 122 | 32 | 154 | 34 | 186 | 36 |
| 27 | 19 | 59 | 26 | 91 | 30 | 123 | 32 | 155 | 34 | 187 | 36 |
| 28 | 19 | 60 | 26 | 92 | 30 | 124 | 32 | 156 | 34 | 188 | 36 |
| 29 | 20 | 61 | 26 | 93 | 30 | 125 | 32 | 157 | 34 | 189 | 36 |
| 30 | 20 | 62 | 26 | 94 | 30 | 126 | 32 | 158 | 34 | 190 | 36 |
| 31 | 20 | 63 | 26 | 95 | 30 | 127 | 32 | 159 | 34 | 191 | 36 |
| 32 | 20 | 64 | 26 | 96 | 30 | 128 | 32 | 160 | 34 | 192 | 36 |

The modulation order Nmod (2 for QPSK, 4 for 16-QAM and 6 for 64-QAM) depends on the parameter  according to Table 15.x.1-3 rule. Allocation size of 1 or 2 LRUs are special cases (separate columns in the table). For allocation of at least 3 LRUs the modulation order depends only on .

Table 15.x.1-3 - Rule for modulation order

|  | Nmod (allocation size ≥ 3) | Nmod (allocation size = 2) | Nmod (allocation size = 1) |  | Nmod (allocation size ≥ 3) | Nmod (allocation size = 2) | Nmod (allocation size = 1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 2 | 2 | 2 | 16 | 2 | 4 | 6 |
| 1 | 2 | 2 | 2 | 17 | 2 | 4 | 6 |
| 2 | 2 | 2 | 2 | 18 | 2 | 4 | 6 |
| 3 | 2 | 2 | 2 | 19 | 4 | 4 | 6 |
| 4 | 2 | 2 | 2 | 20 | 4 | 4 | 6 |
| 5 | 2 | 2 | 2 | 21 | 4 | 4 | 6 |
| 6 | 2 | 2 | 2 | 22 | 4 | 6 | 6 |
| 7 | 2 | 2 | 2 | 23 | 4 | 6 | 6 |
| 8 | 2 | 2 | 2 | 24 | 6 | 6 | 6 |
| 9 | 2 | 2 | 2 | 25 | 6 | 6 | 6 |
| 10 | 2 | 2 | 4 | 26 | 6 | 6 | 6 |
| 11 | 2 | 2 | 4 | 27 | 6 | 6 | 6 |
| 12 | 2 | 2 | 4 | 28 | 6 | 6 | 6 |
| 13 | 2 | 2 | 4 | 29 | 6 | 6 | 6 |
| 14 | 2 | 2 | 4 | 30 | 6 | 6 | 6 |
| 15 | 2 | 2 | 4 | 31 | 6 | 6 | 6 |

The nominal MCS used in a data transmission rank-1 CQI feedback shall be selected from Table 15.x.1-1.

Table 15.x.1-14. MCS table for rank-1 CQI downlink and uplink data channel

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| ‘0000’ | QPSK | 31/256 |
| ‘0001’ | QPSK | 48/256 |
| ‘0010’ | QPSK | 71/256 |
| ‘0011’ | QPSK | 101/256 |
| ‘0100’ | QPSK | 135/256 |
| ‘0101’ | QPSK | 171/256 |
| ‘0110’ | 16QAM | 102/256 |
| ‘0111’ | 16QAM | 128/256 |
| ‘1000’ | 16QAM | 155/256 |
| ‘1001’ | 16QAM | 184/256 |
| ‘1010’ | 64QAM | 135/256 |
| ‘1011’ | 64QAM | 157/256 |
| ‘1100’ | 64QAM | 181/256 |
| ‘1101’ | 64QAM | 205/256 |
| ‘1110’ | 64QAM | 225/256 |
| ‘1111’ | 64QAM | 237/256 |

The size of the k-th FEC block is denoted by NFB, k, k = 0, 1, …, KFB-1. The set of supported FEC block sizes is shown in Table 15.x.1-2.

Table 15.x.1-2. FEC block size table for downlink and uplink data channels

| Index | NFB | Index | NFB | Index | NFB | Index | NFB | Index | NFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 30 | 328 | 60 | 720 | 90 | 1424 | 120 | 2752 |
| 1 | 64 | 31 | 344 | 61 | 736 | 91 | 1448 | 121 | 2816 |
| 2 | 72 | 32 | 352 | 62 | 752 | 92 | 1480 | 122 | 2880 |
| 3 | 80 | 33 | 360 | 63 | 768 | 93 | 1504 | 123 | 2944 |
| 4 | 88 | 34 | 368 | 64 | 776 | 94 | 1536 | 124 | 3008 |
| 5 | 96 | 35 | 376 | 65 | 800 | 95 | 1560 | 125 | 3072 |
| 6 | 104 | 36 | 384 | 66 | 824 | 96 | 1600 | 126 | 3200 |
| 7 | 120 | 37 | 400 | 67 | 848 | 97 | 1640 | 127 | 3264 |
| 8 | 128 | 38 | 416 | 68 | 872 | 98 | 1672 | 128 | 3328 |
| 9 | 136 | 39 | 432 | 69 | 888 | 99 | 1712 | 129 | 3392 |
| 10 | 144 | 40 | 440 | 70 | 912 | 100 | 1752 | 130 | 3456 |
| 11 | 152 | 41 | 456 | 71 | 936 | 101 | 1784 | 131 | 3520 |
| 12 | 160 | 42 | 472 | 72 | 960 | 102 | 1824 | 132 | 3648 |
| 13 | 176 | 43 | 480 | 73 | 984 | 103 | 1864 | 133 | 3712 |
| 14 | 184 | 44 | 496 | 74 | 1000 | 104 | 1896 | 134 | 3776 |
| 15 | 192 | 45 | 512 | 75 | 1024 | 105 | 1920 | 135 | 3840 |
| 16 | 200 | 46 | 528 | 76 | 1048 | 106 | 1952 | 136 | 3904 |
| 17 | 208 | 47 | 544 | 77 | 1072 | 107 | 2000 | 137 | 3968 |
| 18 | 216 | 48 | 552 | 78 | 1096 | 108 | 2048 | 138 | 4096 |
| 19 | 232 | 49 | 568 | 79 | 1112 | 109 | 2096 | 139 | 4160 |
| 20 | 240 | 50 | 584 | 80 | 1136 | 110 | 2144 | 140 | 4224 |
| 21 | 248 | 51 | 600 | 81 | 1160 | 111 | 2192 | 141 | 4288 |
| 22 | 256 | 52 | 608 | 82 | 1184 | 112 | 2232 | 142 | 4352 |
| 23 | 264 | 53 | 624 | 83 | 1216 | 113 | 2280 | 143 | 4416 |
| 24 | 272 | 54 | 640 | 84 | 1248 | 114 | 2328 | 144 | 4544 |
| 25 | 288 | 55 | 656 | 85 | 1280 | 115 | 2368 | 145 | 4608 |
| 26 | 296 | 56 | 664 | 86 | 1312 | 116 | 2432 | 146 | 4672 |
| 27 | 304 | 57 | 680 | 87 | 1336 | 117 | 2496 | 147 | 4736 |
| 28 | 312 | 58 | 696 | 88 | 1368 | 118 | 2560 | 148 | 4800 |
| 29 | 320 | 59 | 712 | 89 | 1392 | 119 | 2624 |  |  |

The burst size NDB including burst CRC and FEC block CRC is defined by the following equation:

NDB=k=0KFB−1NFB,  k.

The payload size excluding burst CRC and FEC block CRC is defined by the following equation:

NPL=NDB−NDB−CRC−IMFB⋅KFB⋅NFB−CRC

where,

IMFB		equals 0 when KFB =1, 1 when KFB >1,
NFB-CRC	equals 16, which is the size of FEC block CRC,
NDB-CRC	equals 16, which is the size of burst CRC.

The burst partition block generates KFB FEC blocks, with each FEC block processed by the FEC block CRC encoding block as described in 15.x.1.4.

<<//////////          Proposed text #2 ////////////////////////>>

- 15.x.1.5.1.7 		Bit grouping

Corresponding to section 8.4.9.2.3.4.3.

The interleaved subblocks shall be multiplexed into four blocks; those four blocks consist of an interleaved A subblock, an interleaved B subblock, a bit-by-bit multiplexed sequence of the interleaved Y1 and Y2 subblock sequences, which is referred to Y, and a bit-by-bit multiplexed sequence of the interleaved W2 and W1 subblock sequences, which is referred to W. Information subblocks, A and B, are by-passed while parity subblocks are multiplexed bit by bit. The bit-by-bit multiplexed sequence of interleaved Y1 and Y2 subblock sequences shall consist of the first output bit from the Y1 subblock interleaver, the first output bit from the Y2 subblock interleaver, the second output bit from the Y1 subblock interleaver, the second output bit from the Y2 subblock interleaver, etc. The bit-by-bit multiplexed sequence of interleaved W2 and W1 subblock sequences shall consist of the first output bit from the W2 subblock interleaver, the first output bit from the W1 subblock interleaver, the second output bit from the W2 subblock interleaver, the second output bit from the W1 subblock interleaver, etc.

After multiplexing subblocks into four blocks, Subblock B and Subblock W are circularly left-shifted by k bits. Subblock Y is circularly left-shifted by 1 bit. When the FEC block size Nep is equal to multiple of the modulation order, k is set as 1. Otherwise, let k be 0. Figure 461 shows interleaving scheme as explained above.

Figure 15.1-1.x Block diagram of interleaving scheme

<<//////////          Proposed text #3 ////////////////////////>>

- 15.x.1.5.1.9		Bit selection and repetition

Bit selection and repetition are performed to generate the subpacket.

Let NCTC, k be the number of coded bits that shall be transmitted for the k-th FEC block. The value of NCTC, k is calculated by the following equation

.

The index in the HARQ buffer for the j-th bit transmitted for the k-th FEC block, uk, j, i, shall be:

uk, j=(Pi,k+j)mod NFB_Buffer, k

for k = 0, 1, …, KFB – 1, and j = 0, 1, …, NCTC, k – 1,

where i is the subpacket ID of the subpacket (SPID = i), Pi, k is the starting position for subpacket i of the k-th FEC block as specified in 15.x.2.1, and  is the buffer size for the k-th FEC block.

- 15.x.2	HARQ

<<//////////          Proposed text #4 ////////////////////////>>

- 15.x.2.1	IR HARQ

HARQ IR is used in 802.16m by changing the starting position, Pi, k, of the bit selection for HARQ retransmissions.

For downlink HARQ, the starting point for the bit selection algorithm as described in section 15.x.1.5.1.8 is determined as a function of SPID using Table 15.x.2-1 .

Table 15.x.2-1. Starting position determination for downlink HARQ

| SPID | Starting position Pi,k |
| --- | --- |
| 0 | Pi,k =0 |
| 1 | Pi,k =−NCTC,kmod NFBBuffer,k |
| 2 | TBD NFBBuffer,k2−NCTC,k2mod NFBBuffer,k |
| 3 | TBD NFBBuffer,k−NCTC,k2mod NFBBuffer,k |

For uplink HARQ, the starting position for the bit selection algorithm as described in section 15.x.1.5.1.8 is determined as a function of SPID for the following equation.

.

For uplink HARQ, subpackets shall be transmitted in sequential order. In other words, for the t-th transmission, the subpacket ID shall be set to SPID = t mod 4.

<<//////////          Proposed text #5 ////////////////////////>>

- 15.x.2.1.1.1.	Constellation Rearrangement

Two constellation rearrangement (CoRe) versions shall be supported. Co-Re only applies to 16QAM and 64QAM. In case of QPSK, it is transparent.

Following table describes the operations that produce different CoRe versions for 16QAM and 64QAM respectively, so that the four bits in a 16QAM symbol (the six bits in a 64 QAM) are of the same resilience. In other words, the two bits of higher quality at CoRe-version 0 are of lower quality at CoRe-version 1 while the two bits of lower quality at CoRe-version 0 are of higher quality at CoRe-version 1.

Table 15.x.2.1.1.1-1: Constellation rearrangement version (rank =1)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - |
| 16 QAM | 4 | 1 | b3 | b2 | b1 | b0 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 |
| 64 QAM | 6 | 1 | b5 | b4 | b3 | b2 | b1 | b0 |

Table 15.x.2.1.1.1-2: Constellation rearrangement version (rank ≥ 2)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | First symbol |  |  |  |  |  | Second symbol |  |  |  |  |  |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - | b4 | b5 | b6 | b7 | - | - |
| 16 QAM | 4 | 1 | b1 | b4 | b3 | b6 | - | - | b5 | b0 | b7 | b2 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 | b9 | b10 | b11 |
| 64 QAM | 6 | 1 | b2 | b7 | b0 | b5 | b10 | b3 | b8 | b1 | b6 | b11 | b4 | b9 |

<<//////////          Proposed text #6 ////////////////////////>>

In DL HARQ operation, CRV is signaled for each subpacket to an AMS as a starting value for CRV. In UL HARQ operation, the information on CRVs is implicitly known at AMS and ABS. The CRV shall be changed to the next version of CoRe in DL and UL HARQ operation whenever the transmitted bits wraparound at the end of circular buffer.

-------------------------------  Text End  ---------------------------------------------------