| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Constellation Rearrangement for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Xu Jin, Sun Bo ZTE Corporation | {xu.jin7, sun.bo1}@zte.com.cn |
| Re: | Category: AWD comments / Area: Chapter 15.3.12 (Channel coding HARQ-PHY) “Comments on AWD 15.3.12 Channel coding HARQ-PHY” |  |
| Abstract | The contribution proposes the text of HARQ section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Channel Coding and HARQ 
for the IEEE 802.16m Amendment

Xu Jin, Sun Bo

ZTE Corporation

- Introduction

The contribution proposes the text of channel coding and HARQ section to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

This contribution proposes text to be included in the HARQ section of the 802.16m amendment working document (AWD).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- Abbreviations and acronyms

Insert the following at section 3 in alphabetic order:

CoRe		constellation re-arrangement

HARQ		hybrid-ARQ

IR			incremental redundancy

MCS			Modulation and Coding Scheme

SPID			subpacket ID

CRV			constellation re-arrangement version

Ncbps 		Number of coded bits per QAM symbol (2/4/6)

Insert the following subsection at a new section 15.3.12.2.1.2:

- 15.3.12.2.1.1 Constellation rearrangement

Two constellation re-arrangement versions shall be supported. Constellation rearrangement only applies to 16QAM and 64QAM. In case of QPSK, it is transparent.

In one HARQ process, all transmissions/retransmissions use the same constellation rearrangement scheme. In this particular constellation rearrangement scheme, two constellation rearrangement versions are exploited for 16QAM and 64 QAM modulation respectively.

Following table describes the operations that produce different Co-Re versions for 16QAM and 64QAM respectively, so that the four bits in a 16QAM symbol (the six bits in a 64 QAM) are of the same resilience. In other words, the two bits of higher quality at CoRe-version 0 are of lower quality at CoRe-version 1 while the two bits of lower quality at CoRe-version 0 are of higher quality at CoRe-version 1.

Table 15.3.12.2.1.2-1: Constellation re-arrangement version (rank =1)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - |
| 16 QAM | 4 | 1 | b3 | b2 | b1 | b0 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 |
| 64 QAM | 6 | 1 | b5 | b4 | b3 | b2 | b1 | b0 |

Table 15.3.12.2.1.2-2: Constellation re-arrangement version (rank ≥ 2)

| Constellation | Ncbps | CRV | Mapping rule |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | First symbol |  |  |  |  |  | Second symbol |  |  |  |  |  |
| 16 QAM | 4 | 0 | b0 | b1 | b2 | b3 | - | - | b4 | b5 | b6 | b7 | - | - |
| 16 QAM | 4 | 1 | b1 | b4 | b3 | b6 | - | - | b5 | b0 | b7 | b2 | - | - |
| 64 QAM | 6 | 0 | b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 | b9 | b10 | b11 |
| 64 QAM | 6 | 1 | b2 | b7 | b0 | b5 | b10 | b3 | b8 | b1 | b6 | b11 | b4 | b9 |

-------------------------------  Text End  ---------------------------------------------------