| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Multicarrier for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jaehee Cho, Samsung Electronics Co., Ltd. 416 Maetan-3, Suwon, 442-600, Korea | Voice: +82-31-279-5596 E-mail: jaehee1.cho@samsung.com |
| Re | “IEEE 802.16m amendment text” IEEE 802.16m-09/020, Target topic: “Multicarrier”. |  |
| Abstract | The contribution proposes the text for Multicarrier |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Multicarrier
for the IEEE 802.16m Amendment

Jaehee Cho

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes the text of multicarrier to be included in the IEEE 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D7 [1], it is compliant to the 802.16m SRD [2], 802.16m SDD [4] and it follows the style and format guidelines in [3].

- Proposal in Multicarrier

Data transmission in multicarrier operation can be categorized as follows

- A modulated symbol sequence is carried in one of active carriers

- A modulated symbol sequence is segmented into multiple bursts and the bursts are carried in more than one active carriers

Followings are key properties of each traffic transmission scheme

- A modulated symbol sequence is carried in one of active carriers

  - HARQ feedback channel shall be allocated in the carrier

  - Allocation information for the burst shall be located in the carrier as if single carrier allocation

- A modulated symbol sequence is segmented into multiple bursts

  - It shall be treated as one HARQ PHY burst if HARQ is applied.

    - In this case, HARQ feedback channel shall be located in the primary carrier.

  - Allocation information for the burst shall be located in the primary carrier

- MAP IE design for the segmented modulated symbol sequence

  - MC USCCH always follows ordinary SC USCCH

  - It indicates addresses of resources in other carriers

  - It includes carrier bitmap indicating carriers of addresses

    - Bitmap spans from primary to last secondary carrier

    - Addresses in precedent SC USCCH and following MC USCCH(s) correspond to carriers indicated by the carrier bitmap

  - The following diagram shows example of multicarrier allocation information

- References

[1] IEEE P802.16 Rev2 / D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE C802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[4] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- 15. Advanced Air Interface

- 15.3Physical layer

[Insert the new section as follows]

- 15.3.3.7 Data burst allocation

15.3.3.7.1 Data Burst Allocation for multi-carrier support

MAC data shall be processed as described in FEC and modulation section and can be mapped to OFDMA data region (see x.x.x) in one of primary or secondary carriers. The allocation information shall be sent through the carrier where the OFDMNA data region is located. The same USCCH IE used for single carrier allocation is used for the allocation.

Alternatively, MAC data shall be processed as described in FEC and modulation section and can be mapped to OFDMA data regions (see x.x.x) over multiple carriers using the algorithms defined below.

- Segment the modulated data sequence into blocks sized to fit into a one LRU.

- Map the each segmented data block onto one LRU from the lowest LRU index in the OFDMA data region of the carrier with the lowest logical carrier index.

- Continue the mapping so that the LRU index increases. When the edge of the data region is reached, continue the mapping from the lowest LRU index in the OFDMA data region of the carrier with the next available logical carrier index.

- Continue the mapping until the all modulated data symbols are mapped.

This type of allocation is only allowed in DRU and the allocation information (MC USCCH, see xxx) shall be sent through the primary carrier. An example is shown in the figure below. For the long TTI in multicarrier support mode, apply the algorithm above from the sub-frame with the lowest sub-frame index. When the all LRUs in data region of the sub-frame are mapped with the modulated data symbols continue the mapping from the next sub-frame. Continue the mapping until the all modulated data symbols are mapped

Figure 1 – Example of data mapping in the multicarrier support

[Insert the new section as follows]

15.3.6.4.2.9 MC allocation IE

MC allocation IE is used to allocate resources across multiple carriers (see 15.3.3.7.1). MC allocation IE shall always follow ordinary SC allocation IEs or MC allocation IE and indicates addresses of resource in designated carriers with the same allocation information of the latest precedent SC allocation IEs. The latest precedent SC allocation IEs shall allocate only DRU resources.

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MC allocation IE { Assignment type Carrier bitmap Resource Indexing Resource Indexing CRC Reserved } | 4 4 10 10 16 zz | 0xYY (MC allocation IE) |
|  | 44 + z | Align with max USCCH IE size |

Carrier bitmap

It indicates which carriers are used for MC allocation across carriers. “0’ means no use and ‘1’ means to use the carrier. The n-th bit corresponds to (4*k + n) th logical carrier where n and k start from 0 and k is the order of MC allocation IE after the latest precedent SC allocation IE.

Resource Indexing

There can be total R RIs in one SC allocation IE and following m consecutive MC allocation IEs. The r-th RI corresponds to r-th carrier among carriers whose Carrier bitmap are ‘1’s. R ranges between 0 and R-1.

-------------------------------  Text End  ---------------------------------------------------