| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the Draft P802.16m Amendment on the PHY Structure for BWREQ channel |  |
| Date Submitted | 2009-05-06 |  |
| Source(s) | Yuan Zhu, Xiangying Yang, Qinghua Li, Changlong Xu, Jong-Kae Fwu, Hujun Yin | E-mail: (yuan.y.zhu, xiangying.yang, jong-kae.fwu)@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.9 (UL-CTRL) “Comments on AWD 15.3.9 UL-CTRL” 802.16m amendment working document |  |
| Abstract | Proposed text for the PHY structure for UL control in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the Draft P802.16m Amendment on the PHY Structure for BWREQ Control

Yuan Zhu, Xiangying Yang, Qinghua Li, Changlong Xu, Jong-Kae Fwu, Hujun Yin

Intel

Introduction

This contribution provides additional amendment text for the BWREQ channel Structure in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The proposed text is developed so that it can be readily combined with IEEE 802.16m-09/0010r1 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. The related simulation results is in [5].

References

[1] IEEE 802.16m-09/0010r1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” March. 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0876, “BWREQ updated LLS performance with TBCC coding|”

Proposed Amendment Text

The following text is proposed for inclusion in Section 15 of the IEEE 802.16m Amendment Working Document [1].

# 15.3.9.1.5 Bandwidth request Channel

In the LZone with PUSC, a BW REQ tile is defined of four contiguous subcarriers by six OFDM symbols. The number of BW REQ tiles per BW REQ channel is three or six. Each BW REQ tile carries

In the MZone, a BW REQ tile is defined as six contiguous subcarriers by six OFDM symbols. Each BW REQ channel consists of three distributed BW-REQ tiles. Each BW REQ tile carries a BW REQ access sequence and a BW REQ message. The AMS may transmit the access sequence only and leave the resources for quick access message unused.

[  ------------------- Proposed Text Starts --------------------]

Figure 1: 6x6 BW REQ Tile Structure in the Advance Air Interface

A BW REQ tile in the Advanced Air Interface specifications is defined as 6 contiguous subcarriers by 6 OFDM symbols. As shown in Figure 1, the BW REQ preamble shall be transmitted on a resource that spans 4 subcarriers by 6 OFDM symbols. Additionally, a quick access BW REQ message shall be carried in the data portion of the tile that spans 2 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel shall comprise of 3 distributed BW REQ tiles for frequency diversity. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

Let  denote a total of 16 bits of information to be carried as the quick access message. 4 bits of the 16 information bits shall be carried in the BW REQ preamble using the preamble index. The combined resource in the data portions of the three tiles that form the BW REQ channel shall be used to transmit the remaining 12 bits of information,. The construction of the BW REQ preamble for a 6x6 tile structure is TBD.

The 12 bits of information in the quick access message transmitted in the BW REQ channel shall be encoded into 72 bits using the 1/6 TBCC code described in section XXX with parameters ( ,  and ). The 72 coded bits shall then be QPSK modulated as described in Section TBD and scrambled to generate 36 data symbols, .  The combined data portions of the three distributed BW REQ tiles that form the BW REQ channel shall be used to transmit these data symbols.

In order to support operation in the legacy mode, a BW REQ tile shall be defined as 4 contiguous subcarriers by 6 OFDM symbols. As shown in Figure 2, only the BW REQ access sequence or BW REQ preamble shall be transmitted in all 24 subcarriers that form the BW REQ tile. In this case, the BWREQ_PREAMBLE_INDEX shall be randomly selected from all available logical preamble sequences, as shown in Figure 4.

Figure 2: 4x6 BW REQ tile structure

Figure 3: 4x6 Bandwidth Request Channel structure with 4x6 tiles

[  ------------------- Proposed Text Ends --------------------]