| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text to DL Extended Assignment A-MAP IE and UL Extended HARQ Feedback Channel |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Zheng Yan-Xiu , Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang, Hsi-Min Hsiao ITRI | zhengyanxiu@itri.org.tw |
| Re: | IEEE 802.16m-09/0020 “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content.” Chapter 15.3.6.4.2.2 DL basic assignment A-MAP IE Chapter 15.3.9.2.2 HARQ feedback control channel |  |
| Abstract | This contribution proposed a text change to downlink and uplink basic assignment A-MAP IEs. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to DL/UL Assignment A-MAP IEs

Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang, His-Min Hsiao

ITRI

- Introduction

This contribution proposes text to downlink extended assignment A-MAP IE and UL extended HARQ feedback for HARQ buffer management using multi-state HARQ feedback.

- References

[1] IEEE 802.16m-09/0010r1a, IEEE 802.16m Amendment Working Document, March, 2009.

- Text proposal for the 802.16m Amendment Working Document

[Proposed text #1]

-------------------------------------------  Text Start  ---------------------------------------------------

[Add the following subclause at the end of 15.3.9.2.2]

# 15.3.9.2.2.1  Extended HARQ feedback control channel

To apply the HARQ buffer management by using tri-state feedback, e.g., ACK, NACK, and DROP, the mapping of the tri-state HARQ feedback to its corresponding feedback sequence are show in Table 695. The orthogonal sequences are linear combination of the sequence in Table 694 by

where Si is the i-th sequence in Table 694 and Ck is the k-th sequence in Table 695.

Table 695––Orthogonal sequences for UL tri-state HARQ feedback channel

| Sequence index | Orthogonal sequence | Tri-state HARQ feedback |
| --- | --- | --- |
| 0 | [-0.433+0.75j -1.299-0.75j 1.299+0.75j -1.299+2.25j] | ACK |
| 1 | [-0.433-0.75j -1.299+0.75j 1.299-0.75j -1.299 - 2.25j] | NACK |
| 2 | [3.4641 0 0 0 ] | DROP |

Two HARQ feedback channels can be jointly encoded on the same resource. The mapping of two tri-state HARQ feedbacks to its corresponding feedback sequence are show in Table 696. The orthogonal sequences are linear combination of the sequence in Table 694 by

where Si is the i-th sequence in Table 694 and Dk is the k-th sequence in Table 696.

Table 696––Orthogonal sequences for UL nine-state HARQ feedback channel

| Sequence index | Orthogonal sequence | Nine-state HARQ feedback |
| --- | --- | --- |
| 0 | [-0.6124+3.4729j 2.9141 + 0.5138j 1.264 + 0.2229j 0.187-1.0607j] | ACK/ACK |
| 1 | [-0.6124-0.2229j -1.264+3.4729j -0.187+0.5138j 2.9141+1.0607j] | ACK/NACK |
| 2 | [-0.6124+1.0607j -1.8371-1.0607j 1.8371+1.0607j -1.8371+3.182j] | ACK/DROP |
| 3 | [-0.6124-0.5138j 0.187-0.2229j -2.9141+3.4729j -1.264-1.0607j] | NACK/ACK |
| 4 | [-0.6124+0.5138j 0.187+0.2229j -2.9141-3.4729j -1.264+1.0607j] | NACK/NACK |
| 5 | [-0.6124-1.0607j -1.8371+1.0607j 1.8371-1.0607j -1.8371-3.182j] | NACK/DROP |
| 6 | [-0.6124+0.2229j -1.264-3.4729j -0.187-0.5138j 2.9141-1.0607j] | DROP/ACK |
| 7 | [-0.6124-3.4729j 2.9141-0.5138j 1.264-0.2229j 0.187+1.0607j] | DROP/NACK |
| 8 | [4.899 0 0 0 ] | DROP/DROP |

-------------------------------------------  Text End   --------------------------------------------------

[Proposed text #2]

-------------------------------------------  Text Start  ---------------------------------------------------

[Add the following subclause after 15.3.6.4.2.2]

# 15.3.6.4.2.3  DL extended assignment A-MAP IE

Table 662 describes the fields in a DL Extended Assignment A-MAP IE used for resource assignment in the DL indicating the AMS to perform buffer management. Multiple CIDs assigned to a Station ID is supported to optimize the resource allocation for a receiver.

Table 662––DL extended assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP_IE { |  |  |
| A-MAP IE Type | 4 | DL Extended Assignment A-MAP IE |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b00: n/a |
| if (MEF == 0b01){ |  | Parameter for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt) 0b0: 1 stream 0b1: 2 stream |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt) 0b00: 1 stream 0b01: 2 stream 0b10: 3 stream 0b11: 4 stream |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt) 0b000: 1 stream 0b001: 2 stream 0b010: 3 stream 0b011: 4 stream 0b100: 5 stream 0b101: 6 stream 0b110: 7 stream 0b111: 8 stream |
| }else if(MEF == 0b10){ |  |  |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt) 0b0: 1 stream 0b1: 2 stream |
| }else |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt) 0b00: 1 stream 0b01: 2 stream 0b10: 3 stream 0b11: 4 stream |
| } |  |  |
| } |  |  |
| Boosting | 3 | 000: Normal (not boosted) 001: +6dB; 010: –6dB 011: +9dB; 100: +3dB 101: –3dB; 110: –9dB 111: –12dB |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL sunframe for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| N_ACID | 1 | Number of channel identifier 0b0: 1 ACID 0b1: 2 ACID |
| if(N_ACID == 0){ |  |  |
| MCS | 4 | 16 modes assumed as baseline |
| Resource Allocation | Variable [TBD] | Variable number of bits – depends on the system bandwidth. Information may include Type of resource unit (DRU/CRU) Location (start/end) Allocation size |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID/CoRe Version | 3 | HARQ subpacket identifier for IR and Constellation Rearrangement versio |
| }else |  | for two channels |
| MCS | 8 | 16 modes assumed as baseline |
| AI_SN | 2 | HARQ identifier sequence number |
| Resource Allocation | Variable [TBD] | Variable number of bits – depends on the system bandwidth. Information may include Type of resource unit (DRU/CRU) Location (start/end) Allocation size |
| ACID | 8 | HARQ channel identifier |
| SPID/CoRe Version | 3 | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | 16 | 16 bit CRC masked by Station ID |
| } |  |  |

-------------------------------------------  Text End   --------------------------------------------------