| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | An Optimized ABS Re-selection Scheme for Initial Ranging Process |  |
| Date Submitted | 2009-4-27 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai, Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) | Voice: +886-2-66000100 Fax: +886-2-66061007 cwchen@iii.org.tw hcy@iii.org.tw |
| Re: | IEEE 802.16m-09/0020. “Call for Contributions on P802.16m Amendment Working Document” Category: AWD-New contribution / Area: Network Entry |  |
| Abstract | This contribution proposes an optimized ABS re-selection during the AMS initial ranging process in the draft P802.16m amendment |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

An Optimized ABS Re-selection Scheme for Initial Ranging Process

Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai 
Institute for Information Industry (III)

Shiann-Tsong Sheu
National Central University (NCU)

Introduction

When an AMS powers on, it will synchronize with an ABS via A-Preamble to obtain necessary information for network entry. The AMS normally selects the ABS with the best signal quality to perform initial network entry procedure. After scanning with ABSs and performing cell selection, an AMS starts the initial ranging process with the selected ABS in order to join the network. In the 16e standard, the AMS shall re-scan and retransmit the RNG-REQ message if it is notified the abort status via RNG-RSP message from the ABS. Although the ABS could indicate the Preamble Indices of the suggested ABS(s) in the RNG-RSP message, the legacy procedure may increase access latency if the ABS does not have enough information to optimize the suggestion.

In order to solve the above problem, the RSSI and the CINR of neighboring ABS(s) could be included in the AAI-RNG-REQ message during the initial ranging process. In this contribution, we propose a TLV to be included in the AAI-RNG-REQ message to optimize the ABS re-selection during the AMS initial ranging process. The AMS shall perform scan to obtain the ABS_ID of ABS(s). The ABS_ID is a 48-bit long field identifying the ABS. The least significant 24 bits of the ABS_ID shall be used as the Operator ID. The AMS may report the information of neighboring ABS(s) who has the same operator ID with the ABS via AAI-RNG-REQ message. Then, if required, the ABS can indicate the suggested ABS(s) for the AMS redo initial ranging.

Text proposal for inclusion in the 802.16m amendment:

----------------------------------------------------Start of the Text---------------------------------------------------------

[Insert the following text in Section 15.2.x:]

15.2.x  AMS network entry

The network entry procedure follows the procedure defined in the Section 6.3.9 unless otherwise specified in this section.

The AMS shall perform scan to obtain the ABS_ID of ABS(s). The ABS_ID is a 48-bit long field identifying the ABS. The least significant 24 bits of the ABS_ID shall be used as the Operator ID. The AMS may report the information of neighboring ABS(s) who has the same operator ID with the ABS via AAI-RNG-REQ message. Then, if required, the ABS can indicate the suggested ABS(s) for the AMS redo initial ranging.

During initial ranging process, AAI-RNG-REQ/RSP shall be transmitted by the AMS and the ABS respectively. The following parameters shall be included in the AAI-RNG-REQ message when the AMS is performing initial network entry. The format of the AAI-RNG-REQ message is shown in Table xxx.

Table xxx  AAI-RNG-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-RNG-REQ_Message_Format(){ | ─ | ─ |
| Management Message Type = TBD | 8 | ─ |
| Reserved | 8 | Shall be set to zero |
| TLV Encoded Information | variable | TLV-specific |
| } | ─ | ─ |

To optimize the ABS re-selection during the AMS initial ranging process, the following TLV parameters shall be included in the AAI-RNG-REQ message. The proposed TLV in Table yyy are specific to the AAI-RNG-REQ message.

Table yyy  AAI-RNG-REQ message encodings

| Name | Type | Length | Value (variable length) |
| --- | --- | --- | --- |
| Neighboring_BS info | TBD | Variable | N_Neighbor_BS (6 bits) Report_metric (2 bits) for(i = 0; i<N_Neighboring_BS; i++){ Neighboring_BS_Index (8 bits) If (Report_metric[Bit 0]==1) Neighboring_BS CINR mean (8 bits) If (Report_metric[Bit 1]==1) Neighboring BS RSSI mean (8 bits) } |

N_Neighboring_BS

This parameter indicates the number of neighboring ABSs which are scanned by the AMS, except the current ABS.

Report_metric

Bitmap indicating metrics on which the corresponding triggers are based:

Bit 0: Neighboring_BS CINR mean

Bit 1: Neighboring_BS RSSI mean

Neighboring_BS_Index

This parameter indicates the neighboring ABS identification.

Neighboring_BS_CINR mean

The parameter indicates the CINR measured by the AMS from the particular ABS. The measurement shall be performed on the subcarriers of the frame preamble that are active in the particular ABS’s segment and averaged over the measurement period.

Neighboring_BS_RSSI mean

The parameter indicates the received signal strength measured by the AMS from the particular ABS. The measurement shall be performed on the subcarriers of the frame preamble and averaged over the measurement period.

The format of the AAI-RNG-RSP message is shown in Table iii.

Table iii  AAI-RNG-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-RNG-RSP_Message_Format(){ | ─ | ─ |
| Management Message Type = TBD | 8 | ─ |
| Reserved | 8 | Shall be set to zero |
| TLV Encoded Information | variable | TLV-specific |
| } | ─ | ─ |

To optimize the ABS re-selection during the AMS initial ranging process, the following TLV parameters shall be included in the AAI-RNG-RSP message. The proposed TLV in Table jjj are specific to the AAI-RNG-REQ message.

Table jjj  AAI-RNG-RSP message encodings

| Name | Type | Length | Value (variable length) |
| --- | --- | --- | --- |
| Ranging Status | TBD | 1 | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 =success |
| A-Preamble Index Override | TBD | Length is defined as Number of Preamble Index * 1 | Preamble Indices of the recommended ABS(s) where the AMS should redo ranging. If this TLV is used, the Ranging Status value shall be set to 2. This TLV shall be used for licensed bands only. |

Ranging Status

This parameter indicates whether UL messages are received within acceptable limits by the ABS.

1 = continue, 2 = abort, 3 = success

Preamble Index Override

This parameter indicates the Preamble Indices of the suggested ABS(s) where the AMS should redo ranging. If the TLV includes two or more Preamble Indices, the first one in the list is the most preferable and the second is the next preferable. When the TLV is used with Downlink frequency override TLV, the MS should redo ranging on the new DL channel identified by the Preamble Indices. If this TLV is used, the Ranging Status value shall be set to 2.

----------------------------------------------------End of the Text---------------------------------------------------------