| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | An Optimized Initial Ranging Process |  |
| Date Submitted | 2009-5-5 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang Tamkang University (TKU) | Voice: +886-2-66000100 Fax: +886-2-66061007 cwchen@iii.org.tw hcy@iii.org.tw |
| Re: | IEEE 802.16m-09/0020. “Call for Contributions on P802.16m Amendment Working Document” Category: AWD-New contribution / Area: Network Entry |  |
| Abstract | This contribution proposes an optimized ABS re-selection during the AMS initial ranging process in the draft P802.16m amendment |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

An Optimized Initial Ranging Process

Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai 
Institute for Information Industry (III)

Shiann-Tsong Sheu
National Central University (NCU)

Whai-En Chen
National Ilan University (NIU)

Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang
Tamkang University (TKU)

Introduction

When an AMS powers on, it will synchronize with an ABS via A-Preamble and will obtain necessary information for network entry. The AMS normally selects the ABS with the best signal quality to perform initial network entry procedure. After scanning for ABSs and performing cell selection, an AMS starts the initial ranging process with the selected ABS in order to join the network. An efficient ranging process could facilitate the AMS performing network entry. In 16e standard, the MS shall re-scan and re-transmit the ranging code if it is notified the abort status via RNG-RSP message from the BS. This procedure may increase access latency.

In current SDD, the ABS could respond to the AMS’s initial ranging code transmission by broadcasting a status indication message in a following predefined DL frame/subframe. We design a RNG-IND A-MAP IE to indicate the decoding results of the initial ranging code and also indicate the bandwidth allocation to a user in order to reduce overhead and decrease access latency. Otherwise, the AAI-RNG-RSP message shall be linked to the corresponding bit of the RNG-IND A-MAP IE to optimized parameter adjustment and ABS re-selection.

Figure 1 provides an example of the initial ranging process, three different cases that an AMS may encountered during initial ranging were depicted. In this contribution, we propose the amendment text of the optimized initial ranging process.

Figure 1  The initial ranging process

Text proposal for inclusion in the 802.16m amendment:

----------------------------------------------------Start of the Text---------------------------------------------------------

[Insert the following text in Section 15.2.x:]

15.2.x  AMS network entry

The network entry procedure follows the procedure defined in the Section 6.3.9 unless otherwise specified in this section.

In order to reduce overhead and decrease access latency, the ABS could respond to the AMS’s initial ranging code transmission via RNG-IND A-MAP IE in a following predefined DL frame/subframe. And if the corresponding bit of the decoding status bitmap is set to 1, the bandwidth allocation to a user is indicated. The following parameters shall be included in the RNG-IND A-MAP IE when the AMS is performing initial network entry. The content of the RNG-IND A-MAP IE is shown in Table xxx.

Table xxx  RNG-IND A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| RNG-IND A-MAP IE(){ | ─ | ─ |
| A-MAP IE Type | 4 | Used to distinguish between other A-MAP IEs. |
| Decoding Status Bitmap | TBD | If the bit value is set to1, this indicates the IR code has been successfully decoded. |
| If (Decoding Status Bitmap[i]=1){ | ─ | ─ |
| N_Allocation_Ranging Code | variable | Used to indicate the number of allocation ranging code in the corresponding bit of bitmap |
| For(i=0; i<N_Allocation_Ranging Code; i++){ | ─ | ─ |
| Ranging Code | 8 | Used to indicate the CDMA code sent by the AMS |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation |
| } | ─ | ─ |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| } |  |  |

Decoding Status Bitmap

If the bit value is set to 1, this indicates the IR code has been successfully decoded.

N_Allocation_Ranging Code

This parameter is used to indicate the number of allocation ranging code in the corresponding bit of bitmap.

Ranging Code

This parameter indicates the CDMA code sent by the AMS.

Resource Allocation

Determines the start, length and number of subframes spanned the allocation.

The format of the AAI-RNG-RSP message is shown in Table yyy.

Table yyy  AAI-RNG-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-RNG-RSP_Message_Format(){ | ─ | ─ |
| Management Message Type = TBD | 8 | ─ |
| Reserved | 8 | Shall be set to zero |
| TLV Encoded Information | variable | TLV-specific |
| } | ─ | ─ |

To optimize the parameter adjustment and ABS re-selection during the AMS initial ranging process, the following TLV parameters shall be included in the AAI-RNG-RSP message. The proposed TLV in Table zzz are specific to the AAI-RNG-REQ message.

Table zzz  AAI-RNG-RSP message encodings

| Name | Type | Length | Value (variable length) |
| --- | --- | --- | --- |
| Ranging Status | TBD | 1 | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort |
| Ranging Code | TBD | 1 | Used to indicate the CDMA code sent by the AMS |
| Adjustment Indicator | TBD | 1 | Used to indicate which information should be adjusted by AMS. 1 = Timing Adjust, 2= Power Level Adjust, 3 = Offset Frequency Adjust |
| Timing Adjust | TBD | 4 | Tx timing offset adjustment. If this TLV is used, the Ranging Status value shall be set to 1. |
| Power Level Adjust | TBD | 1 | Tx power offset adjustment. If this TLV is used, the Ranging Status value shall be set to 1. |
| Offset Frequency Adjust | TBD | 4 | Tx frequency offset adjustment. If this TLV is used, the Ranging Status value shall be set to 1. |
| Downlink Frequency Override | TBD | 4 | When the parameter is used with A-Preamble Index Override, the MS should redo ranging on the new DL channel identified by the Preamble Indices. |
| A-Preamble Index Override | Number of Preamble Index * 8 | Preamble Indices of the recommended ABS(s) where the AMS should redo ranging. | Preamble Indices of the recommended ABS(s) where the AMS should redo ranging. If this TLV is used, the Ranging Status value shall be set to 2. This TLV shall be used for licensed bands only. |

Ranging Status

This parameter indicates whether UL messages are received within acceptable limits by the ABS.

1 = continue, 2 = abort

Ranging Code

This parameter indicates the CDMA code sent by the AMS.

Adjustment Indicator

Used to indicate which information should be adjusted by AMS.

1 = Timing Adjust, 2= Power Level Adjust, 3 = Offset Frequency Adjust

Timing Adjust

Tx timing offset adjustment. The AMS shall advance its transmission time if the value is negative and delay its transmission if the value is positive.

Power Level Adjust

Tx power offset adjustment. This parameter specifies the relative change in transmission power level that AMS is to make in order that transmissions arrive at the ABS at the desired power.

Offset Frequency Adjust

Tx frequency offset adjustment. This parameter specifies the relative change in transmission frequency that the AMS is to make in order to better match the ABS. If the value is less than half of the channel bandwidth, this is fine-frequency adjustment within a channel. Otherwise, this is re-assignment to a different channel. The AMS shall increase its Tx frequency if the value is positive and decrease its Tx frequency if the value is negative.

Downlink Frequency Override

Center frequency, in kHz, of new DL channel where the SS should redo ranging. If this parameter is used, the Ranging Status value shall be set to 2.

A-Preamble Index Override

This parameter indicates the Preamble Indices of the suggested ABS(s) where the AMS should redo ranging. If the parameter includes two or more Preamble Indices, the first one in the list is the most preferable and the second is the next preferable. When the parameter is used with Downlink frequency override, the MS should redo ranging on the new DL channel identified by the Preamble Indices. If this parameter is used, the Ranging Status value shall be set to 2.

----------------------------------------------------End of the Text---------------------------------------------------------