| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame structures for a CP length of 1/4Tb |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Dongguk Lim, Sungho Moon, HanGyu Cho, Jin Sam Kwak LG Electronics. Jaeweon Cho Samsung Electronics Co., Ltd. Sassan Ahmadi, Intel Corp. Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting ITRI Yanfeng Guan, Huiying Fang ZTE | Voice: +82-31-450-xxxx E-mail: dongguk@lge.com, msungho@lge.com, hgcho@lge.com, samji@lge.com jaeweon.cho@samsung.com Sassan.ahmadi@intel.com jefferywu@itri.org.tw guan.yanfeng@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.3 (Frame structure) “Comments on AWD 15.3.3 Frame structure” |  |
| Abstract | The contribution proposes the text of frame structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Frame structures for a CP length of 1/4Tb

Dongguk Lim, Sungho Moon, HanGyu Cho, Jin Sam Kwak

LG Electronics,

Jaeweon Cho

Samsung Electronics Co., Ltd.,

Sassan Ahmadi

Intel Corp.

Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting,

ITRI,

Yanfeng Guan, Huiying Fang

ZTE

# Introduction

The amendement working document (AWD) defines three types of CP lengths, which are 1/8Tb, 1/16Tb, and 1/4Tb. Moreover, there exist detailed OFDMA parameters and frame structures for CP lengths of 1/8Tb and 1/16Tb. However, frame structures for a CP length of 1/4Tb are not defined yet. In this contribution, we propose frame structures for a CP length of 1/4Tb for all the bandwidths of 5/10/20 MHz, 7 MHz, and 8.75 MHz.

# OFDMA parameters for a CP length of 1/4Tb

For CP lengths of 1/8Tb and 1/16Tb, OFDAM parameters for channel bandwidths of 5/10/20 MHz, 7 MHz, and 8.75MHz were already defined in the IEEE amendment working document (AWD). However, OFDAM parameters for a CP length of 1/4Tb are empty in case of the bandwidth of 7 MHz and 8.75 MHz. The derived OFDMA parameters for a CP length of 1/4Tb are specified in Table 1.

Table 1 OFDMA parameters for 1/4 CP

| Nominal Channel Bandwidth (MHz) |  |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor |  |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling Frequency (MHz) |  |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT Size |  |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing (kHz) |  |  | 10.937500 | 7.812500 | 9.765625 | 10.937500 | 10.937500 |
| Useful Symbol Time Tu (µs) |  |  | 91.429 | 128 | 102.4 | 91.429 | 91.429 |
| Cyclic Prefix (CP) Tg=1/4 Tu | Symbol Time Ts (µs) |  | 114.286 | 160 | 128 | 114.286 | 114.286 |
|  | FDD | Number of OFDM symbols per Frame | 43 | 31 | 39 | 43 | 43 |
|  |  | Idle time (µs) | 85.694 | 40 | 8 | 85.694 | 85.694 |
|  | TDD | Number of OFDM symbols per Frame | 42 | 30 | 38 | 42 | 42 |
|  |  | TTG + RTG (µs) | 199.98 | 200 | 136 | 199.98 | 199.98 |

# Proposed frame structures for a CP length of 1/4Tb

In current Amendment, four types of subframes have been defined. But generally, three types of subfraems are considered for all supporting bandwidth, i.e., type-1 with 6 OFDMA symbols, type-2 with 7 OFDMA symbols, and type-3 with 5 OFDMA symbols. Thus, we shall define new frame structures for 1/4 CP length using these three types of subframes. As shown in Table 1, the number of OFDM symbols per 5ms frame is less than other CP lengths. Therefore, a new frame structure shall consist of one less number of subframes than that of other CP lengths in order to have more number of type-1 subframes.

## TDD and FDD frame structures for 5/10/20MHz

Figure 1 shows an example of the proposed TDD and FDD frame structures for 5/10/20MHz. When using the same OFDMA parameters as in Table 1, each 5ms radio frame consists of seven subframes.

A FDD frame shall have six type-1 subframes and one type-2 subframes since 43 OFDMA symbols can exist within a frame. The TDD frame structure has a commonality with the FDD frame structure in the number of subframes. In TDD, one frame only consists of seven type-1 subframes because one OFDMA symbol should be an idle symbol. Therefore, the subframe preceding a DL to UL switching point shall be type-1 subframes instead of one type-2 subframe, and TTG and RTG are 139.988 µs and 60 µs, respectively, which are the same as those in the WiMAX profile.

Figure 1 TDD and FDD frame structures for 5/10/20MHz with a CP 1/4 Tb (DL to UL ratio of 4:3 )

## TDD and FDD frame structures for 8.75MHz

Figure 2 shows an example of the proposed TDD and FDD frame structures for 8.75MHz. When using the same OFDMA parameters as in Table 1, each 5ms radio frame consists of six subframes. A FDD frame shall have three type-1 subframes and three type-2 subframes since 39 OFDMA symbols can exist within a frame. The first subframe should be type-1 subframe since the current superframe header (SFH) should be located in the first subframe. The TDD frame structure has a commonality with the FDD frame structure in the number of subframes. In TDD, one frame consists of four type-1 subframes and three type-2 subframes because one OFDMA symbol should be an idle symbol. Herein, the subframe preceding a DL to UL switching point shall be type-1 subframes instead of one type-2 subframe which is the same as those of 5, 10, 20 MHz bandwidth cases. TTG and RTG are 61.6 µs and 74.4 µs, respectively, which are the same as those in the WiMAX profile.

Figure 2 TDD and FDD frame structure for 8.75MHz with a CP 1/4 Tb (DL to UL ratio of 4:2 )

## TDD and FDD frame structures for 7MHz

Figure 3 shows an example of the proposed TDD and FDD frame structure for 7MHz. When using the same OFDMA parameters as in Table 1, each 5ms radio frame consists of five subframes. A FDD frame shall have four type-1 subframes and one type-2 subframes since 31 OFDMA symbols can exist within a frame. The TDD frame structure has a commonality with the FDD frame structure in the number of subframes. In TDD, one frame consists of five type-1 subframes because one OFDMA symbol should be an idle symbol. Herein, the subframe preceding a DL to UL switching point shall be type-1 subframes instead of one type-2 subframe which is the same as those of 5, 10, 20 MHz bandwidth cases. TTG and RTG are 140 µs and 60 µs, respectively, which are the same as those in the WiMAX profile.

Figure 3 TDD and FDD frame structure for 7MHz with a CP 1/4 Tb (DL to UL ratio of 3:2 )

# Reference

[1] IEEE 802.16m-08/0010r1a, “IEEE 802.16m Amendment Working Document.”

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”

[3] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

# Text Proposal for the 802.16m ammendment

============================== Start of Proposed Text =================================

[Remedy 1: Adopt the following modifications in lines from 58 to 60 on page 17:]

When using the same OFDMA parameters as in Table 647 with the channel bandwidth of 5 MHz, 10 MHz, or 20 MHz, each 5 ms radio frame further consists of eight subframes for G=1/8 and 1/16. With the channel bandwidth of 8.75 and 7 MHz, each 5 ms radio frame further consists of seven and six subframes, respectively for G=1/8 and 1/16. In case of G=1/4, the number of subframes per frame is one less than that of other CP lengths for each bandwidth case.

[Remedy 2: Create a section 15.3.3.x for frame structure for CP = 1/4 Tb, and insert the following text and figure in line 37 on page 26: ]

# Section 15.3.3.x Frame structure for CP = 1/4 Tb

The frame structure for a CP length of 1/4 Tb shall consist of type-1 and type-2 subframes.

For normal channel bandwidth of 5/10/20MHz, a FDD frame shall have six type-1 subframes and one type-2 subframes. A TDD frame shall have seven type-1 subframes.

Figure x-1 illustrates an example of TDD and FDD frame structure with DL/UL ratio = 4:3 for the 5/10/20MHz channel bandwidth with a CP of 1/4 Tb. Assuming OFDMA symbol duration of 114.286μs and a CP length of 1/4Tb ,the length of type-1 and type-2 subframes are 0.6857 ms and 0.80 ms, respectively. TTG and RTG are 139.988 µs and 60 µs, respectively.

Figure x-1 Frame structures for 5, 10, and 20MHz of TDD and FDD modes (G=1/4)

For the coexistence scenarios with other CP lengths in the 5/10/20MHz channel bandwidths, another frame structure may be used.

Figure XX-1 illustrates an example of another TDD and FDD frame structure for the coexistence scenarios with other CP cases in 5, 10 and 20 MHz channel bandwidth. A TDD frame shall have six type-3 subframes and two type-1 subframes, and an FDD subframe shall have five type-3 subframes and three type-1 subframes. In the TDD frame, the first subframe in DL and the first subframe in UL within each frame shall be type-1 subframes. In the FDD frame, the first, fifth, and sixth subframes within each frame shall be type-1 subframes.

Figure. XX-1— Frame structures for 5, 10, and 20MHz when coexisting with other CP lengths

For a channel bandwidth of 7MHz, a FDD frame shall have four type-1 subframes and one type-2 subframes. And a TDD frame shall have five type-1 subframes.

Figure x-2 illustrates an example of TDD and FDD frame structure with DL/UL ratio = 3:2 for the 7MHz channel bandwidth with a CP of 1/4 Tb. Assuming OFDMA symbol duration of 160μs and a CP length of 1/4Tb the length of type-1 and type-2 subframes are 0.960 ms and 1.120 ms, respectively. TTG and RTG are 140 µs and 60 µs, respectively.

Figure x-2 Frame structures for 7MHz TDD and FDD modes (G=1/4)

For a channel bandwidth of 8.75MHz, a FDD frame shall have three type-1 subframes and three type-2 subframes. And a TDD frame shall have four type-1 subframes and two type-2 subframes. In TDD, the subframe preceding a DL to UL switching point shall be type-1 subframes.

Figure x-3 illustrates an example of TDD and FDD frame structure with DL/UL ratio = 4:2 for the 8.75MHz channel bandwidth with a CP of 1/4 Tb. Assuming OFDMA symbol duration of 128μs and a CP length of 1/4Tb the length of type-1 and type-2 subframes are 0.768 ms and 0.896 ms, respectively.

Figure x-3 Frame structures for 8.75MHz TDD and FDD modes (G=1/4)

=============================== End of Text Proposal ===============================