| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Legacy Supports in the Frame Structures for 8.75MHz and 7MHz |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Sungho Moon, Wookbong Lee, Minseok Noh, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1852 E-mail: msungho@lge.com, wbong@lge.com, msnoh@lge.com, samji@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.3 (Frame structure) “Comments on AWD 15.3.3 Frame structure” |  |
| Abstract | The contribution proposes the text of frame structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Legacy Supports in the Frame Structures for 8.75MHz and 7MHz

Sungho Moon, Wookboong Lee, Minseok Noh, Jin Sam Kwak

LG Electronics

# Introduction

This contribution includes the proposed text for the frame structure section in the IEEE 802.16m Amendment. The technical text is inherited from the 802.16m SRD [2], the IEEE 802.16m SDD [3], and the IEEE P802.16 Rev2/D7 [4]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [5]. In this contribution, amendment text including several illustrations is proposed for the section in which 16m members have enough consensus.

# Frame Structure for Legacy Support in 5/10/20 MHz

When the Advanced Air Interface frames support the WirelessMAN-OFDMA for 5, 10, 20MHz channel bandwidths, all subframes in the Advanced Air Interface DL Zone are type-1 subframes. The number of symbols in the WirelessMAN-OFDMA DL Zone is 5+6·(FRAME_OFFSET-1) [1].

Figure 1 shows frame structures for legacy supports in 5/10/20 MHz. It has two major advantages as follows:

- All subframes for AAIF are type-1 subframes.

- Odd number of OFDM symbols in the DL Lzone is defined and it can reduce the number of STC_DL_ZONE_IEs.

# Proposed Frame Structure for Legacy Support in 7 MHz

In the WiMAX Plugfest 5 guideline, the DL to UL symbol ratio employed in the current practical WiMAX systems shall be supported; i.e. DL:UL = 21:12 for 7MHz. When configuring the legacy frame structure, type-1 subframes should be utilized in a frame as many as possible since it is a default subframe type optimized in a PHY design.

Figure 2 shows examples of the TDD frame structure for legacy support in the channel bandwidth of 7 MHz. Since the largest frame offset value K is equal to the number of DL subframes minus two, possible frame offset values are 1 or 2 in case of 7MHz. The first subframe of AAIF should be the type-1 subframe in order to maintain commonality in the SFH design. Moreover, all the subframes of AAIF shall be type-1 subframes. Therefore, the number of symbols in the WirelessMAN-OFDMA DL Zone is 3+6•(FRAME_OFFSET-1).

# Proposed Frame Structure for Legacy Support in 8.75MHz

The DL to UL ratio employed in the current practical WiMAX systems shall be DL:UL = 27:15 for 8.75 MHz. When configuring the legacy frame structure, the best way to divide subframes is to utilize type-1 subframes as many as possible since it is a default subframe type optimized in a PHY design.

Figure 3 shows examples of the TDD frame structure for legacy support in the channel bandwidth of 8.75 MHz. All the DL subframes of AAIF shall be type-1 subframes, and the first type-1 subframe is used for SFH. There exists only one type-4 UL subframe when the uplink of WirelessMAN-OFDMA and AAIF is supported by FDM. However, in case of UL TDM, all the UL subframe can be still type-1 subframes with allocating 9 OFDMA symbols to the WirelessMAN-OFDMA. Since the largest frame offset value K is equal to the number of DL subframes minus two, possible frame offset values are 1, 2, or 3 in case of 8.75 MHz. Therefore, the number of symbols in the WirelessMAN-OFDMA DL Zone is 3+6•(FRAME_OFFSET-1).

Figure 1 Legacy support for a channel bandwidth of 5/10/20 MHz

Figure 2 Proposed legacy support for a channel bandwidth of 7 MHz

Figure 3 Proposed legacy support for a channel bandwidth of 8.75 MHz

# References

[1] IEEE 802.16m-08/0010r1a, “IEEE 802.16m Amendment Working Document.”

[2] IEEE 802.16m-07/002r4, “802.16m System Requirements.”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”

[4] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

# Text proposal for the 802.16m amendment

============================== Start of Proposed Text =================================

[Remedy 1: Insert the following text before "The maximum ..." in line 45 of page 26:]

When the Advanced Air Interface frames support the WirelessMAN-OFDMA for the 8.75 MHz channel bandwidth with 15 UL OFDM symbols and for the 7 MHz channel bandwidth with 12 UL OFDM symbols, all subframes in the Advanced Air Interface DL Zone are type-1 subframe. The number of symbols in the WirelessMAN-OFDMA DL Zone is 3+6•(FRAME_OFFSET – 1).

[Remedy 2: Adopt the following modification in line 2 on page 27:]

In the case of 5, 7, 10, and 20 MHz, all UL subframes are type-1 subframes. In the case of 8.75 MHz with 15 UL OFDM symbols, the [first] UL subframe is type-4 subframe and the [second] UL subframe is type-1 subframe.

[Remedy 3: Adopt the following modification in line 50 on page 27:]

In the case of 5, 7, 10, 20, and 8.75 MHz, all subframes in the Advanced Air Interface UL Zone are type-1 subframes.

============================== End of Proposed Text =================================