| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HO DG AWD Text Proposal on Legacy-Supported HO |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Kelvin Chou HO DG Editor | Kelvin.Chou@mediatek.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD-DG comment / Area: Handover DG “Handover DG AWD Text Proposal on Legacy-Supported HO” |  |
| Abstract | HO DG AWD text proposal on HO with legacy support |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# HO DG AWD Text Proposal on Legacy-Supported HO

Kelvin Chou

HO DG Editor

# Discussion

# Proposed Amendment Text

Add the following text into Section 15.2.5 in IEEE 802.16m-09/0010r1a

---------------------------------------------Text Start ----------------------------------------------------------------

15.2.5 MAC HO Procedure

15.2.5.3 Handover between WirelessMAN-OFDMA Advanced and Reference Systems

15.2.5.3.1 Handover from WirelessMAN-OFDMA Reference to Advanced System

15.2.5.3.1.1 Network Topology Acquisition

15.2.5.3.1.1.1 Network Topology Advertisement

A YBS shall broadcast the system information of the LZone of its neighboring ABS using MOB_NBR-ADV message. This system information is used to facilitate AMS and YMS synchronization with the LZone of neighboring ABS without the need to monitor transmission from the neighboring ABS for DCD/UCD broadcasts.

The support of WirelessMAN-OFDMA advanced system in the neighbor BS is indicated in the MAC version TLV in the MOB_NBR-ADV message transmitted in either the YBS or the LZone of the ABS.

An ABS uses one reserved bit in FCH to indicate the presence of its MZone.

The MAC version may be used for AMS to distinguish between WirelessMAN-OFDMA Reference System/WirelessMAN-OFDMA Advanced co-existing System. A MAC version of 9 indicates an ABS of WirelessMAN-OFDMA Reference/Advanced co-existing system.

The AMS may acquire full system information by scanning of target WirelessMAN-OFDMA Advanced only System.

15.2.5.3.1.1.2 AMS Scanning

The AMS shall follow the same scanning procedure as defined in section 6.3.21.1.2.

In addition, an AMS may use the scanning interval to perform a scanning for the MZone of a neighboring WirelessMAN-OFDMA Advanced/WirelessMAN OFDMA Reference co-existence System.

15.2.5.3.1.2 Handover Procedure

An AMS performs handover from a YBS to an ABS either by using zone switching based handover process or direct handover process. The detailed procedures for zone switch based handover and direct handover are described in 15.2.5.3.1.2.1 and 15.2.5.3.1.2.2.2 respectively. The zone switching based handover is applicable to the ABS supporting WirelessMAN-OFDMA Reference System/WirelessMAN-OFDMA Advanced co-existing System. The direct handover based handover is applicable to the ABS which only supports WirelessMAN-OFDMA Advanced System. An ABS may also decide to keep an AMS in the LZone of a WirelessMAN-OFDMA Reference System/WirelessMAN-OFDMA Advanced co-existing System.

15.2.5.3.1.2.1 Zone Switch based Handover Procedure

The zone switch based HO begins with a decision for an AMS to HO from the serving YBS to the LZone of a target ABS. The HO decision, initiation and cancellation follow the same procedures as defined in section 6.3.21.2. Zone Switch is initiated either by AMS or ABS while the final decision shall be made by ABS. In case of an AMS initiated zone switch, the AMS transmits an RNG-REQ message with the ‘Ranging purpose indicator’ bit set to 4, which implies the zone switch request. Upon reception of such RNG-REQ message, the ABS sends a RNG-RSP message including the zone switch TLV. Before zone switch is initiated, the AMS may inform the target ABS that whether or not it already acquired SFH of the MZone in the target ABS together with the change count of the SFH it acquired.

The AMS performs network reentry in the LZone of the target ABS following the same procedures as defined in section 6.3.21.2.7. In addition, upon knowing the AMS capability of supporting WirelessMAN-OFDMA Advanced System based on the MAC version obtained either from the RNG-REQ sent from AMS in the LZone or from the serving ABS over the backbone, the ABS may direct the AMS to switch from LZone to MZone during or after AMS network re-entry to the LZone.

If ABS knows that MS has acquired the MZone system information (e.g. SFH) and decides that the zone switch operation is needed and the network re-entry procedure at LZone requires more subsequent steps after the ranging process and the delay due to network reentry at MZone is less than the subsequent network reentry procedures at LZone, the ABS directs the AMS to stop the LZone network re-entry and switch from LZone to MZone immediately after receiving the RNG-RSP message of LZone ranging process containing Zone Switch TLV, as shown in Figure x1.

Figure x1 - Handover procedure from YBS to ABS: the T-ABS instructs AMS to switch zone through RNG-RSP with Zone switch TLV

If the ABS decides to switch the AMS to MZone after AMS finishes the network reentry in the LZone, it sends an unsolicited RNG-RSP with Zone Switch TLV in LZone, as shown in Figure x2.

Figure x2 – Handover procedure from YBS to ABS: the T-ABS instructs AMS to switch zone after completion of network reentry at LZone

The Zone Switch TLV shall include the following:

{TBD}

The Zone Switch TLV may also include the following:

- UL grant for the AMS to send AAI_RNG-REQ message in Mzone

- STID for being used in MZone

After receiving zone switch command through RNG-RSP in LZone, the AMS performs network reentry in MZone. The AMS maintains its normal operation in LZone (e.g., exchanging user data with the ABS in LZone) while performing network re-entry in MZone if data path in LZone has been established before the start of zone switch operation.

If synchronization with the MZone has not been accomplished, the AMS starts network reentry in MZone by performing DL synchronization with the MZone using the system information provided in the Zone Switch TLV through RNG-RSP message sent by the ABS in LZone. If the AMS hasn’t acquired SFH of the MZone, the AMS shall acquire the system information of the MZone by listening to the ABS’s SFH.

Then the AMS shall start the ranging process in MZone by sending AAI_RNG-REQ message with Ranging Purpose Indication being “Zone Switch”. The ABS then responds with the AAI_RNG-RSP message.

If UL grant is provided in the MZone at Zone_Switch_Action_time, the AMS uses the grant to send AAI_RNG-REQ message with Ranging Purpose Indication set to “Zone Switch”. Otherwise, the AMS shall request UL bandwidth to send the AAI_RNG-REQ by using the pre-assigned STID provided while in LZone. Upon reception of such BR, the ABS provides a UL grant for AMS to send AAI_RNG-REQ message. After receiving the AAI_RNG-REQ the target ABS responds with the AAI_RNG-RSP message.

The AMS shall also perform capability negotiation during network reentry in MZone through the exchange of AAI_REG-REQ/RSP message. AMS context mapping from LZone to MZone is performed by the ABS per section 15.2.x.4.2.3.

15.2.5.3.1.2.2 Direct Handover Procedure

Direct HO procedure for AMS from YBS to WirelessMAN-OFDMA Advanced System only ABS is FFS.

15.2.5.3.1.3 Context Mapping

With zone switch based handover, the context management process from serving YBS to the LZone of target ABS follows section 6.3.21.2.8.1. The following section describes the context mapping from the LZone to the MZone of the target ABS during network reentry procedure in the MZone in the case of zone switch based handover.

15.2.5.3.1.3.1 MAC Identifiers

The FIDs for the management connections are set to xxx automatically. The FIDs for the transport connections are sequentially derived starting from yyy for all of the transport CIDs used in LZone. The AMS autonomously updates its Flow IDs in the ascending order from the first transport Connection ID.

15.2.5.3.1.4 Zone switch from MZone to Lzone

The ABS indicates zone switch of AMSs that currently operate in the MZone for several reasons, such as load balancing purposes. The HO-Command message is used to trigger the zone switch from MZone to LZone which is specified in TBD. Zone switch from LZone to MZone follows the procedure per section 15.2.5.3.1.2. During the request or command of the zone switch to LZone, the AMS is provided with LZone information in prior, such as CID, FID, security parameters or capability information in the MZone via the AAI-HO-CMD message.

15.2.5.3.2 Handover from Advanced WirelessMAN-OFDMA System to WirelessMAN-OFDMA Reference System

Handover of an AMS/YMS from LZone of an ABS to a YBS follows the same HO procedure defined in section 6.3.21.2. The following section only defines the handover procedure for an AMS from MZone of the serving ABS to a target YBS.

15.2.5.3.2.1 Network Topology Acquisition

15.2.5.3.2.1.1 Network Topology Advertisement

In the WirelessMAN-OFDMA Reference System/WirelessMAN-OFDMA Advanced co-existing System, an ABS shall broadcast the system information of its neighboring YBS in its LZone using MOB_NBR-ADV message. An AMS always obtains the neighbor YBS information from the MOB_NBR-ADV message transmitted in LZone. A reference pointer indicating the time offset of the MOB_NBR_ADV message transmission in LZone is provided.

15.2.5.3.2.1.2 AMS Scanning

The scanning procedure for an YMS/AMS served in LZone of an ABS follow the procedure as defined in section 6.3.21.1.2. The scanning procedure for an AMS served in MZone of an ABS follows the procedure defined in section 15.2.5.1.2.

15.2.5.3.2.2 Handover Procedure

The handover procedure for YMS/AMS served in LZone of an ABS shall follow the procedure defined in section 6.3.21.2. This section specifies the handover process for an AMS served in MZone of the serving ABS to a target YBS.

An AMS uses information acquired from an AAI_NBR-ADV message for cell reselection. The serving ABS may schedule scanning intervals for AMS to conduct cell reselection activity. The cell reselection procedure follows the same procedure defined in section 6.3.21.2.1.

The AMS or the ABS initiates and executes the handover using AAI_HO-REQ or AAI_HO-CMD per section 15.2.5 (intra-16m HO), if the selected target BS is a YBS. The ABS may allocate Basic CID to the AMS to be used in target YBS through the AAI-HO-CMD message. Based on the Basis CID, the AMS can derive its primary management CID and transport CIDs autonomously in the target YBS as defined in section 6.3.21.2. If the AMS information is required to be transferred to the target BS for handover optimization, the serving ABS shall map the AMS context to the format in WirelessMAN-OFDMA Reference System per section 15.2.5.3.2.2.4, and provide it to the target YBS over the backbone. In addition, the serving ABS may indicate the time of the fast ranging opportunity negotiated with the potential target YBSs in the AAI_HO-CMD message. The AMS and target YBS use fast ranging opportunity as defined in section 6.3.21.2.4. Handover cancellation procedure is performed per section 15.2.5.2.6.

The AMS follows the same network reentry procedure to the target YBS as defined in section 6.3.21.2.7.

15.2.5.3.2.3 Context Mapping

15.2.5.3.2.3.1 MAC Identifiers

The management connections with Flow IDs xx (TBD) are mapped to Basic CID and Primary Management CID respectively. The Basic CID is allocated to the AMS by the target YBS and provide to the AMS via the serving ABS using AAI_HO-CMD message. The AMS derives the Primary Management CID based on the procedure defined in section 6.3.21.2. The connection with Flow ID yy is mapped to the first transport connection. The AMS derives the first transport CID based on the procedure defined in section 6.3.21.2, and it autonomously updates its remaining transport CIDs in the ascending order from Flow ID 2. The Station Identifier is released after the AMS handover to the target YBS.

-----------------------------------------------------Text End ---------------------------------------------------------