| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD text specifying the interpretations of some values of the A-MAP “IE Type” field |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Sudhir Ramakrishna Samsung Electronics | sudhir.r@samsung.com |
| Re: | “802.16m amendment working document”: Category – AWD Comments Area – Chapter 15.3.6 (DL Control) |  |
| Abstract | This contribution proposes AWD text specifying the interpretation of some values of the A-MAP “IE Type” field |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD text specifying the interpretations of some values of the A-MAP “IE Type” field

Sudhir Ramakrishna

Samsung Electronics

- Introduction

This contribution proposes text specifying some values and corresponding interpretations of the “A-MAP IE Type” field in Sections 15.3.6.4.2.2 (“DL Basic Assignment A-MAP IE”) and 15.3.6.4.2.3 (“UL Basic Assignment A-MAP IE”) in [1].

- Proposed Text to be added to Section 15.3.6.4.2.2 (“DL Basic Assignment A-MAP IE”)

The following text is proposed to be added on Line 37 on Page 58 of [1], following the existing sentence on Line 37.

------------------------------------------------------ Begin proposed text -----------------------------------

The values 0x0 and 0x1 of the A-MAP IE Type shall be interpreted as defining an allocation of DRU resources and sub-band-based CRU resources, respectively.

------------------------------------------------------ End proposed text -------------------------------------

- Proposed Text to be added to Section 15.3.6.4.2.3 (“UL Basic Assignment A-MAP IE”)

The following text is proposed to be added after the sentence in the “Description/Notes” column corresponding to the “A-MAP IE Type” row on Line 10 of Page 59 of [1].

------------------------------------------------------ Begin proposed text -----------------------------------

The values 0x0 and 0x1 of the A-MAP IE Type shall be interpreted as defining an allocation of DRU resources and sub-band-based CRU resources, respectively.

------------------------------------------------------ End proposed text ------------------------------------

- References

[1] 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document”, March 31, 2009.