| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of DL A-MAP IE for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jerry Pi, Kaushik Josiam, Jay Tsai, Sudhir Ramakrishna Samsung Electronics Co., Ltd. | +1-972-761-7944 zpi@sta.samsung.com |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes the text of a DL A-MAP IE for indicating unallocated RUs to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of DL A-MAP IE for the IEEE 802.16m Amendment

Jerry Pi, Kaushik Josiam, Jay Tsai, Sudhir Ramakrishna

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of DL A-MAP IE for indicating unallocated RUs to be included in the 802.16m amendment ([1]). The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([2]), it is compliant to the 802.16m SRD ([3]) and the 802.16m SDD ([4]), and it follows the style and format guidelines in ([5]).

- References

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[3] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[4] IEEE 802.16m-08/003r6, “IEEE 802.16m System Description Document”

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

Add a new section 15.3.6.4.2.9 on page 67, line 59 as follows:

15.3.6.4.2.9 Unallocated Resource Indication A-MAP IE

The Unallocated Resource Indication A-MAP IE (URI A-MAP IE) indicates the LRUs that are not allocated to any AMS in the cell. An AMS can use the unallocated LRUs for interference estimation. Preferably, an ABS should transmit the URI A-MAP IE using a common Station ID such that all AMS in the cell can receive the information.

Table 1. DL Unallocated Resource Indication A-MAP IE

| Syntax | Size (bit) | Description |
| --- | --- | --- |
| A-MAP IE Type | 4 | A-MAP IE type distinguish the URI A-MAP IE from other A-MAP IEs |
| Resource Allocation | TBD | Determine the LRUs unallocated to any AMS |
| Reserved | TBD | Padding to align the DL URI A-MAP IE to the same length as other A-MAP IEs |
| MCRC | 16 | 16-bit CRC masked by Station ID |

-------------------------------  Text End  ---------------------------------------------------