| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the Draft P802.16m Amendment - SFBCH Contents |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Eldad Zeira, Ron Murias All authors are affiliated with InterDigital Communications LLC | E-mail: eldad.zeira@interdigital.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Contributions on AWD in 802.16m-09/0020 |  |
| Abstract | Text regarding details of SFBCH contents is proposed |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the Draft P802.16m Amendment - SFBCH Contents

Eldad Zeira, Ron Murias

InterDigital Communications LLC

Introduction

Current section on the contents of SFBCH does not provide any details on how is the information to be transmitted. In [4] we have introduced a specific scheme to do so based on Haar transform. The scheme was adapted to the AWD draft and proposed in [5, 7].

In this contribution we re-present briefly the concept, its benefits and propose text to AWD.

Discussion of the proposal

Haar transform is a lossless linear transform of an input vector into a series of progressively insignificant coefficients which are sums and differences of its members. Coefficients can then be quantized with progressively fewer bits resulting in fewer overall bit count than other techniques. It can be defined as a matrix multiply of the input vector, implemented as shifts and sums. Alternatively it could be generated with an iterative procedure.

It should be noted that the number of coefficients isn't immediately related to any number of “best-M”. Instead, more coefficients provide better granularity of information for all the CQI values.

Applied to the full band, the compressed vector contains information regarding:

- CQI values of all subbands – not just selected “best M”.

- Implicitly, the “best M” selection

- Average value of the whole band – “wideband CQI”

In [6] the MIMO DG has outlined the required CQI feedback for the various MIMO modes. It is clear that for many of the modes both wideband CQI and subband CQI selection and values are needed.

The bitcount savings of the proposal stem therefore from:

- No need to explicitly send the best-M selection (ceil(log2 (nchoosek(N,M)))

- Reduce or eliminate the need to schedule and send wideband CQI through PFBCH

- Lower order coefficients in the Haar transform output can be quantized with very few bits

There are sdditional advantages to system performance (not evalueated in [4]). The first stems from the fact that the scheduler has access to all subband CQI, not just selected M. Thus the scheduler knows which subbands to choose if the selected “best-M” are preferably used for another AMS and can make better scheduling decisions. Additional benefits stem from the fact that ABS scheduler has access to progressively refined information with each instantiation of SFBCH, but does not need to wait for all of it.

References

- IEEE 802.16m Amendment Working Document, IEEE 802.16m-08/050

- UL PHY control DG harmonized text IEEE C802.16m-09/0386

- UL PHY control DG contribution text IEEE C802.16m-09/0387

- Efficient CQI Feedback for 802.16m MIMO Transmission, Eldad Zeira et. al., InterDigital,

C80216m-08_1103r1.pdf

- Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control - SFBCH Contents - C80216m-09_0685 Eldad Zeira, InterDigital

- 802.16m SDD IEEE 802.16m-08/003r7

- MIMO DG output IEEE C802.16m-09/0513

- InterDigital proposal to UL control DG: C80216m-UL_Control_DG_AWD_v6_InterDigital

Proposed text

# Secondary Fast Feedback Control Channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive.

15.3.9.3.1.2.1		Measurement Reporting

The CQI information is transmitted during the CQI reporting interval. A CQI reporting interval consists of one or more SFBCH instantiations.

15.3.9.3.1.2.2		Measurement Configuration

CQI feedback information is measured per subband. The number N_cqi_fb and location of the CQI subbands is signaled from the ABS.

15.3.9.3.1.2.2.1		CQI message generation

The N_cqi_fb CQI values are compressed as follows. The ordered vector of measurements C is zero-padded to the nearest power of two vector C’ and then compressed. The compression is defined as a multiplication with matrix W8, W16 or W32 depending on its size. The details of the zero padding are FFS.

Cc=C'Wsized to length

W8=181120112011−2011−20 400 0−400 0040 00−40 0

W16=1161     1     2     0     4     0     0     0     8     0     0     0     0     0     0     01     1     2     0     4     0     0     0    -8     0     0     0     0     0     0     01     1     2     0    -4     0     0     0     0     8     0     0     0     0     0     01     1     2     0    -4     0     0     0     0    -8     0     0     0     0     0     01     1    -2     0     0     4     0     0     0     0     8     0     0     0     0     01     1    -2     0     0     4     0     0     0     0    -8     0     0     0     0     01     1    -2     0     0    -4     0     0     0     0     0     8     0     0     0     01     1    -2     0     0    -4     0     0     0     0     0    -8     0     0     0     01    -1     0     2     0     0     4     0     0     0     0     0     8     0     0     01    -1     0     2     0     0     4     0     0     0     0     0    -8     0     0     01    -1     0     2     0     0    -4     0     0     0     0     0     0     8     0     01    -1     0     2     0     0    -4     0     0     0     0     0     0    -8     0     01    -1     0    -2     0     0     0     4     0     0     0     0     0     0     8     01    -1     0    -2     0     0     0     4     0     0     0     0     0     0    -8     01    -1     0    -2     0     0     0    -4     0     0     0     0     0     0     0     8

Editorial Note: W32 is broken into 4 parts to facilitate editing only

W32=132W32−aW32−bW32−cW32−d

Where the parts are given by:

W32−a=1120400080000000112040008000000011204000−8000000011204000−800000001120−4000080000001120−4000080000001120−40000−80000001120−40000−800000011−2004000080000011−2004000080000011−20040000−80000011−20040000−80000011−200−4000008000011−200−4000008000011−200−400000−8000011−200−400000−80000

W32−b=16000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−1600000000

W32−c=1−1020040000080001−1020040000080001−10200400000−80001−10200400000−80001−10200−40000008001−10200−40000008001−10200−4000000−8001−10200−4000000−8001−10−20004000000801−10−20004000000801−10−20004000000−801−10−20004000000−801−10−2000−4000000081−10−2000−4000000081−10−2000−40000000−81−10−2000−40000000−8

W32−d=0000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16000000000000000016000000000000000−16

The coefficients of Cc are progressively quantized such that more bits are used for the first coefficients than for the latter. The number of coefficients transmitted and their exact quantization is FFS.

The quantized coefficients are transmitted over the SFBCH instantiations in the reporting interval.