| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Constellation Rearrangement for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Wataru Matsumoto, Toshiyuki Kuze, Ramesh Annavajjala, Philip V. Orlik, Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric | {annavajjala, porlik,tao,jzhang}@merl.com Matsumoto.Wataru@aj.MitsubishiElectric.co.jp Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | IEEE 802.16m AWD |  |
| Abstract | This contribution proposes constellation rearrangement for HARQ with QPSK in the current 802.16m Amendment Document |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Constellation Rearrangement for the IEEE 802.16m Amendment

Wataru Matsumoto, Toshiyuki Kuze, Ramesh Annavajjala, Phil Orlik, Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric

# Introduction

In this contribution we propose constellation rearrangement rule for HARQ with QPSK modulation to be included in the 802.16m amendment document ([1]). The proposed text is developed in such a way that it can be readily combined with the IEEE P802.16 Rev2/D8 ([2]). It is compliant to the 802.16m SRD ([3]) and the 802.16m SDD ([4]), and it follows the style and format guidelines in ([5]).

.

# References

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[3] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[4] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Text Proposal for Inclusion in the IEEE 802.16m Amendment Draft

Insert the following text in red on page. 135, on line 37, at the end of Section 15.3.12.2.1.1, of [3] as follows:

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

15.3.12.2.1.1 Constellation rearrangement

With HARQ and rank-2 transmission, the following constellation rearrangement rule shall be applied with QPSK modulation. In Table 705, b0, b1, b2 and b3 are bits taking values from {0,1} and 1-bj denotes negation of bit bj.

Table 705: Constellation rearrangement rule for QPSK modulation with rank-2

| CoRe Version | Antenna 1 | Antenna 2 |
| --- | --- | --- |
| 0 | b0 b1 | b2 b3 |
| 1 | 1-b2 b3 | b0 1-b1 |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]