| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multi-BS MIMO text proposal for AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Philip Orlik, Zhifeng (Jeff) Tao, Ramesh Annavajjala, Jinyun Zhang Mitsubishi Electric Research Lab Toshiyuki Kuze Mitsubishi Electric Corp | Voice: 617-621-7557 Fax: 617-621-7550 tao@merl.com Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “15 Multi-BS MIMO” |  |
| Abstract | This contribution proposes AWD text for Multi-BS MIMO Transmissions to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multi-BS MIMO text proposal for AWD

Philip Orlik, Zhifeng (Jeff) Tao, Ramesh Annavajjala, Jinyun Zhang

Mitsubishi Electric Research Lab

Toshiyuki Kuze,

Mitsubishi Electric Corp

# Introduction

This contribution proposes the text of Multi-BS MIMO transmission to be included in the IEEE 802.16m amendment [1]. The technical text is inherited from the IEEE 802.16m SRD [2], SDD [3], and IEEE P802.16 Rev2/D9 [4].

# Problem Statement

The current AWD text contains no technical content on Multi-BS MIMO.  Section 15.3.11 Multi-BS MIMO in [5] is empty

# Proposed Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the subclause 15.3.11 as follows]

15.3.11 Multi-BS MIMO

Multi-BS MIMO techniques are supported for improving sector throughput and cell-edge throughput through multi-BS collaborative precoding, network coordinated beamforming, or inter-cell interference nulling. Both open-loop and closed-loop multi-BS MIMO techniques can be considered. For closed-loop multi-BS MIMO,

CSI feedback via codebook based feedback or sounding channel will be used. The feedback information may be shared by neighboring base stations via network interface. Mode adaptation between single-BS MIMO and multi-BS MIMO is utilized.

15.3.11.x Multi-BS collaborative Precoding

To assist the ABSs in determining the best collaborative precoding matrix to apply the serving ABS shall request a UL sounding signal from the MS as specified in 15.3.7.2.6.7.1.

Cooperating ABSs that are also in the diversity set of the MS shall be informed of the resource allocation of the requested UL sounding via signaling on the backbone.  Cooperating ABSs may choose to received and estimate their channels to the MS during the scheduled UL sounding interval.

For subsequent transmissions the cooperating ABSs shall exchange the estimated DL responses obtained from the UL sounding.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# Reference

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r1a, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – Advanced Air Interface (working document)”

[6] IEEE C802.16m-08/817, “Base Station Cooperation”