| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Transmission Block size for ARQ in 16m |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Stavros Tzavidas Hua Xu Motorola | stavros.tzavidas@motorola.com hua.xu@motorola.com |
| Re: | Category: AWD-DG Comment Area: ARQ DG |  |
| Abstract | This contribution proposes remedies for issues related to ARQ operations in 16m |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Transmission Block size for ARQ in 16m

Stavros Tzavidas, Hua Xu

Motorola

# Introduction

The output of the ARQ drafting group, as shown in contribution IEEE C802.16m-ARQ_DG-09/004 does not specify how the size of the ARQ blocks is decided during initial transmission. Furthermore, the description of how the size of ARQ blocks that are being retransmitted need to be further developed.

In this contribution we propose changes to the text to address both of these issues.

# Text Proposal

Modify section 15.2.x.1 in contribution IEEE C802.16m-ARQ_DG-09/004 (ARQ DG output) as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

15.2.x.1 ARQ block usage

15.2.x.1.1 Initial transmission

One ARQ block is constructed using one or more MAC SDUs and/or fragments thereof from a given flow.  The ARQ block may vary in size and can be any number of bytes.  FPEH or MEH may be used to indicate the fragmentation and/or packing information.

MAC PDUs shall be of the form illustrated in Figure XXX.  Each PDU shall begin with a MAC header. The header may be followed by zero or more extended headers and Payload of the MAC PDU.  If payload is present, the payload can be multiplex of ARQ blocks from different flows.  No MAC PDU CRC will be used.

Figure XXX   MAC PDU Format

15.2.x.1.2 Retransmission

When an ARQ block transmission fails in the initial transmission, a retransmission is scheduled with or without re-arrangement.

In case of ARQ block retransmission without rearrangement, the MAC PDU shall contain the same ARQ block and corresponding fragmentation and packing information, which was used in the initial transmission.

In case of ARQ block retransmission with rearrangement, single ARQ block may be fragmented into ARQ sub-blocks. A MPDU payload should be constructed from one or more ARQ sub-blocks. ARQ sub-blocks are sequentially numbered using ARQ block SUB_SN. The size of ARQ sub-block is variable size. defined by ARQ_SUB_BLOCK_SIZE (see 15.2.x.3.3), which is fixed in size. ARQ sub-block is maintained during retransmission.

Due to this sub-fragmentation, the original ARQ block number (BSN) can not be used for the sub-block re-assembly.  At the receiver, in order to reassemble the sub-blocks back to the original ARQ block, a sub-sequence numbering scheme is needed.  For instance, if the MSN = X fails after the HARQ process, and the MAC data block X is sub-fragmented into Y sub-blocks, the receiver needs to know which block is the sub-block of X and how many sub-blocks in total.  Additionally, since no in sequence delivery capability exists at lower layer, HARQ, a sub-sequence index (SSI) for each sub-block needs to be sent to the receiver along with the sub-block.

The first SSI value equals the total number of sub-blocks.  Subsequent SSI values are incremented by 1.  For example, for total of Y sub-blocks, the SSIs are Y, Y+1, …, 2Y-1.  By doing this, it avoids the need to indicate in each sub-block a total number of sub-blocks since the smallest SSI implicitly indicates the total number of sub-blocks.  The receiver validates that all sub-blocks by comparing the total number of sub-blocks received Z with the smallest SSI received W.  If W = Z, then all of the sub-blocks of X are received, otherwise, not.  An example of this sub-sequencing index scheme is illustrated in Table YYY.

Table YYY  ARQ Retransmission Sub-Sequencing Index Scheme

Fragmentation and packing information corresponding to retransmitted ARQ sub-blocks shall be included in RFPEH (Rearrangement Fragmentation and Packing Extended Header).

-------------------------------------------------- End of the text proposal --------------------------------------------------