| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Polling for ARQ feedback in 16m |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Stavros Tzavidas Hua Xu Motorola | stavros.tzavidas@motorola.com hua.xu@motorola.com |
| Re: | Category: AWD-DG Comment Area: ARQ DG |  |
| Abstract | This contribution proposes remedies for issues related to ARQ operations in 16m |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Polling for ARQ feedback in 16m

Stavros Tzavidas, Hua Xu

Motorola

# Introduction

The output of the ARQ drafting group, as shown in contribution IEEE C802.16m-ARQ_DG-09/004 includes vague language which allows the ARQ transmitter in general to poll the ARQ receiver to send an ARQ feedback message.

However, this function should only be allowed in the downlink. Polling in the uplink results in resource waste since the AMS will have to request bandwidth to send the polling message thus creating additional and unnecessary load to the random access (ranging) channel. It is far simpler and sufficient to use a timer based mechanism in for uplink connections (feedback sent in the downlink).

# Text Proposal

Modify section 15.2.x.2.2 in contribution IEEE C802.16m-ARQ_DG-09/004 (ARQ DG output) as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

15.2.x.2.2 ARQ feedback polling request

In downlink, the tTransmitter may send ARQ feedback polling request to the receiver, to update the reception status of the transmitted ARQ blocks. The ARQ polling request is sent using a FPEH or MEH.

In downlink, an ABS may assign unsolicited bandwidth using extended header or MAC management message for the AMS to send the ARQ feedback in the same MPDU. The ABS indicates that the purpose of the unsolicited bandwidth is for sending ARQ feedback.

15.2.x.2.3 ARQ feedback triggering conditions

Receiver sends an ARQ feedback when an ARQ feedback polling request is received from the transmitter or when an ARQ block has been missing for a predetermined period.

Two ARQ feedback mechanisms are available, polling based and timer based.  When the transmitter resides at the ABS, polling is recommended for ARQ window flow control.  The transmitter (at ABS) will initiate a polling message based on the status of its ARQ window.  The polling message requests the receiver (at AMS) to send an ARQ feedback payload, which can be either cumulative or selective.  An UL bandwidth allocation shall be placed along with the polling request for the feedback to be sent via UL.  When the transmitter resides at the AMS, a periodic polling timer shall be negotiated and established between the transmitter and receiver.  The receiver (at ABS) shall send feedback payload (cumulative or selective) to the transmitter (at AMS) to prevent ARQ transmission window stall.  In case a temporary polling is needed from the transmitter at AMS, the AMS can send general bandwidth request and use the granted bandwidth to send polling message.

-------------------------------------------------- End of the text proposal --------------------------------------------------