| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for 8Tx Codebook |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Bin-Chul Ihm and Wookbong Lee LG Electronics. | bcihm@lge.com, wbong@lge.com |
| Re: | IEEE 802.16m-09/0020, Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) |  |
| Abstract | This contribution provides the proposed text for DL 8Tx codebook. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for 8Tx Codebook

Bin-Chul Ihm and Wookbong Lee

LG Electronics

- Introduction

In the current AWD [3], we have codebooks for 2Tx, 4Tx and 8Tx CL-MIMO schemes. Usually, the required codebook size per rank is increasing as the number of transmit antenna becomes larger as in example that 2Tx rank 1 codebook of size 3bits is adopted while 4Tx rank 1 codebook of size 6bits is specified. However, we have defined 8Tx codebook per rank of size only 4bits, and this would not be enough for such large dimension of transmit antenna.

To enjoy the gain from 8Tx CL-MIMO, this contribution provides 8Tx codebook of larger size and its simple configuration method reusing 4Tx codebook.

- 8Tx CL-MIMO

To configure 8Tx codebook of larger size than that of the current AWD, it is the simplest way to reuse the 4Tx codebook of size 6bits. The configuration of 8Tx codebook of 8bits for rank 1 and 2 is derived from the subset of 4Tx codebook as follows:

The proposed codebook holds the required properties such as unitary matrices and nested structure. Codebook of 6bits can be constructed in an identical way as in codebook of 8bits. For rank 1 through rank 2, the codebook of 6bits is configured by using subset of 4Tx codebook as follows. These codebook entries are appended to the current 8Tx codebook of 4bits in [3]:

Fig. 1 shows the performance gain from the proposed codebook over the current AWD codebook in uncorrelated case. In this simulation, 8Tx-2Rx CL-MIMO based on codebook under Ped-B 3km/h was evaluated. Periods of PMI feedback, CQI feedback, Band selection and Rank adaptation are 5, 5, 40 and 40ms respectively. Feedback delay is 15ms (=3 frames). Given the results, it is preferable for codebook size to be at least 8bits in order to achieve comparable precoding gain as SVD based approach.

Fig. 1 Link throughput gain of the proposed codebook over AWD codebook in uncorrelated case

Fig. 2 presents the throughput gain from codebook of 8bits and 6bits over AWD codebook in correlated environment. In this case, the codebook of 6bits provides better performance gain rather than the codebook of 8bits.

Fig. 2 Link throughput gain of the proposed codebook over AWD codebook in correlated case (Tx corr.=0.5)

The throughput of 6bit codebook becomes larger as the feedback delay decreases as 5ms (=1 frame) as shown in Fig. 3. The proposed codebook of 6bit gives additional gain around 6~7% in average and maximum 14% at 15dB.

Fig. 3 Link throughput gain of the proposed codebook over AWD codebook in uncorrelated case.

To show the overall impact of the proposed codebook in system level, table 1 provides the SLS results based on the following assumptions.

- BW: 10MHz (TDD)

- ISD: 1.5km

- Fading Channel: Ped-B 3km/h

- Feedback Period: (CQI, PMI 10ms), (Rank, Subband Selection 20ms)

- Feedback Delay: 5ms

| Code book size | Antenna spacing 4.0λ |  |
| --- | --- | --- |
|  | Relative Average Spectral Efficiency | Relative Cell edge user throughput |
| 4 bits | 100 | 100 |
| 6 bits | 104.77 | 105.77 |
|  | Antenna spacing 0.5λ |  |
|  | Relative Average Spectral Efficiency | Relative Cell edge user throughput |
| 4 bits | 100 | 100 |
| 6 bits | 99.35 | 99.1 |

Table 1. Throughput comparison through SLS

Even though 8Tx codebook of 8bits gives the significant gain in terms of throughput especially in low correlated case, it requires another feedback channel type or mechanism different from 2Tx and 4Tx codebook cases. Thus, in this contribution, we suggest to enlarge the 8Tx codebook as big as 6bits for rank 1 and 2 in order to enjoying more throughput gain from the larger codebook while making use of identical feedback mechanism regardless of the number of transmit antennas.

- Reference

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

[Remedy 1] Modify C(8,1,4), C(8,2,4), C(8,3,4), C(8,4,4), C(8,5,4), C(8,6,4),C(8,7,4) and C(8,8,4) in Table 679 and 680 as C(8,1,6), C(8,2,6), C(8,3,6), C(8,4,6), C(8,5,6), C(8,6,6),C(8,7,6) and C(8,8,6) .

[Remedy 2] Add the following text at the end of 15.3.7.2.6.6.2.3.1 SU-MIMO base codebook.

-------------------------------  Text Start  ---------------------------------------------------

---------------------------------  Text End  ---------------------------------------------------