| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO codebook clean-up |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics | wbong@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | In this contribution, we provide clean-up text for MIMO codebook sections. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO codebook clean-up

Wookbong Lee, Bin-Chul Ihm

LG Electronics

- Introduction

In session #60, codebooks from different companies have been adopted in AWD [1]. The codebook format needs to be unified for the sake of developers convenience. In addition to that, specifying exact value for each codeword element would be desirable for reducing gap between different floating point operations in different vendor.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”

- Proposed Remedy

Remedy #1. Modify texts in section 15.3.7.2.6.6.2.1 to 15.3.7.2.6.6.2.4.3

-------------------------------  Text Start  ---------------------------------------------------

- 15.3.7.2.6.6.2.1 Base codebook for two transmit antennas

- 15.3.7.2.6.6.2.1 SU-MIMO base codebook

- The codebook for two transmit antenna is constructed using a similar methodology as described in section 8.4.11.15 of WirelessMAN-OFDMA, with the exception that the first codeword v1 is defined as an Nt  by Mt unitary matrix

- v1=1Nt11⋯1ej2πNtej2πNt2j2πNtMt⋮⋮⋱ej2πNtNt−1ej2πNt2Nt−1ej2πNtMtNt−1			(1)

- All codeword matrices vi for i = 2,3…,2NB in V(Nt, Mt, NB) codebook can be derived from the first codeword matrix v1 using the following equations

- vi=HsQi−1HHsv1,				(2)

- vi=vi⋅e−jϕ1,					(3)

- where phase φ1 is a phase of the first entry of the first column of codeword vi. Operation H generates unitary matrix as follows

- H(v)=I, v=e1I−2wwHwHw, otherwise ,			(4)

- where w=v−e1, e1=1,0,...,0T.

- Matrix Qi is defined as following

- Qi=diage2π2Lu1i,e2π2Lu2i,...,e2π2LuNti,			(5)

- The parameters for generation of the codebooks for two transmit antenna Nt = 2 and number of streams Mt =1, 2 are listed in Table 1.

Table 1 – Generating parameters for two transmit antenna codebook

| Nt | Mt | NB | L | u=u1,u2,...,uNt in Qi(u) | s in H(s) |
| --- | --- | --- | --- | --- | --- |
| 2 | 1 | 3 | 3 | [1, 2] | [1, 0] |
| 2 | 2 | 2 | 3 |  |  |

Table x1 – C(2,1,3)

| Binary Index | m |  |  |
| --- | --- | --- | --- |
|  |  | c1 | c2 |
| 000 | 0 | 0.7071 | -0.7071 |
| 001 | 1 | 0.7071 | -0.5000 - 0.5000i |
| 010 | 2 | 0.7071 | - 0.7071i |
| 011 | 3 | 0.7071 | 0.5000 - 0.5000i |
| 100 | 4 | 0.7071 | 0.7071 |
| 101 | 5 | 0.7071 | 0.5000 + 0.5000i |
| 110 | 6 | 0.7071 | 0.7071i |
| 111 | 7 | 0.7071 | -0.5000 + 0.5000i |

Table x2 – C(2,2,3)

| Binary Index | m |  |  |
| --- | --- | --- | --- |
|  |  | c11 c21 | c12 c22 |
| 000 | 0 | 0.7071 0.7071 | -0.7071 0.7071 |
| 001 | 1 | 0.7071 0.7071 | -0.5000 - 0.5000i 0.5000 + 0.5000i |
| 010 | 2 | 0.7071 0.7071 | - 0.7071i 0.7071i |
| 011 | 3 | 0.7071 0.7071 | 0.5000 - 0.5000i -0.5000 + 0.5000i |
| 100~111 | 4~7 | N/A | N/A |

- 15.3.7.2.6.6.2.1.2 MU-MIMO base codebook

The base codebook for MU-MIMO is the same as the rank 1 base codebook for SU-MIMO, defined in .

- 15.3.7.2.6.6.2.2 Base codebook for four transmit antennas

- 15.3.7.2.6.6.2.2.1 SU-MIMO base codebook

The base codebooks of SU-MIMO with four transmit antennas consist of rank-1 codebook C(4,1,6), rank-2 codebook C(4,2,6), rank-3 codebook C(4,3,46) and rank-4 codebook C(4,4,36). Table 674, Table 675, Table 676, and Table 677 are included to illustrate the rank-1,2,3,4 base codebooks.

Table 674 – C(4,1,6)

| Binary Index | # m | C(4,1,6,m) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | c1 | c2 | c3 | c4 |
| 000000 | 0 | 0.5000 | -0.5000 | 0.5000 | -0.5000 |
| 000001 | 1 | -0.5000 | -0.5000 | 0.5000 | 0.5000 |
| 000010 | 2 | -0.5000 | 0.5000 | 0.5000 | -0.5000 |
| 000011 | 3 | 0.5000 | 0.0000 - 0.5000i | 0.5000 | 0.0000 - 0.5000i |
| 000100 | 4 | -0.5000 | 0.0000 - 0.5000i | 0.5000 | 0.0000 + 0.5000i |
| 000101 | 5 | -0.5000 | 0.0000 + 0.5000i | 0.5000 | 0.0000 - 0.5000i |
| 000110 | 6 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
| 000111 | 7 | 0.5000 | 0.0000 + 0.5000i | 0.5000 | 0.0000 + 0.5000i |
| 001000 | 8 | 0.5000 | 0.5000 | 0.5000 | -0.5000 |
| 001001 | 9 | 0.5000 | 0.0000 + 0.5000i | -0.5000 | 0.0000 + 0.5000i |
| 001010 | 10 | 0.5000 | -0.5000 | 0.5000 | 0.5000 |
| 001011 | 11 | 0.5000 | 0.0000 - 0.5000i | -0.5000 | 0.0000 - 0.5000i |
| 001100 | 12 | 0.5000 | 0.3536 + 0.3536i | 0.0000 + 0.5000i | -0.3536 + 0.3536i |
| 001101 | 13 | 0.5000 | -0.3536 + 0.3536i | 0.0000 – 0.5000i | 0.3536 + 0.3536i |
| 001110 | 14 | 0.5000 | -0.3536 - 0.3536i | 0.0000 + 0.5000i | 0.3536 - 0.3536i |
| 001111 | 15 | 0.5000 | 0.3536 - 0.3536i | 0.0000 – 0.5000i | -0.3536 - 0.3536i |
| 010000 | 16 | 0.5000 | -0.4619 - 0.1913i | 0.3536 + 0.3536i | -0.1913 - 0.4619i |
| 010001 | 17 | 0.3117 | 0.6025 + 0.1995i | -0.4030 - 0.4903i | -0.1122 - 0.2908i |
| 010010 | 18 | 0.3117 | -0.6025 - 0.1995i | -0.1122 - 0.2908i | 0.4030 + 0.4903i |
| 010011 | 19 | 0.3058 | 0.1901 - 0.6052i | 0.1195 + 0.2866i | 0.4884 - 0.4111i |
| 010100 | 20 | 0.5000 | -0.1913 + 0.4619i | -0.3536 - 0.3536i | 0.4619 - 0.1913i |
| 010101 | 21 | 0.5000 | 0.1913 - 0.4619i | -0.3536 - 0.3536i | -0.4619 + 0.1913i |
| 010110 | 22 | 0.5000 | 0.4619 + 0.1913i | 0.3536 + 0.3536i | 0.1913 + 0.4619i |
| 010111 | 23 | 0.3082 | 0.0104 + 0.3151i | 0.4077 + 0.4887i | -0.4783 + 0.4145i |
| 011000 | 24 | 0.3117 | 0.3573 - 0.2452i | 0.6025 - 0.1995i | -0.1578 + 0.5360i |
| 011001 | 25 | 0.3117 | 0.2452 + 0.3573i | -0.6025 + 0.1995i | 0.5360 + 0.1578i |
| 011010 | 26 | 0.3082 | -0.3666 + 0.2426i | 0.6092 - 0.1842i | 0.1615 - 0.5298i |
| 011011 | 27 | 0.3117 | -0.2452 - 0.3573i | -0.6025 + 0.1995i | -0.5360 - 0.1578i |
| 011100 | 28 | 0.3117 | 0.4260 + 0.0793i | 0.1995 + 0.6025i | 0.2674 + 0.4906i |
| 011101 | 29 | 0.3117 | -0.0793 + 0.4260i | -0.1995 - 0.6025i | 0.4906 - 0.2674i |
| 011110 | 30 | 0.3117 | -0.4260 - 0.0793i | 0.1995 + 0.6025i | -0.2674 - 0.4906i |
| 011111 | 31 | 0.3117 | 0.0793 - 0.4260i | -0.1995 - 0.6025i | -0.4906 + 0.2674i |
| 100000 | 32 | 0.5636 | -0.3332 - 0.2672i | 0.1174 + 0.5512i | -0.3308 - 0.2702i |
| 100001 | 33 | 0.5587 | 0.3361 + 0.2735i | -0.3361 - 0.2735i | -0.1135 - 0.5471i |
| 100010 | 34 | 0.5587 | -0.3361 - 0.2735i | -0.1135 - 0.5471i | 0.3361 + 0.2735i |
| 100011 | 35 | 0.5587 | 0.2735 - 0.3361i | 0.1135 + 0.5471i | 0.2735 - 0.3361i |
| 100100 | 36 | 0.3082 | -0.4887 + 0.4077i | -0.6092 - 0.1842i | 0.2837 - 0.1205i |
| 100101 | 37 | 0.5636 | 0.2673 - 0.3331i | -0.1222 - 0.5501i | -0.2673 + 0.3331i |
| 100110 | 38 | 0.5636 | 0.3691 + 0.5142i | 0.3331 + 0.2673i | 0.0862 + 0.3032i |
| 100111 | 39 | 0.5587 | -0.2990 + 0.0880i | 0.3361 + 0.2735i | -0.5216 + 0.3616i |
| 101000 | 40 | 0.5587 | 0.0880 - 0.2990i | 0.3361 - 0.2735i | -0.3616 + 0.5216i |
| 101001 | 41 | 0.5587 | 0.2990 + 0.0881i | -0.3362 + 0.2735i | 0.5216 + 0.3616i |
| 101010 | 42 | 0.5587 | -0.0880 + 0.2990i | 0.3361 - 0.2735i | 0.3616 - 0.5216i |
| 101011 | 43 | 0.5587 | -0.2990 - 0.0880i | -0.3361 + 0.2735i | -0.5216 - 0.3616i |
| 101100 | 44 | 0.5636 | 0.2741 - 0.1559i | 0.2672 + 0.3332i | 0.1081 + 0.6236i |
| 101101 | 45 | 0.5636 | 0.1559 + 0.2741i | -0.2672 - 0.3332i | 0.6236 - 0.1081i |
| 101110 | 46 | 0.5587 | -0.2737 + 0.1492i | 0.2735 + 0.3361i | -0.1132 - 0.6245i |
| 101111 | 47 | 0.5587 | -0.1492 - 0.2737i | -0.2735 - 0.3361i | -0.6245 + 0.1132i |
| 110000 | 48 | 0.5000 | -0.4619 + 0.1913i | 0.3536 - 0.3536i | -0.1913 + 0.4619i |
| 110001 | 49 | 0.3117 | 0.4030 + 0.4903i | -0.6025 - 0.1995i | -0.1122 - 0.2908i |
| 110010 | 50 | 0.3117 | -0.4029 - 0.4904i | -0.1184 - 0.2883i | 0.6067 + 0.1865i |
| 110011 | 51 | 0.3082 | 0.4887 - 0.4077i | 0.1205 + 0.2837i | 0.1842 - 0.6092i |
| 110100 | 52 | 0.5000 | 0.1913 + 0.4619i | -0.3536 + 0.3536i | -0.4619 - 0.1913i |
| 110101 | 53 | 0.5000 | -0.1913 - 0.4619i | -0.3536 + 0.3536i | 0.4619 + 0.1913i |
| 110110 | 54 | 0.5000 | 0.4619 - 0.1913i | 0.3536 - 0.3536i | 0.1913 - 0.4619i |
| 110111 | 55 | 0.3117 | -0.2452 + 0.3573i | 0.6025 + 0.1995i | -0.5360 + 0.1578i |
| 111000 | 56 | 0.3117 | 0.3117 - 0.0000i | 0.4030 - 0.4903i | -0.4030 + 0.4903i |
| 111001 | 57 | 0.3117 | -0.0000 + 0.3117i | -0.4030 + 0.4903i | 0.4903 + 0.4030i |
| 111010 | 58 | 0.3082 | -0.3152 - 0.0036i | 0.4076 - 0.4888i | 0.4040 - 0.4872i |
| 111011 | 59 | 0.3082 | 0.0036 - 0.3152i | -0.4076 + 0.4888i | -0.4872 - 0.4040i |
| 111100 | 60 | 0.3117 | 0.2204 + 0.2204i | 0.4903 + 0.4030i | 0.0618 + 0.6317i |
| 111101 | 61 | 0.3117 | -0.2204 + 0.2204i | -0.4903 - 0.4030i | 0.6317 - 0.0618i |
| 111110 | 62 | 0.3082 | -0.2154 - 0.2302i | 0.4887 + 0.4077i | -0.0451 - 0.6313i |
| 111111 | 63 | 0.3082 | 0.2254 - 0.2204i | -0.4888 - 0.4076i | -0.6302 + 0.0588i |

Table 675 – C(4,2,6)

| # C(4,2,6,m)=C(4,1,6,i)C(4,1,6,j) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary index | m | i,j | Binary index | m | i,j | Binary index | m | i,j |  | Binary index | m | i,j |
| 000000 | 0 | 6,0 | 010000 | 16 | 0,4 | 100000 | 32 | 12,14 | 110000 |  | 48 | 32,34 |
| 000001 | 1 | 6,1 | 010001 | 17 | 0,5 | 100001 | 33 | 12,15 | 110001 |  | 49 | 34,35 |
| 000010 | 2 | 6,2 | 010010 | 18 | 0,33 | 100010 | 34 | 12,31 | 110010 |  | 50 | 35,7 |
| 000011 | 3 | 0,1 | 010011 | 19 | 1,3 | 100011 | 35 | 13,14 | 110011 |  | 51 | 40,11 |
| 000100 | 4 | 0,2 | 010100 | 20 | 1,34 | 100100 | 36 | 5,23 | 110100 |  | 52 | 41,43 |
| 000101 | 5 | 1,2 | 010101 | 21 | 1,55 | 100101 | 37 | 14,15 | 110101 |  | 53 | 44,46 |
| 000110 | 6 | 7,4 | 010110 | 22 | 2,3 | 100110 | 38 | 14,47 | 110110 |  | 54 | 45,47 |
| 000111 | 7 | 7,5 | 010111 | 23 | 8,9 | 100111 | 39 | 17,2 | 110111 |  | 55 | 49,6 |
| 001000 | 8 | 3,4 | 011000 | 24 | 2,39 | 101000 | 40 | 17,3 | 111000 |  | 56 | 52,53 |
| 001001 | 9 | 3,5 | 011001 | 25 | 8,11 | 101001 | 41 | 18,19 | 111001 |  | 57 | 56,10 |
| 001010 | 10 | 6,4 | 011010 | 26 | 8,27 | 101010 | 42 | 18,6 | 111010 |  | 58 | 56,58 |
| 001011 | 11 | 6,5 | 011011 | 27 | 9,10 | 101011 | 43 | 24,9 | 111011 |  | 59 | 57,11 |
| 001100 | 12 | 7,1 | 011100 | 28 | 9,42 | 101100 | 44 | 25,10 | 111100 |  | 60 | 57,59 |
| 001101 | 13 | 7,2 | 011101 | 29 | 10,11 | 101101 | 45 | 28,13 | 111101 |  | 61 | 60,14 |
| 001110 | 14 | 8,10 | 011110 | 30 | 10,43 | 101110 | 46 | 29,14 | 111110 |  | 62 | 61,15 |
| 001111 | 15 | 13,15 | 011111 | 31 | 12,13 | 101111 | 47 | 30,15 | 111111 |  | 63 | 61,63 |

Table 675 – C(4,2,6)

| Binary Index | # m |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | c11 c21 | c12 c22 | c13 c23 | c14 c24 |
| 000000 | 0 | 0.5000 0.5000 | 0.5000 -0.5000 | 0.5000 0.5000 | 0.5000 -0.5000 |
| 000001 | 1 | 0.5000 -0.5000 | 0.5000 -0.5000 | 0.5000 0.5000 | 0.5000 0.5000 |
| 000010 | 2 | 0.5000 -0.5000 | 0.5000 0.5000 | 0.5000 0.5000 | 0.5000 -0.5000 |
| 000011 | 3 | 0.5000 -0.5000 | -0.5000 -0.5000 | 0.5000 0.5000 | -0.5000 -0.5000 |
| 000100 | 4 | 0.5000 -0.5000 | -0.5000 0.5000 | 0.5000 0.5000 | -0.5000 -0.5000 |
| 000101 | 5 | -0.5000 -0.5000 | -0.5000 0.5000 | 0.5000 0.5000 | 0.5000 -0.5000 |
| 000110 | 6 | 0.5000 -0.5000 | 0.5000i - 0.5000i | 0.5000 0.5000 | 0.5000i 0.5000i |
| 000111 | 7 | 0.5000 -0.5000 | 0.5000i 0.5000i | 0.5000 0.5000 | 0.5000i - 0.5000i |
| 001000 | 8 | 0.5000 -0.5000 | - 0.5000i - 0.5000i | 0.5000 0.5000 | - 0.5000i 0.5000i |
| 001001 | 9 | 0.5000 -0.5000 | - 0.5000i 0.5000i | 0.5000 0.5000 | - 0.5000i - 0.5000i |
| 001010 | 10 | 0.5000 -0.5000 | 0.5000 - 0.5000i | 0.5000 0.5000 | 0.5000 0.5000i |
| 001011 | 11 | 0.5000 -0.5000 | 0.5000 0.5000i | 0.5000 0.5000 | 0.5000 - 0.5000i |
| 001100 | 12 | 0.5000 -0.5000 | 0.5000i -0.5000 | 0.5000 0.5000 | 0.5000i 0.5000 |
| 001101 | 13 | 0.5000 -0.5000 | 0.5000i 0.5000 | 0.5000 0.5000 | 0.5000i -0.5000 |
| 001110 | 14 | 0.5000 0.5000 | 0.5000 -0.5000 | 0.5000 0.5000 | -0.5000 0.5000 |
| 001111 | 15 | 0.5000 0.5000 | -0.3536 + 0.3536i 0.3536 - 0.3536i | – 0.5000i – 0.5000i | 0.3536 + 0.3536i -0.3536 - 0.3536i |
| 010000 | 16 | 0.5000 -0.5000 | -0.5000 - 0.5000i | 0.5000 0.5000 | -0.5000 0.5000i |
| 010001 | 17 | 0.5000 -0.5000 | -0.5000 0.5000i | 0.5000 0.5000 | -0.5000 - 0.5000i |
| 010010 | 18 | 0.5000 0.5587 | -0.5000 0.3361 + 0.2735i | 0.5000 -0.3361 - 0.2735i | -0.5000 -0.1135 - 0.5471i |
| 010011 | 19 | -0.5000 0.5000 | -0.5000 - 0.5000i | 0.5000 0.5000 | 0.5000 - 0.5000i |
| 010100 | 20 | -0.5000 0.5587 | -0.5000 -0.3361 - 0.2735i | 0.5000 -0.1135 - 0.5471i | 0.5000 0.3361 + 0.2735i |
| 010101 | 21 | -0.5000 0.3117 | -0.5000 -0.2452 + 0.3573i | 0.5000 0.6025 + 0.1995i | 0.5000 -0.5360 + 0.1578i |
| 010110 | 22 | -0.5000 0.5000 | 0.5000 - 0.5000i | 0.5000 0.5000 | -0.5000 - 0.5000i |
| 010111 | 23 | 0.5000 0.5000 | 0.5000 0.5000i | 0.5000 -0.5000 | -0.5000 0.5000i |
| 011000 | 24 | -0.5000 0.5587 | 0.5000 -0.2990 + 0.0880i | 0.5000 0.3361 + 0.2735i | -0.5000 -0.5216 + 0.3616i |
| 011001 | 25 | 0.5000 0.5000 | 0.5000 - 0.5000i | 0.5000 -0.5000 | -0.5000 - 0.5000i |
| 011010 | 26 | 0.5000 0.3117 | 0.5000 -0.2452 - 0.3573i | 0.5000 -0.6025 + 0.1995i | -0.5000 -0.5360 - 0.1578i |
| 011011 | 27 | 0.5000 0.5000 | 0.5000i -0.5000 | -0.5000 0.5000 | 0.5000i 0.5000 |
| 011100 | 28 | 0.5000 0.5587 | 0.5000i -0.0880 + 0.2990i | -0.5000 0.3361 - 0.2735i | 0.5000i 0.3616 - 0.5216i |
| 011101 | 29 | 0.5000 0.5000 | -0.5000 - 0.5000i | 0.5000 -0.5000 | 0.5000 - 0.5000i |
| 011110 | 30 | 0.5000 0.5587 | -0.5000 -0.2990 - 0.0880i | 0.5000 -0.3361 + 0.2735i | 0.5000 -0.5216 - 0.3616i |
| 011111 | 31 | 0.5000 0.5000 | 0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000i – 0.5000i | -0.3536 + 0.3536i 0.3536 + 0.3536i |
| 100000 | 32 | 0.5000 0.5000 | 0.3536 + 0.3536i -0.3536 - 0.3536i | 0.5000i 0.5000i | -0.3536 + 0.3536i 0.3536 - 0.3536i |
| 100001 | 33 | 0.5000 0.5000 | 0.3536 + 0.3536i 0.3536 - 0.3536i | 0.5000i – 0.5000i | -0.3536 + 0.3536i -0.3536 - 0.3536i |
| 100010 | 34 | 0.5000 0.3117 | 0.3536 + 0.3536i 0.0793 - 0.4260i | 0.5000i -0.1995 - 0.6025i | -0.3536 + 0.3536i -0.4906 + 0.2674i |
| 100011 | 35 | 0.5000 0.5000 | -0.3536 + 0.3536i -0.3536 - 0.3536i | – 0.5000i 0.5000i | 0.3536 + 0.3536i 0.3536 - 0.3536i |
| 100100 | 36 | -0.5000 0.3082 | 0.5000i 0.0104 + 0.3151i | 0.5000 0.4077 + 0.4887i | - 0.5000i -0.4783 + 0.4145i |
| 100101 | 37 | 0.5000 0.5000 | -0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000i – 0.5000i | 0.3536 - 0.3536i -0.3536 - 0.3536i |
| 100110 | 38 | 0.5000 0.5587 | -0.3536 - 0.3536i -0.1492 - 0.2737i | 0.5000i -0.2735 - 0.3361i | 0.3536 - 0.3536i -0.6245 + 0.1132i |
| 100111 | 39 | 0.3117 -0.5000 | 0.6025 + 0.1995i 0.5000 | -0.4030 - 0.4903i 0.5000 | -0.1122 - 0.2908i -0.5000 |
| 101000 | 40 | 0.3117 -0.5000 | 0.6025 + 0.1995i 0.5000 | -0.4030 - 0.4903i 0.5000 | -0.1122 - 0.2908i -0.5000 |
| 101001 | 41 | 0.3117 0.3058 | -0.6025 - 0.1995i 0.1901 - 0.6052i | -0.1122 - 0.2908i 0.1195 + 0.2866i | 0.4030 + 0.4903i 0.4884 - 0.4111i |
| 101010 | 42 | 0.3117 0.5000 | -0.6025 - 0.1995i 0.5000 | -0.1122 - 0.2908i 0.5000 | 0.4030 + 0.4903i 0.5000 |
| 101011 | 43 | 0.3117 0.5000 | 0.3573 - 0.2452i 0.5000i | 0.6025 - 0.1995i -0.5000 | -0.1578 + 0.5360i 0.5000i |
| 101100 | 44 | 0.3117 0.5000 | 0.2452 + 0.3573i -0.5000 | -0.6025 + 0.1995i 0.5000 | 0.5360 + 0.1578i 0.5000 |
| 101101 | 45 | 0.3117 0.5000 | 0.4260 + 0.0793i -0.3536 + 0.3536i | 0.1995 + 0.6025i – 0.5000i | 0.2674 + 0.4906i 0.3536 + 0.3536i |
| 101110 | 46 | 0.3117 0.5000 | -0.0793 + 0.4260i -0.3536 - 0.3536i | -0.1995 - 0.6025i 0.5000i | 0.4906 - 0.2674i 0.3536 - 0.3536i |
| 101111 | 47 | 0.3117 0.5000 | -0.4260 - 0.0793i 0.3536 - 0.3536i | 0.1995 + 0.6025i – 0.5000i | -0.2674 - 0.4906i -0.3536 - 0.3536i |
| 110000 | 48 | 0.5636 0.5587 | -0.3332 - 0.2672i -0.3361 - 0.2735i | 0.1174 + 0.5512i -0.1135 - 0.5471i | -0.3308 - 0.2702i 0.3361 + 0.2735i |
| 110001 | 49 | 0.5587 0.5587 | -0.3361 - 0.2735i 0.2735 - 0.3361i | -0.1135 - 0.5471i 0.1135 + 0.5471i | 0.3361 + 0.2735i 0.2735 - 0.3361i |
| 110010 | 50 | 0.5587 0.5000 | 0.2735 - 0.3361i 0.5000i | 0.1135 + 0.5471i 0.5000 | 0.2735 - 0.3361i 0.5000i |
| 110011 | 51 | 0.5587 0.5000 | 0.0880 - 0.2990i - 0.5000i | 0.3361 - 0.2735i -0.5000 | -0.3616 + 0.5216i - 0.5000i |
| 110100 | 52 | 0.5587 0.5587 | 0.2990 + 0.0881i -0.2990 - 0.0880i | -0.3362 + 0.2735i -0.3361 + 0.2735i | 0.5216 + 0.3616i -0.5216 - 0.3616i |
| 110101 | 53 | 0.5636 0.5587 | 0.2741 - 0.1559i -0.2737 + 0.1492i | 0.2672 + 0.3332i 0.2735 + 0.3361i | 0.1081 + 0.6236i -0.1132 - 0.6245i |
| 110110 | 54 | 0.5636 0.5587 | 0.1559 + 0.2741i -0.1492 - 0.2737i | -0.2672 - 0.3332i -0.2735 - 0.3361i | 0.6236 - 0.1081i -0.6245 + 0.1132i |
| 110111 | 55 | 0.3117 0.5000 | 0.4030 + 0.4903i 0.5000 | -0.6025 - 0.1995i 0.5000 | -0.1122 - 0.2908i 0.5000 |
| 111000 | 56 | 0.5000 0.5000 | 0.1913 + 0.4619i -0.1913 - 0.4619i | -0.3536 + 0.3536i -0.3536 + 0.3536i | -0.4619 - 0.1913i 0.4619 + 0.1913i |
| 111001 | 57 | 0.3117 0.5000 | 0.3117 -0.5000 | 0.4030 - 0.4903i 0.5000 | -0.4030 + 0.4903i 0.5000 |
| 111010 | 58 | 0.3117 0.3082 | 0.3117 -0.3152 - 0.0036i | 0.4030 - 0.4903i 0.4076 - 0.4888i | -0.4030 + 0.4903i 0.4040 - 0.4872i |
| 111011 | 59 | 0.3117 0.5000 | 0.3117i - 0.5000i | -0.4030 + 0.4903i -0.5000 | 0.4903 + 0.4030i - 0.5000i |
| 111100 | 60 | 0.3117 0.3082 | 0.3117i 0.0036 - 0.3152i | -0.4030 + 0.4903i -0.4076 + 0.4888i | 0.4903 + 0.4030i -0.4872 - 0.4040i |
| 111101 | 61 | 0.3117 0.5000 | 0.2204 + 0.2204i -0.3536 - 0.3536i | 0.4903 + 0.4030i 0.5000i | 0.0618 + 0.6317i 0.3536 - 0.3536i |
| 111110 | 62 | 0.3117 0.5000 | -0.2204 + 0.2204i 0.3536 - 0.3536i | -0.4903 - 0.4030i – 0.5000i | 0.6317 - 0.0618i -0.3536 - 0.3536i |
| 111111 | 63 | 0.3117 0.3082 | -0.2204 + 0.2204i 0.2254 - 0.2204i | -0.4903 - 0.4030i -0.4888 - 0.4076i | 0.6317 - 0.0618i -0.6302 + 0.0588i |

Table 676 – C(4,3,6)

| Binary index | m | i,j,k | Binary index | m | i,j,k | Binary index | m | i,j,k |  | Binary Index | m | i,j,k |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 6,0,1 | 000100 | 4 | 7,3,4 | 001000 | 8 | 6,0,4 | 001100 |  | 12 | 8,9,10 |
| 000001 | 1 | 6,0,2 | 000101 | 5 | 7,3,5 | 001001 | 9 | 6,4,5 | 001101 |  | 13 | 8,10,11 |
| 000010 | 2 | 6,1,2 | 000110 | 6 | 7,4,5 | 001010 | 10 | 7,3,1 | 001110 |  | 14 | 12,13,15 |
| 000011 | 3 | 0,1,2 | 000111 | 7 | 3,4,5 | 001011 | 11 | 7,1,2 | 001111 |  | 15 | 13,14,15 |
| 010000 ~ 111111 | 16 ~ 63 | N/A |  |  |  |  |  |  |  |  |  |  |

- The indexes from 16 to 63 are not used in 6-bits downlink PMI feedback for Mt =3 codebook.

Table 676 – C(4,3,4)

| Binary Index | # m |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | c11c21c31 | c12c22c32 | c13c23c33 | c14c24c34 |
| 000000 | 0 | 0.5000 0.5000 -0.5000 | 0.5000 -0.5000 -0.5000 | 0.5000 0.5000 0.5000 | 0.5000 -0.5000 0.5000 |
| 000001 | 1 | 0.5000 0.5000 -0.5000 | 0.5000 -0.5000 0.5000 | 0.5000 0.5000 0.5000 | 0.5000 -0.5000 -0.5000 |
| 000010 | 2 | 0.5000 -0.5000 -0.5000 | 0.5000 -0.5000 0.5000 | 0.5000 0.5000 0.5000 | 0.5000 0.5000 -0.5000 |
| 000011 | 3 | 0.5000 -0.5000 -0.5000 | -0.5000 -0.5000 0.5000 | 0.5000 0.5000 0.5000 | -0.5000 0.5000 -0.5000 |
| 000100 | 4 | 0.5000 0.5000 -0.5000 | 0.5000i - 0.5000i - 0.5000i | 0.5000 0.5000 0.5000 | 0.5000i - 0.5000i 0.5000i |
| 000101 | 5 | 0.5000 0.5000 -0.5000 | 0.5000i - 0.5000i 0.5000i | 0.5000 0.5000 0.5000 | 0.5000i - 0.5000i - 0.5000i |
| 000110 | 6 | 0.5000 -0.5000 -0.5000 | 0.5000i - 0.5000i 0.5000i | 0.5000 0.5000 0.5000 | 0.5000i 0.5000i - 0.5000i |
| 000111 | 7 | 0.5000 -0.5000 -0.5000 | - 0.5000i - 0.5000i 0.5000i | 0.5000 0.5000 0.5000 | - 0.5000i 0.5000i - 0.5000i |
| 001000 | 8 | 0.5000 0.5000 -0.5000 | 0.5000 -0.5000 - 0.5000i | 0.5000 0.5000 0.5000 | 0.5000 -0.5000 0.5000i |
| 001001 | 9 | 0.5000 -0.5000 -0.5000 | 0.5000 - 0.5000i 0.5000i | 0.5000 0.5000 0.5000 | 0.5000 0.5000i - 0.5000i |
| 001010 | 10 | 0.5000 0.5000 -0.5000 | 0.5000i - 0.5000i -0.5000 | 0.5000 0.5000 0.5000 | 0.5000i - 0.5000i 0.5000 |
| 001011 | 11 | 0.5000 -0.5000 -0.5000 | 0.5000i -0.5000 0.5000 | 0.5000 0.5000 0.5000 | 0.5000i 0.5000 -0.5000 |
| 001100 | 12 | 0.5000 0.5000 0.5000 | 0.5000 0.5000i -0.5000 | 0.5000 -0.5000 0.5000 | -0.5000 0.5000i 0.5000 |
| 001101 | 13 | 0.5000 0.5000 0.5000 | 0.5000 -0.5000 - 0.5000i | 0.5000 0.5000 -0.5000 | -0.5000 0.5000 - 0.5000i |
| 001110 | 14 | 0.5000 0.5000 0.5000 | 0.3536 + 0.3536i -0.3536 + 0.3536i 0.3536 - 0.3536i | 0.5000i - 0.5000i - 0.5000i | -0.3536 + 0.3536i 0.3536 + 0.3536i -0.3536 - 0.3536i |
| 001111 | 15 | 0.5000 0.5000 0.5000 | -0.3536 + 0.3536i -0.3536 - 0.3536i 0.3536 - 0.3536i | - 0.5000i 0.5000i - 0.5000i | 0.3536 + 0.3536i 0.3536 - 0.3536i -0.3536 - 0.3536i |
| 010000 ~ 111111 | 16 ~ 63 | N/A |  |  |  |

- The indexes from 16 to 63 are not used in 6-bits downlink PMI feedback for Mt =3 codebook.

Table 677 – C(4,4,36)

| Binary index | m | i,j,k,p | Binary index | m | i,j,k,p | Binary index | m | i,j,k,p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 6,0,1,2 | 000010 | 2 | 6,0,4,5 | 000100 | 4 | 8,9,10,11 |
| 000001 | 1 | 7,3,4,5 | 000011 | 3 | 7,3,1,2 | 000101 | 5 | 12,13,14,15 |
| 000110 ~ 111111 | 6 ~ 63 | N/A |  |  |  |  |  |  |

- The indexes from 6 to 63 are not used in 6-bits downlink PMI feedback for Mt =4 codebook.

Table 677 – C(4,4,3)

| Binary index | m |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | c11c21c31c41 | cc22c32c42 | cc23c33c43 | cc24c34c44 |
| 000000 | 0 | 0.5000 0.5000 -0.5000 -0.5000 | 0.5000 -0.5000 -0.5000 0.5000 | 0.5000 0.5000 0.5000 0.5000 | 0.5000 -0.5000 0.5000 -0.5000 |
| 000001 | 1 | 0.5000 0.5000 -0.5000 -0.5000 | 0.5000i - 0.5000i - 0.5000i 0.5000i | 0.5000 0.5000 0.5000 0.5000 | 0.5000i - 0.5000i 0.5000i - 0.5000i |
| 000010 | 2 | 0.5000 0.5000 -0.5000 -0.5000 | 0.5000 -0.5000 - 0.5000i 0.5000i | 0.5000 0.5000 0.5000 0.5000 | 0.5000 -0.5000 0.5000i - 0.5000i |
| 000011 | 3 | 0.5000 0.5000 -0.5000 -0.5000 | 0.0000 + 0.5000i 0.0000 - 0.5000i -0.5000 0.5000 | 0.5000 0.5000 0.5000 0.5000 | 0.5000i - 0.5000i 0.5000 -0.5000 |
| 000100 | 4 | 0.5000 0.5000 0.5000 0.5000 | 0.5000 0.0000 + 0.5000i -0.5000 0.0000 - 0.5000i | 0.5000 -0.5000 0.5000 -0.5000 | -0.5000 0.5000i 0.5000 0.0000 - 0.5000i |
| 000101 | 5 | 0.5000 0.5000 0.5000 0.5000 | 0.3536 + 0.3536i -0.3536 + 0.3536i -0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000i - 0.5000i 0.5000i - 0.5000i | -0.3536 + 0.3536i 0.3536 + 0.3536i 0.3536 - 0.3536i -0.3536 - 0.3536i |
| 000110 ~ 111111 | 6 ~ 63 | N/A |  |  |  |

- The indexes from 6 to 63 are not used in 6-bits downlink PMI feedback for Mt =4 codebook.

In terms of the chordal distance, the hierarchical structure of C(4,1,6) is depicted in Figure 1. In this hierarchical structure, it is shown that C(4,1,6) consists of 16 codeword clusters. Each codeword cluster has four codewords, of which one codeword is from Layer 1 and the three other codewords are from Layer 2. For any given Layer 2 codeword, its chordal distance to all other Layer 1 codewords of different clusters is always much larger than that distance to the Layer 1 codeword of its same cluster.

Figure 1: Chordal distance map of C(4,1,6)

As a potential benefit, this hierarchical structure can facilitate codeword searching. More specifically, codeword searching in C(4,1,6) can start from all Layer 1 codewords. Only when a Layer 1 codeword satisfies a certain criterion, associated Layer 2 codewords within the same cluster need to be searched.

The binary indices of the codewords in cluster i, i∈[0,⋯,15], is given by Table xxx

Table 678 – Binary indices of the codewords in cluster i

| Codeword in cluster i | Layer 1 codeword | Layer 2 codewords |  |  |
| --- | --- | --- | --- | --- |
|  |  | Codeword 1 | Codeword 2 | Codeword 3 |
| Binary index | 00xi3xi2xi1xi0 | 01xi3xi2xi1xi0 | 10xi3xi2xi1xi0 | 11xi3xi2xi1xi0 |
| i=xi3×23+xi2×22+xi1×2+xi0, xij∈[0,1], i∈[0,⋯,15], j∈[0,1,2,3] |  |  |  |  |

- 15.3.7.2.6.6.2.2.2 MU-MIMO base codebook

The base codebook for MU-MIMO is same as the rank 1 base codebook for SU-MIMO, defined in .

- 15.3.7.2.6.6.2.2.3 Base codebook for eight transmit antennas

- 15.3.7.2.6.6.2.3.1 SU-MIMO base codebook

- The base codebook is constructed from two matrices V8(:,:,1) and V8(:,:,2), which are constructed as described below.

- Define

- (6)

- (7)

- The two rank-8 matrices used for rank-2 to rank-8 transmission for SU-MIMO are

- (8)

- (9)

- The third base matrix, V8(:,:,3), is used for rank-1 transmission for MU-MIMO. The j-th column vector of the base matrix V8(:,:,3) is given by

- where the set θj, j=1,...,16, is given by

- The 4bits 8Tx base codebook is constructed from the three 8×8 base matrices and is specified in Table 2 and Table . Note that only the column indices of the corresponding base matrices are shown in Table 7 for brevity.

Table 2 – Rank 1 of SU MIMO 4bit 8Tx base codebook

| Codebook Matrix Index (CMI) | Base Matrix | C(8,1,4) |
| --- | --- | --- |
| 1 | V8(:,:,3) | V8(:,1,3) |
| 2 |  | V8(:,2,3) |
| 3 |  | V8(:,3,3) |
| 4 |  | V8(:,4,3) |
| 5 |  | V8(:,5,3) |
| 6 |  | V8(:,6,3) |
| 7 |  | V8(:,7,3) |
| 8 |  | V8(:,8,3) |
| 9 |  | V8(:,9,3) |
| 10 |  | V8(:,10,3) |
| 11 |  | V8(:,11,3) |
| 12 |  | V8(:,12,3) |
| 13 |  | V8(:,13,3) |
| 14 |  | V8(:,14,3) |
| 15 |  | V8(:,15,3) |
| 16 |  | V8(:,16,3) |

The base codebooks of SU-MIMO with eight transmit antennas consist of rank-1 codebook C(8,1,4), rank-2 codebook C(8,2,4), rank-3 codebook C(8,3,4), rank-4 codebook C(8,4,4), rank-5 codebook C(8,5,4), rank-6 codebook C(8,6,4), rank-7 codebook C(8,7,4) and rank-8 codebook C(8,8,4). Table 679 and Table 680 are included to illustrate the rank-1,2,3,4,5,6,7,8 base codebooks.

Table 679 – C(8,1,4)

| Binary Index | # m |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | c1 | c2 | c3 | c4 | c5 | c6 | c7 | c8 |
| 0000 | 0 | 0.3536 | -0.3051 - 0.1786i | 0.1732 + 0.3082i | 0.0062 - 0.3535i | -0.1839 + 0.3020i | 0.3112 - 0.1677i | -0.3533 - 0.0124i | 0.2987 + 0.1892i |
| 0001 | 1 | 0.3536 | -0.2514 - 0.2486i | 0.0041 + 0.3535i | 0.2456 - 0.2543i | -0.3535 + 0.0082i | 0.2571 + 0.2427i | -0.0123 - 0.3533i | -0.2397 + 0.2599i |
| 0010 | 2 | 0.3536 | -0.1697 - 0.3102i | -0.1907 + 0.2977i | 0.3527 + 0.0244i | -0.1479 - 0.3211i | -0.2107 + 0.2839i | 0.3502 + 0.0486i | -0.1254 - 0.3306i |
| 0011 | 3 | 0.3536 | -0.0614 - 0.3482i | -0.3322 + 0.1210i | 0.1768 + 0.3062i | 0.2708 - 0.2273i | -0.2709 - 0.2272i | -0.1767 + 0.3062i | 0.3323 + 0.1208i |
| 0100 | 4 | 0.3536 | 0.0638 - 0.3478i | -0.3306 - 0.1254i | -0.1830 + 0.3025i | 0.2646 + 0.2345i | 0.2784 - 0.2180i | -0.1642 - 0.3131i | -0.3376 + 0.1050i |
| 0101 | 5 | 0.3536 | 0.1881 - 0.2994i | -0.1534 - 0.3185i | -0.3513 - 0.0395i | -0.2204 + 0.2764i | 0.1168 + 0.3337i | 0.3447 + 0.0786i | 0.2499 - 0.2501i |
| 0110 | 6 | 0.3536 | 0.2892 - 0.2034i | 0.1196 - 0.3327i | -0.0936 - 0.3409i | -0.2727 - 0.2251i | -0.3525 - 0.0272i | -0.3040 + 0.1805i | -0.1449 + 0.3225i |
| 0111 | 7 | 0.3536 | 0.3461 - 0.0721i | 0.3241 - 0.1412i | 0.2885 - 0.2044i | 0.2407 - 0.2590i | 0.1828 - 0.3026i | 0.1172 - 0.3336i | 0.0467 - 0.3505i |
| 1000 | 8 | 0.3536 | 0.3461 + 0.0721i | 0.3241 + 0.1412i | 0.2885 + 0.2044i | 0.2407 + 0.2590i | 0.1828 + 0.3026i | 0.1172 + 0.3336i | 0.0467 + 0.3505i |
| 1001 | 9 | 0.3536 | 0.2892 + 0.2034i | 0.1196 + 0.3327i | -0.0936 + 0.3409i | -0.2727 + 0.2251i | -0.3525 + 0.0272i | -0.3040 - 0.1805i | -0.1449 - 0.3225i |
| 1010 | 10 | 0.3536 | 0.1881 + 0.2994i | -0.1534 + 0.3185i | -0.3513 + 0.0395i | -0.2204 - 0.2764i | 0.1168 - 0.3337i | 0.3447 - 0.0786i | 0.2499 + 0.2501i |
| 1011 | 11 | 0.3536 | 0.0638 + 0.3478i | -0.3306 + 0.1254i | -0.1830 - 0.3025i | 0.2646 - 0.2345i | 0.2784 + 0.2180i | -0.1642 + 0.3131i | -0.3376 - 0.1050i |
| 1100 | 12 | 0.3536 | -0.0614 + 0.3482i | -0.3322 - 0.1210i | 0.1768 - 0.3062i | 0.2708 + 0.2273i | -0.2709 + 0.2272i | -0.1767 - 0.3062i | 0.3323 - 0.1208i |
| 1101 | 13 | 0.3536 | -0.1697 + 0.3102i | -0.1907 - 0.2977i | 0.3527 - 0.0244i | -0.1479 + 0.3211i | -0.2107 - 0.2839i | 0.3502 - 0.0486i | -0.1254 + 0.3306i |
| 1110 | 14 | 0.3536 | -0.2514 + 0.2486i | 0.0041 - 0.3535i | 0.2456 + 0.2543i | -0.3535 - 0.0082i | 0.2571 - 0.2427i | -0.0123 + 0.3533i | -0.2397 - 0.2599i |
| 1111 | 15 | 0.3536 | -0.3051 + 0.1786i | 0.1732 - 0.3082i | 0.0062 + 0.3535i | -0.1839 - 0.3020i | 0.3112 + 0.1677i | -0.3533 + 0.0124i | 0.2987 - 0.1892i |

- The two rank-8 matrices used for rank-2 to rank-8 transmission for SU-MIMO are

- The 4bits 8Tx base codebook is constructed from the two 8×8 base matrices and is specified in 680. Note that only the column indices of the corresponding base matrices are shown in Table 680 for brevity.

Table 680 – Ranks 2 to 8 of SU MIMO 4bit 8Tx base codebook

| Codebook Matrix Index (CMI) | Base Matrix | C(8,2,4) | C(8,3,4) | C(8,4,4) | C(8,5,4) | C(8,6,4) | C(8,7,4) | C(8,8,4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 1 5 | 1 3 5 | 1537 | 12357 | 123567 | 1234567 | 12345678 |
| 1 |  | 2 6 | 2 4 6 | 2648 | 12468 | 124568 | 1234568 | n/a |
| 2 |  | 3 7 | 2 3 7 | 3726 | 23467 | 234678 | 1234678 | n/a |
| 3 |  | 4 8 | 1 4 8 | 4815 | 13458 | 134578 | 1234578 | n/a |
| 4 |  | 5 3 | 3 5 7 | 5372 | 23567 | 234567 | 2345678 | n/a |
| 5 |  | 4 6 | 4 6 8 | 6481 | 14568 | 134568 | 1345678 | n/a |
| 6 |  | 2 7 | 2 6 7 | 7264 | 24678 | 124678 | 1245678 | n/a |
| 7 |  | 8 1 | 1 5 8 | 8153 | 13578 | 123578 | 1235678 | n/a |
| 8 |  | 1 3 | 1 2 3 | 1234 | 12345 | 123456 | 1234567 | 12345678 |
| 9 |  | 2 4 | 1 2 4 | 1246 | 12456 | 124567 | 1245678 | n/a |
| 10 |  | 2 3 | 2 3 4 | 2437 | 23478 | 123478 | 1234578 | n/a |
| 11 |  | 1 4 | 1 3 4 | 1348 | 13478 | 134678 | 1234678 | n/a |
| 12 |  | 5 8 | 5 7 8 | 3578 | 23578 | 235678 | 1235678 | n/a |
| 13 |  | 6 7 | 6 7 8 | 4678 | 14678 | 145678 | 1345678 | n/a |
| 14 |  | 5 7 | 5 7 6 | 5678 | 35678 | 345678 | 2345678 | n/a |
| 15 |  | 6 8 | 5 6 8 | 1568 | 13568 | 123568 | 1234568 | n/a |

- The indexes from 1 to 7 and 8 to 15 are not used in 4-bits downlink PMI feedback for Mt =8 codebook.

- 15.3.7.2.6.6.2.3.2 MU-MIMO base codebook

The base codebook for MU-MIMO is same as the rank 1 base codebook for SU-MIMO, defined in .

- 15.3.7.2.6.6.2.4 Codebook subset selection

In codebook-based precoding, the precoding matrix W(k) shall be derived from PMI within the base codebook or a subset thereof. Subset information such as BC_SI, SU_CT, or MU_CT [TBD] is transmitted in [TBD].

Base Codebook Subset Indication (BC_SI) field determines which element of rank-1 codebook element is restricted or recommended for PMI feedback in case of MIMO mode 2 and 4. If the i-th element of BC_SI is set to 0, then i -th element of rank-1 codebook, C(Nt, 1, NB, i), is restricted for PMI feedback. This field shall be ignored when Codebook Coordination Enable (CCE) is set to 0b0. CCE is transmitted in [Feedback_Allocation_IE].

- 15.3.7.2.6.6.2.4.1 OL MIMO subset

- 15.3.7.2.6.6.2.4.2 CL SU-MIMO subset

- 15.3.7.2.6.4.2.4.2.1. CL SU-MIMO subset for four transmit antennas

Codebook subset selection for four transmit antennas is specified in Table 681.

Table 681 – Subset selection of the base codebook for four transmit antennas

| Rank | One | Two | Three | Four |
| --- | --- | --- | --- | --- |
| Subset selection | C(4,1,6,m) m = 0 to15 | C(4,2,6,m) m = 0 to15 | C(4,3,46,m) m = 0 to15 | C(4,4,36,m) m = 0 to 5 |

- 15.3.7.2.6.6.2.4.3 CL MU-MIMO subset

-------------------------------  Text End  ---------------------------------------------------

Remedy #2. Modify texts in section 15.3.10.3 to 15.3.10.3.2.2

-------------------------------  Text Start  ---------------------------------------------------

- 15.3.10.3 Codebook for closed-loop transmit precoding

- 15.3.10.3.1 Base codebook for two transmit antennas

- 15.3.10.3.1.1 SU-MIMO base codebook

The base codebook of uplink 2 Tx is the same as the downlink 2 Tx base codebook (SU MIMO base codebook), defined in Error: Reference source not found.

- 15.3.10.3.1.2 MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as base codebook for SU-MIMO, defined in Error: Reference source not found

- 15.3.10.3.2 Base codebook for four transmit antennas

- 15.3.10.3.2.1 SU-MIMO base codebook

- The codebooks for UL MIMO with four transmit antenna MS are constructed using the methodology described in section Error: Reference source not found for two transmit antenna case.

- The parameters for the generation of codebooks for four transmit antenna Nt = 4 and number of streams Mt =1, 2, 3, 4 are listed in Table 3.

Table 3 – Generating parameters for four transmit antenna codebook

| Nt | Mt | NB | L | u=u1,u2,...,uNt in Qi(u) | s in H(s) |
| --- | --- | --- | --- | --- | --- |
| 4 | 1 | 6 | 6 | [18, 55, 22, 6] | [1, 0, 0, 0] |
| 4 | 2 | 6 | 6 |  |  |
| 4 | 3 | 6 | 6 |  |  |
| 4 | 4 | 6 | 6 |  |  |

The base codebooks of SU-MIMO with four transmit antennas consist of rank-1 codebook C(4,1,6), rank-2 codebook C(4,2,6), rank-3 codebook C(4,3,6) and rank-4 codebook C(4,4,6). Rank-1 codebook entry C(4,1,6,m) is the first column of C(4,4,6,m). Rank-2 codebook entry C(4,2,6,m) is the first two columns of C(4,4,6). Rank-3 codebook elements C(4,3,6,m) is the first three columns of C(4,4,6). Table y1 is included to illustrate the rank-4 base codebooks.

Table y1 – C(4,4,6)

| Binary Index | # m |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | c1 | c2 | c3 | c4 |
| 000000 | 0 | 0.5000 0.5000i -0.5000 - 0.5000i | 0.5000 -0.5000 0.5000 -0.5000 | 0.5000 - 0.5000i -0.5000 0.5000i | 0.5000 0.5000 0.5000 0.5000 |
| 000001 | 1 | 0.5000 0.2357 - 0.4410i -0.4619 - 0.1913i -0.4619 - 0.1913i | 0.5000 0.4410 + 0.2357i 0.4619 + 0.1913i -0.1913 + 0.4619i | 0.5000 -0.2357 + 0.4410i -0.4619 - 0.1913i 0.4619 + 0.1913i | 0.5000 -0.4410 - 0.2357i 0.4619 + 0.1913i 0.1913 - 0.4619i |
| 000010 | 2 | 0.5000 -0.4157 + 0.2778i -0.3536 - 0.3536i -0.3536 + 0.3536i | 0.5000 -0.2778 - 0.4157i 0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 0.4157 - 0.2778i -0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 0.2778 + 0.4157i 0.3536 + 0.3536i -0.3536 - 0.3536i |
| 000011 | 3 | 0.5000 0.4976 - 0.0490i -0.1913 - 0.4619i 0.1913 + 0.4619i | 0.5000 0.0490 + 0.4976i 0.1913 + 0.4619i 0.4619 - 0.1913i | 0.5000 -0.4976 + 0.0490i -0.1913 - 0.4619i -0.1913 - 0.4619i | 0.5000 -0.0490 - 0.4976i 0.1913 + 0.4619i -0.4619 + 0.1913i |
| 000100 | 4 | 0.5000 -0.4619 - 0.1913i - 0.5000i 0.5000 | 0.5000 0.1913 - 0.4619i 0.5000i - 0.5000i | 0.5000 0.4619 + 0.1913i - 0.5000i -0.5000 | 0.5000 -0.1913 + 0.4619i 0.5000i 0.5000i |
| 000101 | 5 | 0.5000 0.3172 + 0.3865i 0.1913 - 0.4619i 0.1913 - 0.4619i | 0.5000 -0.3865 + 0.3172i -0.1913 + 0.4619i -0.4619 - 0.1913i | 0.5000 -0.3172 - 0.3865i 0.1913 - 0.4619i -0.1913 + 0.4619i | 0.5000 0.3865 - 0.3172i -0.1913 + 0.4619i 0.4619 + 0.1913i |
| 000110 | 6 | 0.5000 -0.0975 - 0.4904i 0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 0.4904 - 0.0975i -0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 0.0975 + 0.4904i 0.3536 - 0.3536i 0.3536 + 0.3536i | 0.5000 -0.4904 + 0.0975i -0.3536 + 0.3536i 0.3536 - 0.3536i |
| 000111 | 7 | 0.5000 -0.1451 + 0.4785i 0.4619 - 0.1913i -0.4619 + 0.1913i | 0.5000 -0.4785 - 0.1451i -0.4619 + 0.1913i 0.1913 + 0.4619i | 0.5000 0.1451 - 0.4785i 0.4619 - 0.1913i 0.4619 - 0.1913i | 0.5000 0.4785 + 0.1451i -0.4619 + 0.1913i -0.1913 - 0.4619i |
| 001000 | 8 | 0.5000 0.3536 - 0.3536i 0.5000 0.5000i | 0.5000 0.3536 + 0.3536i -0.5000 0.5000 | 0.5000 -0.3536 + 0.3536i 0.5000 - 0.5000i | 0.5000 -0.3536 - 0.3536i -0.5000 -0.5000 |
| 001001 | 9 | 0.5000 -0.4785 + 0.1451i 0.4619 + 0.1913i 0.4619 + 0.1913i | 0.5000 -0.1451 - 0.4785i -0.4619 - 0.1913i 0.1913 - 0.4619i | 0.5000 0.4785 - 0.1451i 0.4619 + 0.1913i -0.4619 - 0.1913i | 0.5000 0.1451 + 0.4785i -0.4619 - 0.1913i -0.1913 + 0.4619i |
| 001010 | 10 | 0.5000 0.4904 + 0.0975i 0.3536 + 0.3536i 0.3536 - 0.3536i | 0.5000 -0.0975 + 0.4904i -0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 -0.4904 - 0.0975i 0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 0.0975 - 0.4904i -0.3536 - 0.3536i 0.3536 + 0.3536i |
| 001011 | 11 | 0.5000 -0.3865 - 0.3172i 0.1913 + 0.4619i -0.1913 - 0.4619i | 0.5000 0.3172 - 0.3865i -0.1913 - 0.4619i -0.4619 + 0.1913i | 0.5000 0.3865 + 0.3172i 0.1913 + 0.4619i 0.1913 + 0.4619i | 0.5000 -0.3172 + 0.3865i -0.1913 - 0.4619i 0.4619 - 0.1913i |
| 001100 | 12 | 0.5000 0.1913 + 0.4619i 0.5000i -0.5000 | 0.5000 -0.4619 + 0.1913i - 0.5000i 0.5000i | 0.5000 -0.1913 - 0.4619i 0.5000i 0.5000 | 0.5000 0.4619 - 0.1913i - 0.5000i - 0.5000i |
| 001101 | 13 | 0.5000 0.0490 - 0.4976i -0.1913 + 0.4619i -0.1913 + 0.4619i | 0.5000 0.4976 + 0.0490i 0.1913 - 0.4619i 0.4619 + 0.1913i | 0.5000 -0.0490 + 0.4976i -0.1913 + 0.4619i 0.1913 - 0.4619i | 0.5000 -0.4976 - 0.0490i 0.1913 - 0.4619i -0.4619 - 0.1913i |
| 001110 | 14 | 0.5000 -0.2778 + 0.4157i -0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 -0.4157 - 0.2778i 0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 0.2778 - 0.4157i -0.3536 + 0.3536i -0.3536 - 0.3536i | 0.5000 0.4157 + 0.2778i 0.3536 - 0.3536i -0.3536 + 0.3536i |
| 001111 | 15 | 0.5000 0.4410 - 0.2357i -0.4619 + 0.1913i 0.4619 - 0.1913i | 0.5000 0.2357 + 0.4410i 0.4619 - 0.1913i -0.1913 - 0.4619i | 0.5000 -0.4410 + 0.2357i -0.4619 + 0.1913i -0.4619 + 0.1913i | 0.5000 -0.2357 - 0.4410i 0.4619 - 0.1913i 0.1913 + 0.4619i |
| 010000 | 16 | 0.5000 -0.5000 -0.5000 - 0.5000i | 0.5000 - 0.5000i 0.5000 -0.5000 | 0.5000 0.5000 -0.5000 0.5000i | 0.5000 0.5000i 0.5000 0.5000 |
| 010001 | 17 | 0.5000 0.4410 + 0.2357i -0.4619 - 0.1913i -0.4619 - 0.1913i | 0.5000 -0.2357 + 0.4410i 0.4619 + 0.1913i -0.1913 + 0.4619i | 0.5000 -0.4410 - 0.2357i -0.4619 - 0.1913i 0.4619 + 0.1913i | 0.5000 0.2357 - 0.4410i 0.4619 + 0.1913i 0.1913 - 0.4619i |
| 010010 | 18 | 0.5000 -0.2778 - 0.4157i -0.3536 - 0.3536i -0.3536 + 0.3536i | 0.5000 0.4157 - 0.2778i 0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 0.2778 + 0.4157i -0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 -0.4157 + 0.2778i 0.3536 + 0.3536i -0.3536 - 0.3536i |
| 010011 | 19 | 0.5000 0.0490 + 0.4976i -0.1913 - 0.4619i 0.1913 + 0.4619i | 0.5000 -0.4976 + 0.0490i 0.1913 + 0.4619i 0.4619 - 0.1913i | 0.5000 -0.0490 - 0.4976i -0.1913 - 0.4619i -0.1913 - 0.4619i | 0.5000 0.4976 - 0.0490i 0.1913 + 0.4619i -0.4619 + 0.1913i |
| 010100 | 20 | 0.5000 0.1913 - 0.4619i - 0.5000i 0.5000 | 0.5000 0.4619 + 0.1913i 0.5000i - 0.5000i | 0.5000 -0.1913 + 0.4619i - 0.5000i -0.5000 | 0.5000 -0.4619 - 0.1913i 0.5000i 0.5000i |
| 010101 | 21 | 0.5000 -0.3865 + 0.3172i 0.1913 - 0.4619i 0.1913 - 0.4619i | 0.5000 -0.3172 - 0.3865i -0.1913 + 0.4619i -0.4619 - 0.1913i | 0.5000 0.3865 - 0.3172i 0.1913 - 0.4619i -0.1913 + 0.4619i | 0.5000 0.3172 + 0.3865i -0.1913 + 0.4619i 0.4619 + 0.1913i |
| 010110 | 22 | 0.5000 0.4904 - 0.0975i 0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 0.0975 + 0.4904i -0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 -0.4904 + 0.0975i 0.3536 - 0.3536i 0.3536 + 0.3536i | 0.5000 -0.0975 - 0.4904i -0.3536 + 0.3536i 0.3536 - 0.3536i |
| 010111 | 23 | 0.5000 -0.4785 - 0.1451i 0.4619 - 0.1913i -0.4619 + 0.1913i | 0.5000 0.1451 - 0.4785i -0.4619 + 0.1913i 0.1913 + 0.4619i | 0.5000 0.4785 + 0.1451i 0.4619 - 0.1913i 0.4619 - 0.1913i | 0.5000 -0.1451 + 0.4785i -0.4619 + 0.1913i -0.1913 - 0.4619i |
| 011000 | 24 | 0.5000 0.3536 + 0.3536i 0.5000 0.5000i | 0.5000 -0.3536 + 0.3536i -0.5000 0.5000 | 0.5000 -0.3536 - 0.3536i 0.5000 -0.5000i | 0.5000 0.3536 - 0.3536i -0.5000 -0.5000 |
| 011001 | 25 | 0.5000 -0.1451 - 0.4785i 0.4619 + 0.1913i 0.4619 + 0.1913i | 0.5000 0.4785 - 0.1451i -0.4619 - 0.1913i 0.1913 - 0.4619i | 0.5000 0.1451 + 0.4785i 0.4619 + 0.1913i -0.4619 - 0.1913i | 0.5000 -0.4785 + 0.1451i -0.4619 - 0.1913i -0.1913 + 0.4619i |
| 011010 | 26 | 0.5000 -0.0975 + 0.4904i 0.3536 + 0.3536i 0.3536 - 0.3536i | 0.5000 -0.4904 - 0.0975i -0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 0.0975 - 0.4904i 0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 0.4904 + 0.0975i -0.3536 - 0.3536i 0.3536 + 0.3536i |
| 011011 | 27 | 0.5000 0.3172 - 0.3865i 0.1913 + 0.4619i -0.1913 - 0.4619i | 0.5000 0.3865 + 0.3172i -0.1913 - 0.4619i -0.4619 + 0.1913i | 0.5000 -0.3172 + 0.3865i 0.1913 + 0.4619i 0.1913 + 0.4619i | 0.5000 -0.3865 - 0.3172i -0.1913 - 0.4619i 0.4619 - 0.1913i |
| 011100 | 28 | 0.5000 -0.4619 + 0.1913i 0.5000i -0.5000 | 0.5000 -0.1913 - 0.4619i - 0.5000i 0.5000i | 0.5000 0.4619 - 0.1913i 0.5000i 0.5000 | 0.5000 0.1913 + 0.4619i -0.5000i -0.5000i |
| 011101 | 29 | 0.5000 0.4976 + 0.0490i -0.1913 + 0.4619i -0.1913 + 0.4619i | 0.5000 -0.0490 + 0.4976i 0.1913 - 0.4619i 0.4619 + 0.1913i | 0.5000 -0.4976 - 0.0490i -0.1913 + 0.4619i 0.1913 - 0.4619i | 0.5000 0.0490 - 0.4976i 0.1913 - 0.4619i -0.4619 - 0.1913i |
| 011110 | 30 | 0.5000 -0.4157 - 0.2778i -0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 0.2778 - 0.4157i 0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 0.4157 + 0.2778i -0.3536 + 0.3536i -0.3536 - 0.3536i | 0.5000 -0.2778 + 0.4157i 0.3536 - 0.3536i -0.3536 + 0.3536i |
| 011111 | 31 | 0.5000 0.2357 + 0.4410i -0.4619 + 0.1913i 0.4619 - 0.1913i | 0.5000 -0.4410 + 0.2357i 0.4619 - 0.1913i -0.1913 - 0.4619i | 0.5000 -0.2357 - 0.4410i -0.4619 + 0.1913i -0.4619 + 0.1913i | 0.5000 0.4410 - 0.2357i 0.4619 - 0.1913i 0.1913 + 0.4619i |
| 100000 | 32 | 0.5000 - 0.5000i -0.5000 - 0.5000i | 0.5000 0.5000 0.5000 -0.5000 | 0.5000 0.5000i -0.5000 0.5000i | 0.5000 -0.5000 0.5000 0.5000 |
| 100001 | 33 | 0.5000 -0.2357 + 0.4410i -0.4619 - 0.1913i -0.4619 - 0.1913i | 0.5000 -0.4410 - 0.2357i 0.4619 + 0.1913i -0.1913 + 0.4619i | 0.5000 0.2357 - 0.4410i -0.4619 - 0.1913i 0.4619 + 0.1913i | 0.5000 0.4410 + 0.2357i 0.4619 + 0.1913i 0.1913 - 0.4619i |
| 100010 | 34 | 0.5000 0.4157 - 0.2778i -0.3536 - 0.3536i -0.3536 + 0.3536i | 0.5000 0.2778 + 0.4157i 0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 -0.4157 + 0.2778i -0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 -0.2778 - 0.4157i 0.3536 + 0.3536i -0.3536 - 0.3536i |
| 100011 | 35 | 0.5000 -0.4976 + 0.0490i -0.1913 - 0.4619i 0.1913 + 0.4619i | 0.5000 -0.0490 - 0.4976i 0.1913 + 0.4619i 0.4619 - 0.1913i | 0.5000 0.4976 - 0.0490i -0.1913 - 0.4619i -0.1913 - 0.4619i | 0.5000 0.0490 + 0.4976i 0.1913 + 0.4619i -0.4619 + 0.1913i |
| 100100 | 36 | 0.5000 0.4619 + 0.1913i - 0.5000i 0.5000 | 0.5000 -0.1913 + 0.4619i 0.5000i - 0.5000i | 0.5000 -0.4619 - 0.1913i -0.5000i -0.5000 | 0.5000 0.1913 - 0.4619i 0.5000i 0.5000i |
| 100101 | 37 | 0.5000 -0.3172 - 0.3865i 0.1913 - 0.4619i 0.1913 - 0.4619i | 0.5000 0.3865 - 0.3172i -0.1913 + 0.4619i -0.4619 - 0.1913i | 0.5000 0.3172 + 0.3865i 0.1913 - 0.4619i -0.1913 + 0.4619i | 0.5000 -0.3865 + 0.3172i -0.1913 + 0.4619i 0.4619 + 0.1913i |
| 100110 | 38 | 0.5000 0.0975 + 0.4904i 0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 -0.4904 + 0.0975i -0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 -0.0975 - 0.4904i 0.3536 - 0.3536i 0.3536 + 0.3536i | 0.5000 0.4904 - 0.0975i -0.3536 + 0.3536i 0.3536 - 0.3536i |
| 100111 | 39 | 0.5000 0.1451 - 0.4785i 0.4619 - 0.1913i -0.4619 + 0.1913i | 0.5000 0.4785 + 0.1451i -0.4619 + 0.1913i 0.1913 + 0.4619i | 0.5000 -0.1451 + 0.4785i 0.4619 - 0.1913i 0.4619 - 0.1913i | 0.5000 -0.4785 - 0.1451i -0.4619 + 0.1913i -0.1913 - 0.4619i |
| 101000 | 40 | 0.5000 -0.3536 + 0.3536i 0.5000 0.5000i | 0.5000 -0.3536 - 0.3536i -0.5000 0.5000 | 0.5000 0.3536 - 0.3536i 0.5000 -0.5000i | 0.5000 0.3536 + 0.3536i -0.5000 -0.5000 |
| 101001 | 41 | 0.5000 0.4785 - 0.1451i 0.4619 + 0.1913i 0.4619 + 0.1913i | 0.5000 0.1451 + 0.4785i -0.4619 - 0.1913i 0.1913 - 0.4619i | 0.5000 -0.4785 + 0.1451i 0.4619 + 0.1913i -0.4619 - 0.1913i | 0.5000 -0.1451 - 0.4785i -0.4619 - 0.1913i -0.1913 + 0.4619i |
| 101010 | 42 | 0.5000 -0.4904 - 0.0975i 0.3536 + 0.3536i 0.3536 - 0.3536i | 0.5000 0.0975 - 0.4904i -0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 0.4904 + 0.0975i 0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 -0.0975 + 0.4904i -0.3536 - 0.3536i 0.3536 + 0.3536i |
| 101011 | 43 | 0.5000 0.3865 + 0.3172i 0.1913 + 0.4619i -0.1913 - 0.4619i | 0.5000 -0.3172 + 0.3865i -0.1913 - 0.4619i -0.4619 + 0.1913i | 0.5000 -0.3865 - 0.3172i 0.1913 + 0.4619i 0.1913 + 0.4619i | 0.5000 0.3172 - 0.3865i -0.1913 - 0.4619i 0.4619 - 0.1913i |
| 101100 | 44 | 0.5000 -0.1913 - 0.4619i 0.5000i -0.5000 | 0.5000 0.4619 - 0.1913i - 0.5000i 0.5000i | 0.5000 0.1913 + 0.4619i 0.5000i 0.5000 | 0.5000 -0.4619 + 0.1913i -0.5000i -0.5000i |
| 101101 | 45 | 0.5000 -0.0490 + 0.4976i -0.1913 + 0.4619i -0.1913 + 0.4619i | 0.5000 -0.4976 - 0.0490i 0.1913 - 0.4619i 0.4619 + 0.1913i | 0.5000 0.0490 - 0.4976i -0.1913 + 0.4619i 0.1913 - 0.4619i | 0.5000 0.4976 + 0.0490i 0.1913 - 0.4619i -0.4619 - 0.1913i |
| 101110 | 46 | 0.5000 0.2778 - 0.4157i -0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 0.4157 + 0.2778i 0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 -0.2778 + 0.4157i -0.3536 + 0.3536i -0.3536 - 0.3536i | 0.5000 -0.4157 - 0.2778i 0.3536 - 0.3536i -0.3536 + 0.3536i |
| 101111 | 47 | 0.5000 -0.4410 + 0.2357i -0.4619 + 0.1913i 0.4619 - 0.1913i | 0.5000 -0.2357 - 0.4410i 0.4619 - 0.1913i -0.1913 - 0.4619i | 0.5000 0.4410 - 0.2357i -0.4619 + 0.1913i -0.4619 + 0.1913i | 0.5000 0.2357 + 0.4410i 0.4619 - 0.1913i 0.1913 + 0.4619i |
| 110000 | 48 | 0.5000 0.5000 -0.5000 -0.5000i | 0.5000 0.5000i 0.5000 -0.5000 | 0.5000 -0.5000 -0.5000 0.5000i | 0.5000 -0.5000i 0.5000 0.5000 |
| 110001 | 49 | 0.5000 -0.4410 - 0.2357i -0.4619 - 0.1913i -0.4619 - 0.1913i | 0.5000 0.2357 - 0.4410i 0.4619 + 0.1913i -0.1913 + 0.4619i | 0.5000 0.4410 + 0.2357i -0.4619 - 0.1913i 0.4619 + 0.1913i | 0.5000 -0.2357 + 0.4410i 0.4619 + 0.1913i 0.1913 - 0.4619i |
| 110010 | 50 | 0.5000 0.2778 + 0.4157i -0.3536 - 0.3536i -0.3536 + 0.3536i | 0.5000 -0.4157 + 0.2778i 0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 -0.2778 - 0.4157i -0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 0.4157 - 0.2778i 0.3536 + 0.3536i -0.3536 - 0.3536i |
| 110011 | 51 | 0.5000 -0.0490 - 0.4976i -0.1913 - 0.4619i 0.1913 + 0.4619i | 0.5000 0.4976 - 0.0490i 0.1913 + 0.4619i 0.4619 - 0.1913i | 0.5000 0.0490 + 0.4976i -0.1913 - 0.4619i -0.1913 - 0.4619i | 0.5000 -0.4976 + 0.0490i 0.1913 + 0.4619i -0.4619 + 0.1913i |
| 110100 | 52 | 0.5000 -0.1913 + 0.4619i - 0.5000i 0.5000 | 0.5000 -0.4619 - 0.1913i 0.5000i - 0.5000i | 0.5000 0.1913 - 0.4619i -0.5000i -0.5000 | 0.5000 0.4619 + 0.1913i 0.5000i 0.5000i |
| 110101 | 53 | 0.5000 0.3865 - 0.3172i 0.1913 - 0.4619i 0.1913 - 0.4619i | 0.5000 0.3172 + 0.3865i -0.1913 + 0.4619i -0.4619 - 0.1913i | 0.5000 -0.3865 + 0.3172i 0.1913 - 0.4619i -0.1913 + 0.4619i | 0.5000 -0.3172 - 0.3865i -0.1913 + 0.4619i 0.4619 + 0.1913i |
| 110110 | 54 | 0.5000 -0.4904 + 0.0975i 0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 -0.0975 - 0.4904i -0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 0.4904 - 0.0975i 0.3536 - 0.3536i 0.3536 + 0.3536i | 0.5000 0.0975 + 0.4904i -0.3536 + 0.3536i 0.3536 - 0.3536i |
| 110111 | 55 | 0.5000 0.4785 + 0.1451i 0.4619 - 0.1913i -0.4619 + 0.1913i | 0.5000 -0.1451 + 0.4785i -0.4619 + 0.1913i 0.1913 + 0.4619i | 0.5000 -0.4785 - 0.1451i 0.4619 - 0.1913i 0.4619 - 0.1913i | 0.5000 0.1451 - 0.4785i -0.4619 + 0.1913i -0.1913 - 0.4619i |
| 111000 | 56 | 0.5000 -0.3536 - 0.3536i 0.5000 0.5000i | 0.5000 0.3536 - 0.3536i -0.5000 0.5000 | 0.5000 0.3536 + 0.3536i 0.5000 -0.5000i | 0.5000 -0.3536 + 0.3536i -0.5000 -0.5000 |
| 111001 | 57 | 0.5000 0.1451 + 0.4785i 0.4619 + 0.1913i 0.4619 + 0.1913i | 0.5000 -0.4785 + 0.1451i -0.4619 - 0.1913i 0.1913 - 0.4619i | 0.5000 -0.1451 - 0.4785i 0.4619 + 0.1913i -0.4619 - 0.1913i | 0.5000 0.4785 - 0.1451i -0.4619 - 0.1913i -0.1913 + 0.4619i |
| 111010 | 58 | 0.5000 0.0975 - 0.4904i 0.3536 + 0.3536i 0.3536 - 0.3536i | 0.5000 0.4904 + 0.0975i -0.3536 - 0.3536i -0.3536 - 0.3536i | 0.5000 -0.0975 + 0.4904i 0.3536 + 0.3536i -0.3536 + 0.3536i | 0.5000 -0.4904 - 0.0975i -0.3536 - 0.3536i 0.3536 + 0.3536i |
| 111011 | 59 | 0.5000 -0.3172 + 0.3865i 0.1913 + 0.4619i -0.1913 - 0.4619i | 0.5000 -0.3865 - 0.3172i -0.1913 - 0.4619i -0.4619 + 0.1913i | 0.5000 0.3172 - 0.3865i 0.1913 + 0.4619i 0.1913 + 0.4619i | 0.5000 0.3865 + 0.3172i -0.1913 - 0.4619i 0.4619 - 0.1913i |
| 111100 | 60 | 0.5000 0.4619 - 0.1913i 0.5000i -0.5000 | 0.5000 0.1913 + 0.4619i - 0.5000i 0.5000i | 0.5000 -0.4619 + 0.1913i 0.5000i 0.5000 | 0.5000 -0.1913 - 0.4619i -0.5000i - 0.5000i |
| 111101 | 61 | 0.5000 -0.4976 - 0.0490i -0.1913 + 0.4619i -0.1913 + 0.4619i | 0.5000 0.0490 - 0.4976i 0.1913 - 0.4619i 0.4619 + 0.1913i | 0.5000 0.4976 + 0.0490i -0.1913 + 0.4619i 0.1913 - 0.4619i | 0.5000 -0.0490 + 0.4976i 0.1913 - 0.4619i -0.4619 - 0.1913i |
| 111110 | 62 | 0.5000 0.4157 + 0.2778i -0.3536 + 0.3536i 0.3536 + 0.3536i | 0.5000 -0.2778 + 0.4157i 0.3536 - 0.3536i 0.3536 - 0.3536i | 0.5000 -0.4157 - 0.2778i -0.3536 + 0.3536i -0.3536 - 0.3536i | 0.5000 0.2778 - 0.4157i 0.3536 - 0.3536i -0.3536 + 0.3536i |
| 111111 | 63 | 0.5000 -0.2357 - 0.4410i -0.4619 + 0.1913i 0.4619 - 0.1913i | 0.5000 0.4410 - 0.2357i 0.4619 - 0.1913i -0.1913 - 0.4619i | 0.5000 0.2357 + 0.4410i -0.4619 + 0.1913i -0.4619 + 0.1913i | 0.5000 -0.4410 + 0.2357i 0.4619 - 0.1913i 0.1913 + 0.4619i |

- 15.3.10.3.2.2 MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as the base codebook for UL SU-MIMO, defined in 15.3.10.3.1.1.1.2.1.

-------------------------------  Text End  ---------------------------------------------------