| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Issues related to Idle Mode in 16m |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | Category: AWD-DG Comment Area: Power Conservative-Sleep/Idle DG |  |
| Abstract | This contribution proposes remedies for issues related to idle mode operations in 16m |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Issues related to Idle Mode in 16m

Stavros Tzavidas

Motorola

# Introduction

The output of the power conservation draft group on idle mode operations is described in contribution IEEE C802.16m-09/0798. In this contribution we propose changes to the baseline text therein.

The text specifies that during idle mode entry a temporary identifier is assigned to the AMS but does not provide any description of how this temporary identifier is calculated.

In this contribution we propose to reuse the mechanisms already present in 802.16e to identify an AMS in idle mode. This allows maximum reuse of existing implementations while achieving the stated goals of 802.16m to reduce overhead, complexity and also to provide location privacy to idle mode AMS.

# Text Proposal

Modify the text in IEEE C802.16m-09/0798 (power conservation DG output) as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

- 15.2.x.1 Idle mode initiation

Idle mode for an AMS can be initiated either by the AMS or by its serving ABS.

Temporary identifier is assigned to an AMS when idle mode is initiated. A temporary identifier is used to uniquely identify an AMS within a paging group.

The temporary identifier shall be the result of feeding the string "AMS MAC address | 8-bit nonce | PG-ID" into the hash function defined in section 6.3.2.3.51 (see definition of "MS MAC Address hash"), where the 8-bit nonce is assigned by the ABS and sent to the AMS during idle mode entry or during location update. The AMS Mac Address hash identifier may be used in paging messages or during AMS’s network re-entry procedure from idle mode as response to paging.

- 15.2.x.1. 1 AMS initiated

In case of AMS initiated idle mode entry, during normal operation with its serving ABS, an AMS may signal intent to begin idle mode by sending a DREG-REQ message with the De-registration_Request_Code parameter = 0x01; request for AMS deregistration from serving ABS and initiation of AMS idle mode. The MS may request the paging controller to …

-------------------------------------------------- End of the text proposal --------------------------------------------------