| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Multi-carrier PHY Operation for the IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jong-kae Fwu, Kamran Etemad Intel Corporation | Jong-kae.Fwu@intel.com Kamran.etemad@intel.com |
| Re: | “802.16m AWD”: IEEE 802.16m-09/xxxx, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “19 Multi-carrier” |  |
| Abstract | The contribution proposes the text of multi-carrier operation section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Multi-carrier PHY Operation 
for the IEEE 802.16m AWD

Jong-kae Fwu, Kamran Etemad

Intel  Corporation

- Introduction

This contribution proposes amendment text to describe the multi-carrier operation in 802.16m and to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. Current SDD [3] is used as a basis for the text development. In Section 2, the high level outline of the multi-carrier operation is presented.

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r1a, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – Advanced Air Interface (working document)”

- Text proposal for inclusion in the 802.16m amendment working document

-------------------------------  Text Start  ---------------------------------------------------

- Advanced Air Interface

  - Medium access control

[Insert the followings at the subclause 15.2:]

……

15.2.x.3 Physical layer operation

15.2.x.3.1 Data Transmission over Guard Resource

The use of guard sub-carriers for multi-carrier operation shall not affect other single carrier devices operation in the same network. On the basic multi-carrier mode, the PHY symbol structure, permutation and the number of used subcarriers in each carrier is the same as in single carrier mode. Optionally, each carrier can exploit subcarriers at band edges as its additional data subcarriers. The guard resource forms integer multiples of PRUs. The PRU structure used for guard resource is the same as the structure of the ordinary PRU in 15.3.5 and 15.3.7. When optionally used, the guard subcarriers are used for miniband CRU. The ABS provides information regarding the use of guard resource for data channels. Guard resource is not used for control channels transmission.

15.2.x.3.4 DL control channel

All DL controls channel needed for single carrier operation are needed for the fully configured carrier. For partially configured carrier, DL control channels necessary for UL transmission are not required.

Obtaining System Information of Secondary Carriers

- For the case where the AMS can simultaneously decode multiple carriers, the AMS can decode the Superframe Headers and Extended system information of its secondary carriers. ABS may instruct the AMS, through control signaling on the primary carrier, to decode Superframe Headers of specific set of secondary carriers.

- When the the AMS cannot simultaneously decode multiple carriers, the ABS conveys the system information of secondary carriers to AMS, through control signaling on the primary carrier.

15.2.x.3.5 UL control channel

All UL controls channel needed for single carrier operation are supported for the fully configured carrier. A partially configured carrier does not have any uplink capability as it is optimized for downlink only transmissions such as multicast and broadcast services.

- Secondary carrier may only assign UL control channels which are needed for PHY support, such as CQI, ACK/NAK, power control.15.3Physical layer

[Insert the new section as follows]

15.3.5.5 Downlink physical structure for multicarrier support

For multicarrier operation, guard subcarriers between carriers can be optionally used to form additional PRUs. The definition of PRU is the same as defined in 15.3.5.1 and 15.3.5.4. The additional PRUs only can be used as miniband CURs for partition FP0. The number of useable guard subcarriers is predefined and should be known to both AMS and ABS (it may depends on bandwidth).

15.3.8.6 Uplink physical structure for multicarrier support

For multicarrier operation, guard subcarriers between carriers can be optionally used to form additional PRUs. The definition of PRU is the same as defined in 15.3.8.1 and 15.3.8.4. The additional PRUs only can be used as miniband CURs for partition FP0. The number of useable guard subcarriers is predefined and should be known to both AMS and ABS (it may depends on bandwidth).

-------------------------------  Text End  ---------------------------------------------------