| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Multi-carrier MAC Operation for the IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Kamran Etemad, Jong-kae Fwu, Intel Corporation | Kamran.etemad@intel.com |
| Re: | “802.16m AWD”: IEEE 802.16m-09/xxxx, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “19 Multi-carrier” |  |
| Abstract | The contribution proposes the text of multi-carrier operation section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Multi-carrier MAC Operation 
for the IEEE 802.16m AWD

Kamran Etemad

Intel  Corporation

- Introduction

This contribution proposes amendment text to describe the multi-carrier operation in 802.16m and to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. Current SDD [3] is used as a basis for the text development. In Section 2, the high level outline of the multi-carrier operation is presented.

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r1a, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – Advanced Air Interface (working document)”

- Text proposal for inclusion in the 802.16m amendment working document

-------------------------------  Text Start  ---------------------------------------------------

- Advanced Air Interface

  - Medium access control

[Insert the followings at the subclause 15.2:]

    - Multi-carrier operation

15.2.x.1 Multi-carrier Types and Operational Modes

The carriers involved in multi-carrier mode of operation from an AMS point of view are of two types:

• A primary carrier is a carrier used by the ABS to exchange traffic and PHY/MAC control signaling (e.g., MAC management messages) with an AMS.  An ABS may be deployed with multiple carriers but each AMS in the ABS has only one primary carrier which is also used when AMS is operating in single carrier mode.

• Secondary carriers are additional carriers which the AMS may use for traffic, only per ABS’s specific commands and rules received on the primary carrier.

In the multicarrier operation a common MAC can utilize radio resources in one or more of the secondary carriers, while maintaining full control of AMS mobility, state and context through the primary carrier.

Based on the primary and/or secondary usage and target services, the carriers of a multi-carrier system may be configured differently as follows:

- Fully configured carrier: A standalone carrier for which all control channels including synchronization, broadcast, multicast and unicast control signaling are configured. Fully configured carrier supports both single carrier AMS and multi-carrier AMS.

- Partially configured carrier: A carrier configured for downlink only transmission in TDD or a downlink carrier without paired UL carrier in FDD mode. . Such supplementary carriers may be used only in conjunction with a primary carrier and cannot operate standalone to offer IEEE 802.16m services for an AMS. Whether a carrier is fully configured or partially configured is indicated using Advanced Preamble of the carrier. The AMS shall not attempt network entry or handover to partially configured carrier

A primary carrier is fully configured while a secondary carrier may be fully or partially configured depending on deployment scenarios. A secondary carrier for an AMS, if fully configured, may serve as primary carrier for other AMS’s. Multiple AMSs, each with a different primary RF carrier may also share the same secondary carrier. The following multi-carrier operation modes are identified, which may all or independently be supported:

- Multi-Carrier Aggregation: The multicarrier mode in which the AMS maintains its physical layer connection and monitors the control signaling on the primary carrier while processing data on the secondary carrier. The resource allocation to an AMS may span across a primary and multiple secondary RF carriers. Link adaptation feedback mechanisms should incorporate measurements relevant to both primary and secondary carriers. In this mode the system may assign secondary carriers to an AMS in the downlink and/or uplink asymmetrically based on system load (i.e., for static/dynamic load balancing), peak data rate, or QoS demand.

- Multi-Carrier Switching: The multicarrier mode in which the AMS switches its physical layer connection from the primary to the secondary carrier per ABS’ instruction. The AMS connects with the secondary carrier for the specified time period and then returns to the primary carrier. When the AMS is connected to the secondary carrier, the AMS is not required to maintain its physical layer connection to the primary carrier. This mode is used for switching to partially configured carriers for downlink transmission only service.

- Primary Carrier Change: The multicarrier mode in which the ABS changes the primary carrier of an AMS. This mode is similar to a fast inter-carrier handover where only the PHY changes while maintaining AMS control on the same MAC. This mode of multicarrier is required by all AMS’s.

The following is common to all multi-carrier modes of operation:

- The system defines N standalone fully configured RF carriers; each fully configured with all synchronization, broadcast, multicast and unicast control signaling channels. Each AMS in the cell is connected to and its state being controlled through only one of the fully configured carriers designated as its primary carrier.

- The system may also define M (M >= 0) partially configured RF carriers, which can only be used as secondary carriers along with a primary carrier,  for downlink only data transmissions.

- The set of all supported radio carriers in an ABS is called Available Carriers.

- The multiple carriers may be in different parts of the same spectrum block or in non-contiguous spectrum blocks. Support of non-contiguous spectrum blocks may require additional control information on the secondary carriers.

- In addition to information about the (serving) primary carrier an ABS, supporting any multicarrier mode, also provides AMSs with some basic information about other available carriers through such primary carrier. The basic multicarrier configuration informs AMS’s of the presence, bandwidth, duplexing, and location in the spectrum for all available carriers to help AMS prepare for any multicarrier operation. The primary carrier may also provide an AMS the extended information about the configuration of the secondary carrier.

15.2.x.2 MAC operation

15.2.x.2.1 Addressing

A multi-carrier supporting ABS or AMS follows the same MAC addressing mechanism defined in 15.2.1[5].

15.2.x.2.2 Security

A multi-carrier supporting AMS follows the same security procedure defined in 10.6[3]. All the security procedures between an AMS and an ABS are performed using the AMS’s primary carrier. The security context created and maintained by the procedures is managed per ABS through the primary carrier.

15.2.x.2.3 Network Entry

An AMS can only perform network entry (or network re-entry) procedures with a fully configured carrier. By detecting A-PREAMBLE designated for partially configured carriers the AMS skips that carrier and proceed with scanning and selection of alternative carrier. Once the AMS detects the A-PREAMBLE on a fully configured carrier, the AMS may proceed with reading SFH or Extended system parameters and system configuration information where the ABS indicates its configuration, and its support for multi-carrier feature.

The AMS can decide on proceeding with network entry with the current carrier or going to alternative carriers based on this information. The carrier selected for network entry is the AMS’s primary carrier unless redirected by the ABS to another carrier. The initial network entry/re-entry follows the procedures defined in 10.8[3] with the exception of operations described in below.

The ABS may indicate if it supports any of multicarrier modes using a flag in the BCH. The ABS also provides AMS’s with basic radio configuration for other available carriers in the ABS through a MAC management message called MC-CONFIG-ADV. This message is periodically broadcast by BS, supporting any multicarrier mode and it may also be unicast to AMS if needed. The same configuration information may also be unicast per AMS request in TBD message. The multi-carrier configuration information is relevant to and may be used by all AMS’s in any of multicarrier modes.

The MC-CONFIG-ADV includes information such as center frequency, duplexing mode, bandwidth and other parameters if different than the serving carrier and it also assigns each carrier a physical channel index, which is used by ABS and AMS for any reference to any available carrier. The configuration information about each carrier is listed if different than the serving carrier. Each carrier is also assigned a physical carrier index which is used for any subsequent reference to this carrier. The configuration information in MC-CONFIG-ADV message is applicable to all MS’s supporting any of multicarrier modes.

The following information shall be included in MC-CONFIG-ADV

- Physical Carrier Index

- Band Class Index

- Channel number or Center Frequency

- Channel Bandwidth

- Carrier Type (fully/partially configured)

- Duplexing Mode

- Preamble Index

The Details Structure of MC-CONFIG-ADV is TBD.

After successful ranging, the AMS follows the capability negotiation procedure. During pre-authentication capability negotiation, the AMS and ABS shall exchange their basic multi-carrier capabilities, such as supported multicarrier modes, the number of supportable RF carriers in the downlink and uplink and maximum throughput.  These Capabilities can be negotiated via post authentication as they do not affect network entry and operation in basic single carrier mode. Such Capability negotiation can be exchanges using REG-REQ/RSP messages or other MAC management messages specially designed for these parameters.

Following the capability negotiation, the ABS may assign secondary carrier(s) to the AMS using MC-SEC-ASMNT Management message if required by QoS.   This information can also be encoded and provided to MS as part of capability negotiation if REG-REQ/RSP is used. The MS is not expected to perform any MAC/PHY processing of the assigned carriers until it receive a MC-IND message activating such assigned carrier as described later in this document.

The AMS may not be able to communicate with the ABS over the secondary carrier(s), if it needs ranging to adjust time/frequency synchronization and power for the carrier(s). Under the assumption of high channel correlation between the carriers, the transmission parameters of secondary carrier(s) could be quite similar with those of primary carrier. Since the AMS already completed the network entry with the ABS over the primary carrier, it does not need to perform the initial ranging over the secondary carrier(s) again to acquire its STID, etc. Therefore, over the secondary carrier(s), the periodic ranging instead of initial ranging may be performed. So once secondary carriers are assigned, the AMS may perform the periodic ranging over the assigned secondary carrier(s) if directed by the ABS.

When the AMS omit the ranging for the secondary carrier(s), the AMS uses the same timing, frequency and power adjustment parameters for the secondary carrier(s) as in the primary carrier. The AMS may perform the fine timing, frequency and power adjustment on the secondary carrier(s) through measuring the synch channel and/or pilot on the secondary carrier(s).

15.2.x.2.4 Ranging

CDMA initial ranging with an AMS’s primary carrier shall be the same as defined in 6.3.10.3.1[1].

CDMA periodic ranging may be performed on primary carrier following the same procedure defined in 6.3.10.3.2[1]. Periodic ranging may only be performed on the assigned secondary carrier(s) if directed by the ABS.

CDMA handover ranging shall be done only with the AMS’s primary carrier. Upon moving to a target ABS, the AMS shall initiate CDMA handover ranging as defined in section 15.2.5.2.5.1[5].

15.2.x.2.5 MPDU processing

The construction and transmission of MAC PDU is the same as that in single carrier operation.

For each service flow the ARQ operates for a common MAC as defined in 10.4[3].

15.2.x.2.6 Bandwidth Request and Resource Allocation

All bandwidth requests are transmitted on the AMS’s primary carrier using the assigned bandwidth request channel following the same procedures as defined in 15.2.2[5]. Bandwidth request using piggyback may be transmitted in MPDUs over the secondary carrier(s) as well as the primary carrier.

When a bandwidth request is received, the ABS may allocate downlink or uplink resources which belong to a specific carrier or a combination of multiple carriers based on available resources, QoS requirements and other factorsThe multicarrier resource assignment for carrier aggregation can use the same A-MAP IE’s as single carrier mode, where A-MAP messages for each active carrier are transmitted in the respective carrier.

[Alternatively a multicarrier AMAP-IE may be used and transmitted on the primary carrier to allocate resources across primary and secondary carriers. The use and details structure of AMAP-IE’s optimized for multicarrier allocations is TBD. ]

15.2.x.2.7 QoS and connection management

The STID and all FIDs assigned to an AMS are unique identifiers for a common MAC and used over all the carriers of the AMS. The service setup/change messages (i.e., DSx messages) are transmitted only through the AMS’s primary carrier. The service flow is defined for a common MAC entity and AMS’s QoS context represented by an SFID is applicable across primary carrier and secondary carrier(s) and collectively applied to all carriers of the AMS.

15.2.x.2.8 DL CINR report operation

An ABS may assign CQI channels to each carrier of an AMS. When CQI channel is assigned, the AMS reports CINR for a carrier over the assigned CQI channel of the corresponding carrier. When REP-REQ/RSP messages are used for DL CINR report operation, the messages are transmitted on the AMS’s primary carrier. The REP-RSP message may contain CINR reports for all carriers or for each carrier of the AMS.

The MS may be directed not to send CINR measurement for secondary carriers if their signal quality is lower than a threshold, in which case the BS would refrain from sending data on such secondary carrier until signal quality is improved above the thresholds and the MS restarts sending CINR measurements on that secondary carrier.

15.2.x.2.9 Handover

15.2.x.2.9.1 Network topology acquisition

In a network with multiple carriers are deployed at ABS’s, regardless of support for multicarrier feature, the ABS’s broadcast the AAI-NBR-ADV message which includes the configuration parameters of the carriers of neighboring ABS’s. Each AMS receives the AAI-NBR-ADV message on its primary carrier.

The scanning procedure of a multi-carrier capable AMS follows the same procedure in 15.2.5.1.2[5]. When scanning interval is explicitly negotiated, AAI-SCN-REQ/RSP messages are transmitted with the primary carrier of the AMS.

15.2.x.2.9.2 HO process

An AMS follows the handover process defined in 15.2.5.2[5] All MAC management messages in relation with handover are transmitted over the primary carrier.

During HO preparation phase, when an ABS receives AAI-HO-REQ message from an AMS or the ABS sends AAI-HO-CMD message to the AMS, the ABS may exchange the information of multiple carrier capability with possible target ABSs. The information includes the AMS’s available number of carriers, carrier aggregation or carrier switching capability.

When the information regarding AMS identify (e.g., STID) or security context at a target ABS is pre-updated during HO preparation, the multi-carrier related information (e.g., secondary carrier) may be provided to the AMS using AAI-HO-CMD message.

After handover to a target ABS is determined, an AMS conducts its network re-entry with a selected carrier which becomes the AMS’s primary carrier at the target ABS. If a secondary carrier is not provided to the AMS in advance, the target ABS may allocate the secondary carrier during network re-entry or the connection may change to single carrier mode.

15.2.x.2.10 Sleep mode

When an AMS enters sleep mode, the AMS negotiates its sleep mode parameters (i.e., sleep window and listening window configuration) with an ABS. The negotiated parameters of sleep mode are applied to an AMS and all carriers power down according to the negotiated sleep mode parameters. The messages and procedures before entering sleep mode and during sleep mode are processed over the primary carrier.

During the listening window, the AMS monitors the traffic indication message with its primary carrier. Upon receiving negative traffic indication in the traffic indication message, the AMS goes back to sleep mode. If positive traffic indication is received, the AMS continues to monitor the primary carrier. Data transmission follows the normal operation for multi-carrier mode.

15.2.x.2.11 Idle mode

A multi-carrier supporting AMS in idle state follows the same procedures defined in 10.5.2[3]. The carrier selected by such AMS, using procedures similar to single carrier mode, is the default Primary carrier to be monitored by the AMS’s for paging and other broadcast control messages.

Messages and procedures to initiate the idle mode are processed with AMS’s primary carrier. During paging listening interval, the AMS monitors paging message on its Primary carrier consistent with single carrier mode.  When paged, the AMS performs network re-entry procedure with the primary carrier.

15.2.x.2.12 Secondary Carrier management

The basic configuration information of all available carriers is provided through MC-CONFIG–ADV message which broadcast by ABS and is typically received by the AMS before network entry.

Following the network entry, and based on AMS’s multicarrier capabilities, the ABS may assign one or more carriers from its available carriers to an AMS as Assigned secondary carriers, through [REG-RSP/MC-SEC_ASNMT] message. The message refers to carriers using their physical carrier index based on MC-CONFIG–ADV and each assigned carrier is given implicitly a logical carrier index based on the order of its appearance in the [REG-RSP/MC-SEC_ASNMT] message. [If the AMS has not received the basic multicarrier configuration information through MC-CONFIG-ADV,  it may request this information to be included in the [REG-RSP/MC-SEC_ASNMT] message. ]

The following multicarrier information shall be included in [REG-RSP/MC-SEC_ASNMT] message on the primary carrier:

- Physical Carrier index

- Preamble Index

- Static Scanning Rules

[The use of a TLV in REG-RSP or the MC-SEC-ASNMT message and their structure are TBD. ]

The AMS does not start PHY/MAC processing of secondary carriers until it receives a MC-IND message from ABS, where ABS specified one or more of assigned carrier to be actively scanned and monitored as secondary carriers for upcoming multi carrier allocations as well as other scanning directions and timers. MC-IND message may also be used to change the secondary carriers or stop multicarrier processing at the AMS.

The activation or deactivation of secondary carrier(s) is decided by ABS based on QoS requirement, load condition of carriers and other factors. The ABS may transmit the number of active carriers to the AMS as a QoS parameter via MC-IND messages.

The MC-IND message is transmitted on the primary carrier and shall include the following informations:

- Indication Type: (start, extend, change, stop)

- List of Secondary Carriers: (referred by logical carrier index)

- Dynamic Scanning Rules if different than static rules defined in [REG-RSP/MC-SEC_ASNMT] message

- Combined Feedback on Primary Carrier (On/Off)

- Scope: STID or FID. If STID is used MC mode may apply to any of services/connections for that AMS.

The use of a TLV in DSx and/or a new MC-IND message for providing these functionalities and the structure such messages/TLV’s are TBD.

The ABS may allocate a fast feedback channel for each newly activated secondary carrier on the MC-IND message. After the ABS receives the fast feedback channel(s) allocated for newly activated carrier(s), ABS may start transmitting data on the active secondary carrier(s). The active secondary carrier(s) shall be in connected mode to receive or to transmit the data burst. AMS may report CQI for all downlink active secondary carriers. AMS may perform periodic ranging for the uplink active secondary carriers. The sounding channel may be transmitted by AMS when directed by ABS for uplink active secondary carriers. The AMS may update the system information of all active secondary carriers and may monitor resource allocation messages for all active secondary carrier(s).

* MC Capability support is a flag which is set to 0 none of MC modes are supported

** The use of other messages for capability negotiation post authentication is TBD

*** The use of DSx messages instead of MC Indication is TBD

Figure xxx: Initiating and terminating Multicarrier Processing at the AMS

15.2.x.2.13 Primary Carrier Change

The ABS may instruct the AMS, through control signaling on the current primary carrier, to change its primary carrier to one of the available fully configured carriers within the same ABS for load balancing purpose, carriers’ varying channel quality or other reasons. AMS switches to the target fully configured carrier at action time specified by the ABS. The carrier change may also be requested by the AMS through control signaling on the current primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. ABS may provide the system information of the target primary carrier that is different from the serving primary carrier via the serving primary carrier.  ABS may allocate new STID when the current STID is already served by other AMS. The logical carrier indices may be re-arranged. ABS may direct an AMS to change the primary carrier without scanning.

The ABS may instruct AMS to perform scanning on other carriers which are not serving the AMS. AMS reports the scanning results back to the serving ABS, which may be used by the ABS to determine the carrier for the AMS to switch to. In this case, if the target carrier is not currently serving the AMS, the AMS may perform synchronization with the target carrier if required.

15.2.x.2.13 Carrier switching mode

Primary to secondary carrier switching in multi-carrier mode is supported when secondary carrier is partially configured. The carrier switching between a primary carrier and a secondary carrier can be periodic or event-triggered with timing parameters defined by multi-carrier switching message on the primary carrier.  When an AMS switches to a secondary carrier, its primary carrier may provide basic information such as timing and frequency adjustment to help with AMS’s with fast synchronization with the secondary carrier.