| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Assignment AMAP Structure in the IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Yi Hsuan, Hujun Yin Intel Corporation Jinsoo Choi, Sunam Kim, Bin-chul Ihm, Jin Sam Kwak LGE Hyunkyu Yu, Jason Junsung Lim, Heewon Kang, Hokyu Choi Samsung | yi.hsuan@intel.com emptylie@lge.com hk.yu@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes updates to the text related to AMAP in the 802.16m AWD |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Assignment A-MAP Structure in the IEEE 802.16m AWD

Introduction

The current AWD text has the following definitions regarding assignment AMAP

- Each AMAP IE is separately coded.

- MLRU is the minimum resource unit for assignment AMAP.

- Two MCS levels can be used for assignment AMAP.

- Non-user specific information indicates the size of assignment AMAP.

However the structure of assignment AMAP to allow efficient decoding is missing in the current AWD. This contribution describes more details on the assignment AMAP structure so that blind detection at MS side can be implemented with reasonable complexity and false detection probability. Detailed analysis can be found in [1, 2].

References

[1] Assignment AMAP Design, S802.16m-09/0953, May 2009.

[2] Analysis of Assignment A-MAP Transmission in IEEE 802.16m, IEEE 802.16m-09/0721, March 09, 2009.

Text proposal for inclusion in 802.16m amendment

------------------------------------------- Start of Proposed Amendment Text -----------------------------------------------

### 15.3.6.2.2.2 Assignment A-MAP

…

The assignment A-MAP IE shall be transmitted with one MLRU or multiple concatenated MLRUs in the A-MAP region. The number of logically contiguous MLRUs is determined based on the assignment IE size and channel coding rate, where channel coding rate is selected based on AMS’ link condition.

Assignment A-MAPs are grouped together based on MCS level and A-MAP IE sizes. Assignment A-MAPs in the same group are transmitted with the same MCS level and contain the same A-MAP IE size. Each assignment A-MAP group contains several logically contiguous MLRUs. The number of assignment A-MAPs in each assignment A-MAP group is signaled through non-user specific A-MAP.

### 15.3.6.4.2.1 Non-user-specific A-MAP IE

Non-user-specific A-MAP IE consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode assignment A-MAP IE. The number size of assignment A-MAPs the in each assignment A-MAP group is indicated in the Non-user-specific A-MAP IE, in the unit of MLRUs. The detailed information included in non-user specific information is TBD. The non-user specific A-MAP IE is shown in Table 660.

Table 660 – Non-user Specific A-MAP IE

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| Assignment A-MAP size | TBD | Indicates the size of the assignment A-MAP in the unit of one or multiple MLRU. Indicate the number of assignment A-MAPs in each assignment A-MAP group as shown in Table xxx. |

Table xxx [Detailed entry is TBD] shows the number of the assignment A-MAPs in each assignment A-MAP group.

Table xxx – The number of assignment A-MAPs in each assignment A-MAP group

| Index | Assignment A-MAP group-1 | Assignment A-MAP group-2 | … |
| --- | --- | --- | --- |
| … | … | … | … |

--------------------------------------------- End of Proposed Amendment Text ----------------------------------------------