| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized AWD Text on the MAC Procedure for Group Resource Allocation |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Jason Junsung Lim, Seho Kim, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co. LTD. Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan Intel Corporation Wei Xia, Yangfeng Guan, Ping Qiu, Jerry Chow, Ying Liu ZTE Corporation Kevin Power Fujitsu | Junsung.lim@samsung.com Shweta.shrivastava@intel.com Guan.yanfeng@zte.com.cn Kevin.power@uk.fujitsu.com |
| Re: | Category: AWD comments/Area: Chapter 15.2.7 (Group Resource Allocation) “Comments on AWD 15.2.7 Group Resource Allocation” |  |
| Abstract | Proposed text for Group Resource Allocation MAC Procedure |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized AWD Text on the MAC Procedure for Group Resource Allocation

Jason Junsung Lim, Seho Kim, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics Co. LTD

Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan

Intel Corporation

Wei Xia, Yangfeng Guan, Ping Qiu, Jerry Chow, Ying Liu

ZTE Corporation

- Introduction

This contribution proposes text to be included in the MAC procedure for Group Resource Allocation of the 802.16m amendment working document (AWD).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

15.2.7	Group Resource Allocation

Group Resource Allocation mechanism allocates resources to multiple users as a group in order to save control overhead. The mechanism takes advantage of common traffic characteristics and grouping is done based on some common parameters such as MCS (modulation and coding scheme) and resource size, which further saves overhead.

15.2.7.1 Grouping Mechanism

UsersAMSs are assigned to groups based on the combination of MCS used and the resource allocation size (number of LRUs) required. A set of n-bit codes can be used to represent the different combinations of MCSs and resource sizes that are used by a group. These codes are included in a bitmap as part of the group’s resource allocation information.

15.2.7.2 Group Configuration

A group facilitates the dynamic link adaptation on the limited set of MIMO mode, MCS level and HARQ data burst size.

ABS configures a Group MIMO Mode Set for each group among the predefined candidate sets listed in Table xxx for downlink and Table yyy for uplink. When an AMS is added into the group, the configured Group MIMO Mode Set ID is indicated through Group Configuration IE. The assigned MIMO mode to AMS in the group shall be chosen from the configured set.

Table xxx – DL MIMO mode set candidates

| ID | DL Group MIMO Mode Set | SM restriction |
| --- | --- | --- |
| 0b00 | Mode 0 |  |
| 0b01 | Mode 0, Mode 1 | Mt =2 |
| 0b10 | Mode 2 | Mt= 1 |
| 0b11 | Mode 2, Mode 4 | Mt=1 |

Table yyy – UL MIMO mode set candidates

| MIMO mode set ID | UL Group MIMO Mode Set | SM restriction |
| --- | --- | --- |
| 0b00 | Mode 0 |  |
| 0b01 | Mode 0, Mode 1 | Mt=2 |
| 0b10 | Mode 3 | Mt =1 |
| 0b11 | Mode 2, Mode 4 | Mt=1 |

ABS configures a MCS Set for each group among the predefined candidate sets listed in Table aaa for both downlink and uplink. When an AMS is added into the group, the configured MCS Set ID is indicated through Group Configuration IE. The assigned MCS to AMS in the group shall be chosen from the configured set.

Table aaa - MCS Set Candidates

| ID | MCS Set |  |  |  |
| --- | --- | --- | --- | --- |
| 000 | 0000 | 0001 | 0010 | 0011 |
|  | 0100 | 0101 | 0110 | 0111 |
|  | 1000 | 1001 | 1010 | 1011 |
|  | 1100 | 1101 | 1110 | 1111 |
| 001 | 0000 | 0001 | 0010 | 0011 |
|  | 0100 | 0101 | 0110 | 0111 |
| 010 | 1000 | 1001 | 1010 | 1011 |
|  | 1100 | 1101 | 1110 | 1111 |
| 011 | 0000 | 0001 | 0010 | 0011 |
| 100 | 0100 | 0101 | 0110 | 0111 |
| 101 | 1000 | 1001 | 1010 | 1011 |
| 110 | 1100 | 1101 | 1110 | 1111 |

ABS configures a HARQ Burst Size Set for each group among the predetermined [4, TBD] HARQ Burst Size Set Candidates. Those candidates are signaled to AMSs via the S-SFH(TBD). When an AMS is added into the group, the configured HARQ Burst Size Set ID is indicated through Group Configuration IE. The assigned HARQ burst size to AMS in the group shall be chosen from the configured set.

As an optional, the combinations of MCS/MIMO mode/Resource size can be signaled explicitly by the ABS to the AMS (TBD).

15.2.7.3 Group Management

15.2.7.3.1 Addition of AMS to a Group

Addition of an AMS to a group occurs when group resource allocation is initialized for the AMS or when AMS in a group moves to other group. For inclusion, the group information shall be informed to AMS in order to interpret resource assignment information from Group Resource Allocation A-MAP IE. The information is transmitted through Group Configuration A-MAP IE.  The Group Configuration A-MAP IE can be either unicast or broadcast. The details on broadcasting this IE are TBD.

15.2.7.3.1.1 ABS Operation

When an ABS decides to use group resource allocation for an AMS, the ABS adds the AMS into an appropriate group among existing groups. If the existing groups are not appropriate to the AMS, the ABS may form a new group. ABS shall indicate group configuration information via Group Configuration A-MAP IE which includes the added Group ID and the assigned User Bitmap Index to the AMS.

Once the AMS is added to the group, resources used for initial transmission of HARQ data burst may be allocated as part of the group until the AMS is deleted from the group.

15.2.7.3.1.2 AMS Operation

Upon receiving Group Configuration A-MAP IE, the AMS receives the required information to interpret the assigned MIMO mode, MCS level and resource index from the bitmaps in the corresponding Group Resource Allocation A-MAP IE. Once the AMS receives a Group Configuration A-MAP IE, the AMS shall monitor its allocation in the Group Resource Allocation A-MAP IE until it is deleted from the group.

15.2.7.3.2 Deletion of AMS from a Group

The ABS may delete an AMS from a group when one or more of the following conditions applies: (i) the connection is terminated (ii) the MIMO mode/MCS/HARQ burst size suitable for the AMS no longer belongs to the MIMO Mode Set/MCS set/HARQ burst size set corresponding to the group.

15.2.7.3.2.1 ABS Operation

ABS may delete multiple AMSs from a group in a subframe. The deletion is informed by listing the User Bitmap Index of deleted AMS in Group Resource Allocation A-MAP IE. The deletion shall apply from the subframe in which the deletion information is sent.

15.2.7.3.2.2 AMS Operation

15.2.7.3.3 Bitmaps in Group Resource Allocation

GRA makes use of bitmaps to signal resource allocation information for users AMS within a group. These bitmaps are sent in the Group Resource Allocation IE. The first bitmap is the User Bitmap which uses 1 bit per userAMS to signal which users are scheduled in the frame.

The second bitmap is MIMO Bitmap which are used to indicate the assigned MIMO mode, when multiple MIMO modes and SM parameters are supported in the group. The existence of second bitmap and the length of bits per scheduled AMS are listed in Table bbb and Table ccc.

Table bbb – Second Bitmap Information for DL

| MIMO Mode Set | Existence of second bitmap | Length of bit per scheduled AMS | MIMO mode indication |
| --- | --- | --- | --- |
| 0b00 | No | - | OL SU-MIMO (SFBC with non-adaptive precoder) |
| 0b01 | Yes | 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) 0b1: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 |
| 0b10 | No | - | CL SU-MIMO with Mt=1 |
| 0b11 | Yes | 1 | 0b0: CL SU-MIMO with Mt=1 0b1: CL MU-MIMO with Mt=1, Nt=2 |

Table ccc – Second Bitmap Information for UL

| MIMO Mode Set | Existence of second bitmap | Length of bit per scheduled AMS | MIMO mode indication |
| --- | --- | --- | --- |
| 0b00 | No | - | OL SU-MIMO (SFBC with non-adaptive precoder) |
| 0b01 | Yes | 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) with Mt=2 0b1: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 |
| 0b10 | No | - | CL SU-MIMO with Mt=1 |
| 0b11 | Yes | 1 | 0b0: CL SU-MIMO with Mt=1 0b1: CL MU-MIMO with Mt=1, TNS=2 |

When MIMO Mode Set of the group contains MU-MIMO, PSI Bitmap and Pairing Bitmap are appeared to determine AMS pair sharing same resource. PSI Bitmap uses 1 bit per scheduled AMS to indicate the assigned pilot stream index(PSI).

Pairing Bitmap uses to indicate a pair of two AMSs using different PSI. The number of bits per pair in the Pairing Bitmap depends on the total number of pairs in the group. If there are n pairs in the group, the number of bits per pair is p = ceil[log2(n)]. The AMSs using PSI=0 is assigned an index starting from 0 to n – 1 in the same order in which they appear in the bitmap. Every p bits in the pairing bitmap are assigned to the AMS using PSI=1 in the same order in which they appear in the PSI Bitmap. These p bits carry the index of AMS with PSI=0 that is paired with corresponding AMS with PSI=1.

The secondthird bitmap is the Resource Allocation bitmap which uses n bits per userAMS to signal MCS and Resource Size for users the scheduled AMS in the subframe or extended subframe that are scheduled in the frame. The scheduled AMSs may have different number of bits in the third bitmap when they are assigned different MIMO mode and SM parameter.

The bit length used for a scheduled AMS is determined by the total number of effective combinations for MCS and resource size for the assigned MIMO mode with SM parameter. Effective combinations are derived by subtracting useless combinations among all possible combinations. The following steps are needed to find the effective combinations.

- Step 1: List all possible combination set C={ C(0,0), C(0,1), …, C(M,B) }

Table ddd. Combination indexes

| MCS/ HARQ data burst size | 1 | 2 | … | B (Highest burst size) |
| --- | --- | --- | --- | --- |
| 1 | C(1,1) | C(1,2) | … | C(1,B) |
| 2 | C(2,1) | C(2,2) | … | C(2,B) |
| … | … | … | … | … |
| M (Highest MCS) | C(M,1) | C(M,2) | … | C(M,B) |

C(m,b): Combination index for MCS m, HARQ data burst size b

- Step 2: For each HARQ data burst size, useless combination is chosen when it requires same resource size with a lower MCS level comparing to others due to the resource granularity. For b∈IB, m∈IM, n∈IM, and m>n,

{C(m,b)} U1 	if N(m,b) = N(n,b)

Where

U1 : Useless combination set type 1

IM : Group MCS set

IB : Group HARQ burst size set

N(m,b) : Required number of RUs for MCS m, HARQ data burst size b

- Step 3: For a given MCS, useless combination is chosen when it requires same resource size supporting a smaller HARQ data burst size than others due to the resource granularity. For m∈IM, b∈IB, d∈IB, and b>d,

{C(m,b)} U2 	if N(m,b) = N(m,d)

Where U2 is useless combination set type 2

- Step 4: Derive effective combination set (E) which is

E = C-U1-U2

An index code is identified to each effective combination from lower MCS level and lower HARQ data burst size where n is determined by Ceil{log2(Total number of effective combinations)}.

An example of the bitmaps is shown in Figure 390.

Examles of the utilizing bitmaps are shown in Figure xx, xy and xz.

Figure xx: Example of Bitmaps for Group MIMO Mode Set 0b00 and 0b10

Figure xy: Example of Bitmaps for Group MIMO Mode Set 0b01

Figure xz: Example of Bitmaps for Group MIMO Mode Set 0b11

15.2.7.4 HARQ Operation for Group Resource Allocation

15.2.7.54 Error Handling Procedure

FFS