| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized AWD Text on the Unicast Control Information Elements for Group Resource Allocation |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Jason Junsung Lim, Seho Kim, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co. LTD. Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan Intel Corporation Wei Xia, Yangfeng Guan, Ping Qiu, Jerry Chow, Ying Liu ZTE Corporation | Junsung.lim@samsung.com Shweta.shrivastava@intel.com Guan.yanfeng@zte.com.cn |
| Re: | Category: AWD comments/Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6.4.2.4 Group Resource Allocation A-MAP IE |  |
| Abstract | Proposed text for Unicast Control Information Elements for Group Resource Allocation |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized AWD Text on the Unicast Control Information Elements

for Group Resource Allocation

Jason Junsung Lim, Seho Kim, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics Co. LTD

Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan

Intel Corporation

Wei Xia, Yangfeng Guan, Ping Qiu, Jerry Chow, Ying Liu

ZTE Corporation

- Introduction

This contribution proposes text to be included in the section 15.3.6.4.2 Unicast Control Information Elements of the 802.16m amendment working document (AWD).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

15.3.6.4.2.4  Group Resource Allocation A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple mobile stationsAMSs within a group.

Group scheduling requires two operations

- Assignment of a user AMS to a group. In order to add a userAMS to a group in the DL or UL, the ABS shall transmit a [Group Configuration MAC management message] [Group Configuration A-MAP IE]

- Allocation of resources to usersAMSs within a group. In order to assign resources to one or more usersAMSs in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled usersAMSs and signal resource assignment, MCS, MIMO mode and resource size.

Table xxx DLUL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | TBD A-MAP IE types distinguish between UL/DL, SU/MU, OL/CL MIMO operation, persistent/non-persistent allocation, basic/extended IEs including GRA DL Group Resource Allocation A-MAP IE |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACID | TBD | TBD Required for explicit assignment of a single ACID group. Not required if implicit cycling of ACIDs starting from an initial ACID value is used as in 802.16e |
| ACK Channel Offset | TBD | Indicates the start of ACK index used for scheduled allocations at this subframe in the group. |
| HARQ ReTx Indicator | TBD | TBD Indicates whether this group resource assignment IE is for HARQ retransmissions or initial transmission |
| NDA | [2][3] | Indicates the number of deleted AMSs in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | TBD Size of the user bitmap; may not be needed if user bitmap size is included in configuration message/A-MAP IE Indicates the length of User Bitmap |
| User Bitmap | Variable | Bitmap to indicate scheduled users AMSs in a group. The size of the bitmap is equal to the User Bitmap Size |
| Resource Allocation Bitmap descriptor | TBD | TBD MCS group type, packet/resource size type information or attributes required to decode resource assignment bitmap |
| If( Group MIMO mode set ==0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user AMS |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

Table xxx UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Resource Allocation A-MAP IE |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACK Channel Offset | TBD | Indicates the start of ACK index used for scheduled allocations at this subframe in the group. |
| NDA | [2][3] | Indicates the number of deleted AMS in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | Size of the user bitmap; may not be needed if user bitmap size is included in configuration message/A-MAP IE |
| User Bitmap | Variable | Bitmap to indicate scheduled users in a group. The size of the bitmap is equal to the User Bitmap Size |
| If( Group MIMO mode set == 0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b10 or 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

15.3.6.4.2.X  Group Configuration A-MAP IE

The unicast group configuration A-MAP IE is used for initiating and maintaining a group for resource assignment. The broadcast group configuration A-MAP IE is TBD.

Table xxx DL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Period | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframe for FDD or all DL frame subframe for TDD |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: CL SU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC |

Table xxx UL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Period | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframe for FDD or all UL subframe for TDD |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: OL MU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| If(DL:UL !=3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| }else if (DL:UL !=3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC |

-------------------------------  Text End  ---------------------------------------------------