| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed TBCC code for DL control channels in the IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Yi Hsuan, Changlong Xu, Debdeep Chatterjee, Hujun Yin Intel Corporation | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes updates to the text related to DL control coding in the 802.16m AWD |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed TBCC Code for DL Control Channels in the IEEE 802.16m AWD

Introduction

In the current 16m DL control design, many DL control channels have short IE block length, e.g., SFH, assignment AMAP, and non-user specific AMAP. Tail-biting convolutional code (TBCC) has good performance when the code block length is short and is therefore a natural choice for DL control channels.

To further improve the reliability and coverage of DL control channels, we propose a ¼ rate TBCC which is based on the ½ rate TBCC used in 16e. Like the ½ TBCC, the ¼ TBCC also has a constraint length of 7 so there is no significant increase of coding complexity. Two of the four generator polynomials of the ¼ TBCC are the same as generator polynomials of the ½ TBCC code.

We evaluate the performance of ¼ TBCC vs. ½ TBCC over different block sizes and scenarios and find that using the proposed ¼ TBCC yields performance improvements that ranges from 0.4 dB to 1 dB [1]. Therefore we propose to use this ¼ TBCC for appropriate DL control channels.

References

[1] TBCC Code for DL Control Channels, S802.16m-09/0957, May 2009.

Text proposal for inclusion in 802.16m amendment

------------------------------------------- Start of Proposed Amendment Text -----------------------------------------------

15.3.6.3.1.1 Primary Superframe Header

The resulting sequence of bits shall be encoded by the TBCC convolutional encoder described in 15.3.6.3.3. <<reference to 15.3.x channel coding section>>. A coding rate of 1/2 is used.

15.3.6.3.1.2 Secondary Superframe Header

The resulting sequence of bits shall be encoded by the TBCC described in 15.3.6.3.3. channel encoder. The channel encoder for S-SFH is described in <<15.3.x channel coding section>>. A code rate of 1/2 or 1/3 is used.

15.3.6.3.2.2 Assignment A-MAP

The resulting sequence of bits shall be encoded by a the TBCC described in 15.3.6.3.3. channel encoder.

15.3.6.3.3 Tail-biting convolutional code for DL control channels

Figure DL-1 shows a block diagram of the rate 1/4 tail-biting convolutional codes for DL control channels. 1/4 TBCC encoded codewords are separated to four subblock and go through subblock interleaver. The length of coded block size is chosen according to the needed coding rate.

Figure DL-1 Block diagram of the tail-biting convolutional code for DL control channels

15.3.6.3.3.1 TBCC encoder with rate of 1/4

The binary TBCC shall have native rate of 1/4, a constraint length equal to K=7, and shall use the following generator polynomials codes to derive its five coded bits:

G1=171OCT FOR A    G2=133OCT FOR B

G3=165OCT FOR C    G4=117OCT FOR D

The generator is depicted in Figure DL-2.

15.3.6.3.3.2 Bit Seperation

All of the encoded bits shall be demultiplexed into four subblocks denoted A, B, C, and D. Suppose L information bits are input to the encoder. The encoder output bits shall be sequentially distributed into four subblocks with the first L encoder output bits going to the A subblock, the second L encoder output going to the B subblock, the third L to the C subblock, the fourth L to the D subblock.

15.3.6.3.3.3 Subblock interleaver

First, the table for interleaving index with length of 128 entries was generated as follows.

x = 1 : 128

index = (15x+32x2)mod 128 + 1.

when the number of information bits is less than 128, the corresponding index table can be generated by removing the entries whose values are larger than the number of information bits.

Figure DL-2 TBCC encoder of rate ¼

15.3.6.3.3.4 Bit grouping

The channel interleaver output sequence shall consist of the interleaved A and B subblock sequences, followed by interleaved C, and D subblock sequences

15.3.6.3.3.5 Bit selection

Suppose L information bits are input to the encoder. The output sequence of bit group consists of 4L bits denoted as di,i=0,1,⋯,4L. Parameter Kbufsize is used to indicate the size of buffer using for repetition. Its value is not larger than 4L. If the output bits are M, the output sequence can be expressed as cj=djmodKbufsize, j=0,1,⋯,M.

--------------------------------------------- End of Proposed Amendment Text ----------------------------------------------