| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On Scheduling System Information Sub-Packets in Super Frame Header |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Ying Li, Zhouyue Pi, Baowei Ji, Kaushik Josiam, Sudhir Ramakrishna, Jiann-An Tsai Mihyun Lee, Rakesh Taori Samsung Electronics | yli2@sta.samsung.com |
| Re: | Call for Change for IEEE 802.16m Amendment, 15. 3.6 Downlink control, on SFH |  |
| Abstract | The contribution proposes text for DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On Scheduling System Information Sub-Packets in Super Frame Header

Ying Li, Zhouyue Pi, Baowei Ji, Kaushik Josiam, Sudhir Ramakrishna, Jiann-An Tsai

Mihyun Lee, Rakesh Taori

Samsung Electronics

- Introduction on SP scheduling in SFH

Note that although the scheduling bitmap of SP in SP1 can give MS the information of whether the SPs follow in the same SFH, it does not give MS any information about where the future possible SPs are scheduled. However, an MS may not monitor SP1 all the time, e.g., when it is not in active communication state. If an MS knows when the next SP it needs will be scheduled and the MS only is interested in getting that particular next SP, MS can stop monitoring SP1, and resume to monitor at the transmission time of the SP it is interested in. This can reduce the time of monitoring SFH and MS can reduce power consumption. In other words, it is good for MS to have configurable scheduling information which could be known even in earlier SFH than the actual transmission time of SPs, rather than MS constant monitoring SFH.

The configurable periodicity for SPs meets various requirements for diverse deployment scenarios.

Due to the reasons above, we propose the removal of the scheduling bitmap for SPs and the inclusion of the SP periodicity in SP1.

In addition, the scheduling of SPs can be calculated based on SFN, the SP periodicity, and the number of repetitions.

Main changes (Assume there are only three SPs)

- Remove scheduling bitmap.

- Add periodicity of SP2,3 to SP1.

- Add Section 15.3.6.4.1.3 on scheduling SPs and propose text.

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 80216m-09/0010r1a, AWD, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

# DL Control Information Elements

# Broadcast Control Information Elements

# S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted in different timing and periodicity.

S-SFH SP1 IE presented in Table 655 includes essential information needed for select and access the network.

Table 655 – S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| …….. |  |  |
| S-SFH SP scheduling information S-SFH SP scheduling information { | TBD | SP scheduling bitmap |
| Periodicity of SP2, SP3 | 3 | 000 SP2: 8, SP3: 16 001 SP2: 8, SP3: 32 010 SP2: 8, SP3: 64 011 SP2: 16, SP3: 16 010 SP2: 16, SP3: 32 011 SP2: 16, SP3: 64 (in the unit of superframes) |
| } |  |  |
| …….. |  |  |
| } |  |  |

# Scheduling of SPs

The periodicity of an SP is defined as the time in-between two first transmissions of the SP. Within a period, there are a number of repetitions of the SP.

SP1 has a periodicity of 80ms and is repeated 4 times within 80ms. The first transmission is scheduled in SFH for which the SFN mod 4=0.

The scheduling of other SPs is configurable. P-SFH configures the transmission periodicity for the SPs.

The first transmission and repeated transmission of SPs are scheduled with offsets. The offsets are calculated based on SFN, periodicity (in the unit of superframes), and the number of repetitions of SPs. For SP2, the k-th transmission is scheduled in SFH for which the SFN mod (Periodicity_OF_SP2) = 2 * k, k = 0, 1,2,…,(N_REP_SP2-1). For SP3, the k-th transmission is scheduled in SFH for which the SFN mod (Periodicity_OF_SP3) = 2 * k + 1, k = 0, 1,2,…,(N_REP_SP3-1).

Figure.  Example of SP scheduling with N_REP_SP2 = 4, and N_REP_SP3 = 4

-------------------------------  Text End  ---------------------------------------------------