| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message Design to support Secondary Carrier Activation |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Chieh-Yuan Ho, Ming-Hsien Wu, Ching-Yao Huang, I-Kang Fu, Yih-Shen Chen, Paul Cheng NCTU/MediaTek | IK.Fu@mediatek.com thorpe.ho@mediatek.com cyhuang@faculty.nctu.edu.tw |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” – AWD New Contribution for Multi-Carrier |  |
| Abstract | According to P802.16m SDD, assigned secondary carriers will not always be activated for saving unnecessary AMS processing power and signaling overheads. This contribution proposes to have a new message for ABS to activate the secondary carrier transmission by AMS. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Message Design to support Secondary Carrier Activation

Chieh-Yuan Ho, Ming-Hsien Wu, Ching-Yao Huang, I-Kang Fu, Yih-Shen Chen, Paul Cheng

National Chiao Tung University (NCTU) / MediaTek

- Introduction

In the 802.16m System, Advanced Mobile Station (AMS) can support data transmission over multiple carriers. In order to best utilize the scarce radio resources, Advanced Base Station (ABS) may change carriers to serve data transmission for specific AMS from time to time, i.e. on a per frame basis. Therefore, an efficient method for carrier activation and de-activation will be necessary. Because AMS might consume more power while operating on more than one carrier, a detailed procedure of multi-carrier power management need to be carefully designed in order to further extend AMS battery life. The motivation of this contribution is to design an efficient carrier activation procedure and supporting message design to improve the performances of multi-carrier support in the 802.16m system.

In the latest P802.16m System Description Document (SDD), the carrier which may be potentially used by the AMS is defined as the assigned carriers. The assigned carriers are supposed to be determined based on ABS and AMS hardware capabilities. Among the assigned carriers, some of the carriers will be activated for data transmission based on system requirements and called the active carriers. Since the primary carrier will always be one of the active carriers, the secondary carriers of AMS may be activated or be de-activated from time to time. In order to fulfill member’s classification on assigned carrier and active carrier, the stage 3 standard shall define proper procedure and messaging support for secondary carrier activation and de-activation.

- Recommended Procedure for Secondary Carrier Activation

The primary carrier shall be capable to carry a multi-carrier indication by the message such as MOB_MC-IND, so that the AMS know it has to decode the A-MAPs in those secondary carriers after receiving this indication.

Up on receiving the active secondary carrier index through the MOB_MC-IND, the AMS shall start processing the designated secondary carriers as the way it process the primary carrier. The A-A-MAP IE to support resource allocation in the secondary carriers for AMS is supposed to be transmitted through corresponding carriers respectively, so that AMS doesn’t need to decode the A-A-MAP IE in the primary carrier for the traffic in secondary carrier. In addition, the secondary carrier can also be de-activated by ABS through the MOB_MC-IND in unsolicited manner. While ABS is not supposed to send further control information to AMS after sending unsolicited message in primary carrier to deactivate the secondary carrier, AMS doesn’t need to monitor the deactivated carrier since next frame.

For carrier aggregation mode, the MOB_MC-IND is supposed to be sent in on-off manner. Therefore, the indication to deactivate the secondary carrier should be sent after a period of time after activating it. For the carrier switching mode, this multi-carrier indication A-MAP IE may be sent in frame base. How to coordinate the transmission in carrier switching mode is FFS and up to members’ consensus.

The Figure 1 illustrates basic idea of the MOB_MC-IND for IEEE 802.16m multi-carrier system. As shown in figure, assuming three AMSs treat carrier 1 as their primary carrier, the MOB_MC-IND sent in the primary carrier indicates a set of secondary index to AMSs over the primary carrier. After the reception of the MOB_MC-IND, AMSs has to decode the corresponding A-MAP IEs and A-A-MAP IE in the designated carriers and/or sub-frames. The current idea is that the IE content is pointing the situation in next frame, but this can be further discussed by members.

[Figure will be updated in r1 version]

Fig. 1 Basic idea of the MOB_MC-IND to activate secondary carrier and to identify which sub-frames contain the A-MAP IE or A-A-MAP IE for AMS

- Proposed Text Input to P802.16m Amendment Working Document

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the following text into P802.16m Amendment Working Document]

# 15.3.x MOB_MC-IND (Multi-Carrier Indication) message

Table xxx – MOB_MC-IND message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_MC-IND _Message_format() { | - | - |
| Management Message Type = xx | 8 | - |
| Multi-Carrier Mode | 1 | 0: Multi-Carrier Aggregation 1: Multi-Carrier Switching |
| If (Multi-Carrier Mode == 0) { |  |  |
| Secondary Carrier Activation Bitmap | TBD | Refer to logical carrier index for assigned carriers |
| } |  |  |
| If (Multi-Carrier Mode == 1) { | - |  |
| Secondary Carrier Index | TBD | Refer to logical carrier index for assigned carriers |
| } |  |  |
| } |  |  |

----------------------------------------------------------End of the Text-----------------------------------------------------------