| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Messaging Design to Support FFR operation |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Chun-Ting Chou, I-Kang Fu, Paul Cheng NTU/ MediaTek | chuntingchou@cc.ee.ntu.edu.tw IK.Fu@mediatek.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” – AWD New Contribution for Interference Mitigation |  |
| Abstract | This contribution proposes the basic message format design to support FFR (Fractional Frequency Reuse) operation in IEEE 802.16m |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Messaging Design to Support FFR Operation

Chun-Ting Chou, I-Kang Fu, Paul Cheng

National Taiwan University (NTU) / MediaTek

- Introduction

This contribution aims to investigate the reasonable MAC protocol design to support FFR operation in IEEE 802.16m system. According to the latest Amendment Working Document (AWD), the FFR configuration and the associated permutation applied in each frequency partition is expected to be transmitted over SFH (Super Frame Header). Therefore, the AMS can learn this information after the network entry and start to decode the A-MAP IEs transmitted over each frequency partitions.

In order to collect the necessary interference statistics to help ABS to determine the AMS should be served by which frequency partition, proper message should be provided to allow AMS to report the interference measurement results to ABS. Since the AMS can perform interference measurement over specific frequency partition, it can also report the measurement result for specific region of LRUs. Because the measurement result obtained over CRUs and over DRUs should not mix together since the interference statistics are usually much different from each other. For the CRUs, the interfering scenario is more like narrow band jamming, while the DRU usually leads to the randomized interference with flat distribution.

If the AMS perform autonomous interference measurement, it may report the measurement result in unsolicited manner, or it can directly make recommendation to the serving ABS on the preferred or non-preferred frequency partition. Upon AMS’s request on frequency partitions, ABS can reply its decision to ABS via the MAC management message. ABS should be able to use the same message to allocate/de-allocate the frequency partitions to AMS. For the frequency partitions not allocated to AMS, AMS doesn’t need to decode the A-MAP IE transmitted over that frequency partitions to save computation power.

Fig.1 Example on MOB_FFR-REQ/RSP message flow

Base on the typical MAC management message format, this contribution suggests defining MOB_FFR-REQ/RSP messages to support FFR operation in IEEE 802.16m. Figure 1 is an example on the possible usage of the MOB_FFR-REQ/RSP messages. In addition, the ABS may also send MOB_FFR-RSP to request AMS perform interference measurement over the designated frequency partition and LRUs in unsolicited manner. Note that ABS may also use the same way to assign or to update frequency partition used by AMS.

- Proposed Text Input to P802.16m Amendment Working Document

-------------------------------------------------------Start of the Text-------------------------------------------------------

[Add the following text before the end of P802.16m Amendment Working Document (802.16m-09/0010r1a)]

15.2.4 MAC Management Message

15.2.4.x MOB_FFR-REQ (Fractional Frequency Reuse Request) message

A MOB_FFR-REQ message is sent by an AMS to request specific frequency partition be used to serve for its data transmission, or to report the measurement results over specific frequency partition.

Table xxx MOB_FFR-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_MC-ADV_Message_format() { | - | - |
| Management Message Type = xx | 8 | - |
| Frequency Partition Index | 2 | - |
| Request Type | 2 | 0b00: Frequency Partition Request 0b01: Frequency Partition Remove 0b10: Measurement Report – Interference Level 0b11: Measurement Report – SINR Level |
| If (Request Type == 0b10){ |  |  |
| Start LRU Index | 8 | LRU Index based on ABS broadcast information |
| End LRU Index | 8 | LRU Index based on ABS broadcast information |
| Interference Measurement Report - Mean | 8 |  |
| Interference Measurement Report - Variance | 8 |  |
| } else if (Request Type == 0b11){ |  |  |
| Start LRU Index | 8 | LRU Index based on ABS broadcast information |
| End LRU Index | 8 | LRU Index based on ABS broadcast information |
| SINR Measurement Report - Mean | 8 |  |
| SINR Measurement Report - Variance | 8 |  |
| } |  |  |
| } |  |  |

15.2.4.x +1 MOB_FFR-RSP (Fractional Frequency Reuse Response) message

A MOB_FFR-RSP shall be transmitted by the ABS either unsolicited or in response to a MOB_FFR-REQ message sent by an AMS. An ABS may transmit MOB_FFR-RSP with or without the updated frequency partition or the permutation.

Table xxx MOB_FFR-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_MC-ADV_Message_format() { | - | - |
| Management Message Type = xx | 8 | - |
| Frequency Partition Index | 2 | - |
| Command Type | 2 | 0b00: Add frequency partition 0b01: Remove frequency partition 0b10: Perform interference measurement 0b11: Perform SINR measurement |
| If (Request Type == 0b10 or 0b11){ |  |  |
| Start LRU Index | 8 | LRU Index based on ABS broadcast information |
| End LRU Index | 8 | LRU Index based on ABS broadcast information |
| } |  |  |
| } |  |  |

--------------------------------------------------------End of the Text-------------------------------------------------------