| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the IEEE 802.16m Amendment on Multi-Radio Coexistence: Management Messages and TLVs |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Chi-Chen Lee, I-Kang Fu, Li-Chun Ko, Chih-Hao Yeh, Kyle Hsu, Paul Cheng MediaTek Inc. Jing Zhu Intel Corporation | chichen.lee@mediatek.com IK.Fu@mediatek.com LC.Ko@mediatek.com jing.z.zhu@intel.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” – Comments on AWD 15.2.4 MAC Management Message |  |
| Abstract | Proposed text for multi-radio coexistence in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the IEEE 802.16m Amendment on Multi-Radio Coexistence: Management Messages and TLVs

Chi-Chen Lee, I-Kang Fu, Li-Chun Ko, Chih-Hao Yeh, Kyle Hsu, Paul Cheng

MediaTek Inc.

Jing Zhu

Intel Corporation

- Introduction

The contribution proposes the text on multi-radio coexistence for 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] C802.16m-09/543r1, “Proposed Text for the IEEE 802.16m Amendment on Multi-Radio Coexistence”

Add the following text in section 15.2.4

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.4 AAI MAC Management Messages

15.2.4.x CLC-REQ (Co-Located Coexistence Request) message

The AMS send the CLC-REQ message to activate, terminate, or reconfigure one or several Type I, Type II, or/and Type III CLC classes. The CLC-REQ message is sent from the AMS to the ABS on the AMS’s Basic CID. The AMS may include CLC-INFO TLV.

Table x1—CLC-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| CLC-REQ_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Request Action | 8 |  |
| TLV encoded items | variable |  |
| } |  |  |

Parameters shall be as follows:

Request Action

Bit #i of the field set to “0” indicates that AMS requests to terminate the existing CLC class with CLC ID = i, or the CLC class with CLC ID = i does not exist

Bit #i of the field set to “1” indicates that AMS requests to activate the CLC class with CLC ID = i if it does not exist. For existing CLC ID, MS may request to re-configure or replace existing CLC class.

The following TLVs may be included:

CLC Information compound TLV

This TLV may be present more than once if AMS tries to active multiple CLC classes.

15.2.4.x CLC-RSP (Co-Located Coexistence Request) message

The AMS sends the CLC-RSP message to AMS on the MS’s basic CID in response to CLC-REQ.

Table x2—CLC-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| CLC-RSP_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Confirmed Action | 8 |  |
| for (i=1; i n; i++) { |  | may be included only for Type II or III CLC class |
| CLC ID | 3 |  |
| Start Superframe Number | 3 |  |
| Start Frame Index | 2 | shall be set to 0 for Type III CLC class |
| } |  |  |

Parameters shall be as follows:

Confirmed Action

Bit #i of the field set to “0” indicates that ABS confirms the termination of the existing CLC class with CLC ID = i, or the CLC class with CLC ID = i does not exist

Bit #i of the field set to “1” indicates that ABS confirms the activation of the CLC class with CLC ID = i,. For existing CLC ID, the ABS confirms the reconfiguration or the replacement requested by the MS.

CLC ID

An integer number (0~7) to uniquely identify a CLC class

Start Time information

Start Superframe Number

The 3 LSB of the superframe number of CLC start time

Start Frame Index

The frame index of CLC start time

15.x AAI TLV encodings

15.x.1 CLC_INFO

Table x3 CLC-INFO TLV Format

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| XX | varied | Unified TLV encoding for CLC information fields | REG-REQ,REG-RSP,CLC-REQ, SBC-REQ, SBC-RSP |

Table x4— Unified TLV encoding for CLC information fields

| Syntax | Size (bits) | Note | Scope |
| --- | --- | --- | --- |
| Information Type | 8 |  |  |
| If (Information Type = 0) { |  |  |  |
| CLC Limit } | 24 | Table x5 | REG-RSP |
| If (Information Type = 1) { |  |  |  |
| CLC Request} | variable | Table x6 | RNG-REQ, SBC-REQ, CLC-REQ |
| If (Information Type = 2) { |  |  |  |
| CLC Response} | variable | Table x7 | RNG-RSP, SBC-RSP |
| If (Information Type = 3) { |  |  |  |
| CLC Report} | variable | Table x8 | CLC-REQ |

Table x5: Syntax of CLC Limit field

| Syntax | Size (bit) |
| --- | --- |
| the bit field set to 1 indicates the CLC limits are provided for Type I CLC class | 1 |
| the bit field set to 1 indicates the CLC limits are provided for Type II CLC class | 1 |
| maximum number of active CLC classes of the type | 4 |
| maximum CLC active ratio in unit of 1/100 | 6 |
| maximum CLC active interval in unit of subframes | 8 |
| Reserved | 4 |

Table x6: Syntax of CLC Request field

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| for (i=0; i < n; i++) { |  |  |
| CLC ID | 3 |  |
| Scheduling Impact | 2 |  |
| Start Superframe Number | 3 |  |
| Start Frame Index | 2 |  |
| Flag | 2 |  |
| Reserved | 4 |  |
| If (Flag = 0b01) { |  |  |
| CLC active interval of Type I CLC class | 8 |  |
| CLC active cycle of Type I CLC class | 21 |  |
| Start Subframe Index | 3 |  |
| } |  |  |
| If (Flag = 0b10) { |  |  |
| CLC Active Interval of Type II CLC class with subtype 1 | 8 |  |
| CLC Active Cycle of Type II CLC class with subtype 1 | 8 |  |
| } |  |  |
| If (Flag = 0b00) { |  |  |
| Extended Bitmap Indicator | 1 |  |
| If (Extended Bitmap Indicator == 0) { |  |  |
| CLC Active Bitmap } | 7 |  |
| If (Extended Bitmap Indicator == 1) { |  |  |
| Length of Extended CLC Active Bitmap (k) | 2 |  |
| Extended CLC Active Bitmap | (k+2) x m | m: number of subrames per frame |
| padding } |  |  |
| } |  |  |
| If (Flag = 0b11) { |  |  |
| CLC Active Interval of Type III CLC class } | 8 |  |
| } |  |  |

Parameters in the CLC Request field shall be as follows:

Flag

0b01: Type I CLC class;

0b10: Type II CLC class subtype 1;

0b00: Type II CLC class subtype 2 or 3;

0b11: Type III CLC class

Scheduling Impact

0b00 (default) = both DL and UL allocations are prohibited in CLC Active Interval

0b01 = only DL allocations are prohibited in CLC Active Interval

0b10 = only UL allocations are prohibited in CLC Active Interval

0b11 = reserved

CLC active interval of Type I CLC class

The number of subframes of the CLC Active Interval for Type I CLC class

CLC active cycle of Type I CLC class

The number of microseconds of the CLC Active Cycle for Type I CLC class

CLC active interval of Type II CLC class with subtype 1

The number of subframes of the CLC Active Interval for Type II CLC class

CLC active cycle of Type II CLC class with subtype 1

The number of frames of the CLC Active Cycle for Type II CLC class

Extended CLC Active Bitmap Indicator

Indicate whether the Extended CLC Active Bitmap field is used

CLC Active Bitmap

Setting a bit of the field to “1” indicates the corresponding subframe in each frame is in CLC active interval

Extended CLC Active Bitmap

Setting a bit of the field to “1” indicates the corresponding subframe in each CLC active cycle is in CLC active interval

CLC Active Interval of Type III CLC class

The number of superframes of the CLC Active Interval for Type III CLC class

Table x7—Syntax of CLC Response field

| Syntax | Size (bit) |
| --- | --- |
| Confirmed Action | 8 |
| for (i=1; i n; i++) { |  |
| CLC ID | 3 |
| Start Superframe Number | 3 |
| Start Frame Index | 2 |
| } |  |

Table x8: Syntax of CLC Report field

| Syntax | Size (bit) |
| --- | --- |
| TBD | TBD |

------------------------------------------------------------End of the Text---------------------------------------------------------