| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD New Contribution on Multi-Carrier Handover Procedure |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Category: AWD-New contribution / Area: Multi-Carrier “AWD New Contribution on Multi-Carrier Handover Procedure” |  |
| Abstract | This contribution proposes IEEE 802.16m AWD text on the Multi-Carrier HO procedure. |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD New Contribution on Multi-Carrier Handover Procedure

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Proposed Amendment Text

------------------------------------------------ Text Start -----------------------------------------------------------------------

# 15.2.x Multi-Carrier Operation

## 15.2.x.y Handover with Multi-Carrier Support

The multi-carrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers. An AMS with multi-carrier capability may follow the single-carrier handover procedure per section 15.2.5. It may also decide to perform MCHO procedure as defined in this section.

### 15.2.x.y.1 Network topology acquisition

#### 15.2.x.y.1.1 Network topology advertisement

The AAI_NBR-ADV message shall carry neighbor ABS’s multi-carrier configuration information to facilitate AMS’s scanning of neighbor ABSs’ fully configured carriers.

#### 15.2.x.y.1.2 AMS scanning of target carriers

The AMS with multi-carrier capability may perform the single-carrier scanning procedure per section 15.2.5.1.2. It may also perform multi-carrier scanning procedure, i.e. scanning procedure which involves multiple radio carriers, as defined in this subsection.

The AMS scans each fully configured carrier of the neighbor ABSs as advertised in the AAI_NBR-ADV message. The AMS may also scan other fully configured carriers of the serving ABS which are not in use by the AMS. Figure xx1 illustrates the example message flows for neighbor ABS advertisements and scanning of fully configured carriers of serving and neighbor ABSs.

Figure xx1 — Example message flows for neighbor ABS advertisement and scanning of fully configured carriers of serving and neighbor ABSs. [Note: This figure is modified from Rev2/D9, Annex D, Figure D.1]

An AMS capable of concurrently processing multiple radio carriers may perform scanning with neighbor ABSs using one or more of its available radio carriers without interruption to its normal communication with the serving ABS on the primary carrier and/or secondary carriers. In this case, the AMS may inform the serving ABS through AAI_SCN-REQ its carriers to be assigned for scanning operations to avoid resource allocation on those carriers, as illustrated in Figure xx2.

Figure xx2 — Example message flows for performing scanning using available radio carriers of the AMS while maintaining normal communication with its serving ABS [Note: This figure is modified from Rev2/D9, Annex D, Figure D.1]

### 15.2.x.y.2 Multi-carrier handover (MCHO) procedure

The multi-carrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers, as described in this section.

#### 15.2.x.y.2.2 MCHO preparation

An AMS in multi-carrier operation follows the handover operations defined in 15.2.5.2. MAC management messages in relation with handover preparation and initiation between the AMS and the serving ABS are transmitted over the primary carrier of the AMS.

During HO preparation, the AMS may indicate its multi-carrier capability through AAI_MSHO-REQ or AAI_HO-IND messages. The ABS may also inform AMS the multi-carrier configurations of one or more potential target ABSs through AAI_BSHO-CMD message.

#### 15.2.x.y.2.3 MCHO execution and network re-entry

The AMS with multi-carrier capability follows the network re-entry procedure per section 15.2.5.2.5. The AMS may use the original primary carrier for network re-entry to the target ABS, as illustrated in Figure xx3. It may also use another carrier different from its original primary carrier for network re-entry procedures, as illustrated in Figure xx4. In both cases, if the HO_Reentry_Mode in AAI_BSHO-CMD message is set to 1, the AMS maintains normal communication with the serving ABS on another carrier not performing network re-entry procedure. The AMS may inform the serving ABS through AAI_HO-IND the carrier to be assigned for network re-entry operations to avoid resource allocation by the serving ABS on that carrier.

Figure xx3 — A call flow for multi-carrier HO in which the AMS performs network re-entry to the target ABS on one radio carrier while maintaining communication with the serving ABS on another carrier.

Figure xx4 — A call flow for multi-carrier HO in which the AMS performs network re-entry on the target primary carrier which is different from the serving primary carrier

----------------------------------------------------- Text End ---------------------------------------------------------------